- Updated 28 April 2011

11th ASIA-PACIFIC SATELLITE DATA EXCHANGE AND UTILISATION /

23rd  N. AMERICA/EUROPE DATA EXCHANGE MEETING - 2011

ASIA-PACIFIC REQUIREMENTS

This document presents the combined status of and requirements for data and products from North America for operational NWP and related activities in the Asia-Pacific. Unless otherwise specified, the stated requirement represents a common requirement of two or more centres.

A key requirement common to all the products listed below is the reliable exchange of information on any changes impacting any of the transmitted products. Timely exchange of information will help in explaining anomalies in the monitoring and if assimilated in forecast trends.

The Asia-Pacific Meteorological Services referred to in this document together with the abbreviations used are:

| - Bureau | Bureau of Meteorology |
| --- | --- |
| - CMA | China Meteorological Administration |
| - JMA | Japan Meteorological Agency |
| - KMA | Korea Meteorological Agency |

- POLAR SATELLITES: ORBIT-BY-ORBIT DATA/PRODUCTS

| - ATOVS - ATOVS level 1 B data |  |
| --- | --- |
| Availability | Data copied from NESDIS via dedicated link to the Met Office who encode the 1B data into BUFR (1C data). Access given to other European Met. Services and ECMWF. There is a continuing requirement to receive AMSU data in 1B rather than 1C form as the AMSU housekeeping information is monitored by the Met Office. |
| Usage and plans (by when) | Bureau: ATOVS radiances assimilated operationally NOAA-16 HIRS & AMSU-A NOAA-17 HIRS only NOAA-18 AMSU-A & MHS NOAA-19 HIRS & AMSU-A MetOp2 HIRS& AMSU-A & MHS EOS2 AMSU-A(9) monitored only CMA: Data is received from EUMETCast JMA: AMSU-A data from NOAA-15, 16, 18, 19, Metop-A, Aqua, AMSU-B data from NOAA-15, 16, MHS data from NOAA-18, 19, Metop-A are assimilated operationally (1). The data are also used for the tropical cyclone intensity estimation (2). KMA: 1D radiances (AMSU-A from NOAA-15, 16 , 18 and METOP, HIRS from NOAA-17 and METOP, AMSU-B from NOAA-16 and 17, MHS from NOAA18 and METOP) from Met Office ftp server assimilated operationally |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - ATOVS and AVHRR level-0 raw HRPT data from NOAA/NESDIS |  |
| Availability | Not received via NESDIS. ATOVS derived products with strict timeliness requirement (<30 minutes after start of pass) have been provided since 12 November 2002 with the EUMETSAT ATOVS Retransmission Service (EARS). Usage is for short-range NWP in Europe. The approach for transmitting the data to Europe is via a dedicated IP Virtual Private Network. The current service provides ATOVS derived products from Kangerlussuaq (Greenland), Svalbard, Maspalomas (Canary Islands), Athens (Greece), Monterey, Wallops, Gilmore Creek (US) and Edmonton, Gander Canada., Lannion France. Recent additional stations include Hawaii, Miami and La Reunion. |
| Usage and plans (by when) | Bureau: AP-RARS ATOVS available and used. AIRS available, and expect to be used inside 12 months CMA: JMA: AP-RARS, SA-RARS and EARS L1C data are assimilated operationally (1). KMA: EARS, AP-RARS and SA-RARS(AMSU-A and AMSU-B) data through GTS are assimilated operationally |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - AVHRR products on HIRS grid, derived as part of NESDIS ATOVS processing |  |
| Availability | Some AVHRR data from NOAA copied via dedicated link to METO. Some satellites have not data. |
| Usage and plans (by when) | ECMWF: No EUMETSAT: METO: No METEO-FRANCE: DWD: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - AVHRR L1 data |  |
| Availability | EUMETSAT is now providing METOP AVHRR data via its EARS network in near real time. The current service is based on reception at Mas Palomas, Athens, Lannion, Svalbard and Kangerlussuaq stations. Global METOP AVHRR distrubted via EUMETCAST. CAC (4km) from NOAA-19 via EUMETCAST. |
| Usage and plans (by when) | Bureau: CMA: JMA: The data are used for the operational SST analysis (1) and the operational weather monitoring (4). KMA: |
| Requirement Access and timeliness | Bureau: Require METOP FRAC AVHRR L1 (radiances) or L2 (SST) data within 3 hours of observation for operational SST analyses. CMA: JMA: KMA: |
| - AVHRR polar winds |  |
| Availability | NOAA/CIMSS are producing AVHRR polar AMVs in BUFR and these are provided to users by internet. A similar product is available from NESDIS. |
| Usage and plans (by when) | Bureau: AVHRR polar AMVs used operationally CMA: to be used for comparison with FY-3/SBUS data，ftp, daily JMA: The data are used for the operational SST analysis (1) and the operational weather monitoring (4). KMA: AVHRR polar wind data (METOP and NOAA satellites) through NESDIS satepsdist1e server and WISCONSIN ftp sites are assimilated operationally. But we want to get that data through GTS or RMDCN directly for time saving. |
| Requirement Access and timeliness | Bureau: CMA: ftp, daily JMA: KMA: we want to get that data through GTS or RMDCN directly for time saving. |
| - Ozone retrieved profiles and/or total column from SBUV on NOAA |  |
| Availability | Ozone profile data are received in BUFR from NOAA/NESDIS. |
| Usage and plans (by when) | Bureau: CMA: JMA: The SBUV ozone data are currently tested for the operational use of ozone monitoring and prediction (1). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Interest to monitor |
| - SeaWinds Scatterometer Data – QuikSCAT |  |
| Avaiability | Via NESDIS. Nearing the end of its life time |
| Usage and plans (by when) | Bureau: Reject List. QuikSCAT winds used for NWP verification and in operational wave models. Require in delayed mode. CMA: JMA: QuikSCAT operation was terminated in 2009. But the data had been assimilated operationally and was beneficial (1). The data had been used for the operational typhoon analysis (1). KMA: |
| Requirement Access and timeliness | Bureau: Require in delayed mode. CMA: JMA: KMA: |
| - SSM/I and SSM/IS on DMSP satellite |  |
| - SSM/I brightness temperatures |  |
| Availability | F-15 data received from NESDIS server. Data passed to ECMWF where they are BUFR-encoded and returned to METO for onward transmission to other European centres. Fully operational service requires BUFR encoding at NESDIS. Also being copied to Italian Met Service. |
| Usage and plans (by when) | Bureau: Reject List CMA: Data will be used to retrieve total precipitable preciptation JMA: F15 radiance data are used for the snow cover analysis in the climate analysis and forecast system (1). The data are also used for tropical cyclone analysis (2). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - SSM/I retrieved products (surface wind speed (WS), total column water vapour (TCWV), cloud liquid water (CLW), rain rate, others?) |  |
| Availability | Available on NESDIS server to be copied via Internet |
| Usage and plans (by when) | Bureau: CMA: JMA: The data may be used for the model verification (3). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: The data may be used for the model verification (3). KMA: |
| - SSM/IS |  |
| Availability | The F-16 and F-17 data is received by the Met Office after unified pre-processing by NRL and also receive data via UPP run at NRL which corrects for calibration errors. F-18 has been launched recently and early data access is required.. |
| Usage and plans (by when) | Bureau: received, and will be operationally assimilated in 6-12 months. CMA: Data will be used to retrieve total precipitable preciptation JMA: The radiance data of F16 and F17 imager channels from NESDIS are assimilated operationally. F18 pre-processed radiance data (imager and sounder channels) are under investigation. F16 radiance data are also used for the sea ice analysis. For the snow cover analysis in the climate analysis and forecast system, F16 radiance data are used operationally and F17 and F18 radiance data are under investigation (1). The data are also used for the tropical cyclone analysis and intensity estimation (2). KMA: DMSP-16 radiance data from Met Office ftp server is assimilated operationally. But we want to get that data through GTS or RMDCN for time saving. |
| Requirement Access and timeliness | Bureau: CMA: Data will be used to retrieve total precipitable preciptation JMA: KMA: Desire to get data through GTS or RMDCN for time saving. |
| - Altimeter data (JASON/GEOSAT/JASON-2/ENVISAT) - Altimeter wave/wind data |  |
| Availability | JASON-2 data is available via EUMETCAST. |
| Usage and plans (by when) | Bureau: CMA: JMA: The data are used in the wind wave analysis. (1). KMA: Plans to use data for monitoring and comparison |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: : Plans to use data for monitoring and comparison |
| - Altimeter sea surface height data |  |
| Availability | Not received in near real-time. Delayed mode data received via CLS (Toulouse). NASA is expected to produce the data using a similar processing chain. Information is requested about the planned provision of the data via the GTS. BUFR formats have recently been defined. |
| Usage and plans (by when) | Bureau: Jason-1, Jason-2 and Envisat SSHA data used for assimilation and validation for operational ocean, wave and future seasonal prediction models. Also used to validate Tsunami models. Currently using delayed mode SSHA data but investigating using real-time (< 24 hour) data. CMA: JMA: The data are used in the ocean data assimilation (1). KMA: : Plans to use data for monitoring and comparison |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Plans to use data for monitoring and comparison |
| - TERRA, AQUA and AURA data (NASA) - AQUA AIRS+AMSU-A data |  |
| Availability | NESDIS process data in near-real time and transfer them in BUFR to METO. METO also receives AIRS by direct broadcast processed using IMAPP-AIRS. |
| Usage and plans (by when) | Bureau: AIRS & AMSU-A used operationally CMA: JMA: The AIRS data are currently investigated for the operational use (1). AMSU-A data are used operationally (1). The data are also used for the tropical cyclone intensity estimation (2). KMA: AQUA AIRS radiance data from NESDIS dds ftp server is assimilated operationally. But we want to get that data through GTS or RMDCN for time saving |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get data through GTS or RMDCN for time saving |
| - AQUA AMSR-E data |  |
| Availability | Access via internet at present. |
| Usage and plans (by when) | Bureau: Used in research into soil moisture initialization. AMSR-E SSTs from Remote Sensing Systems are used in operational SST analyses and ocean models. Require within 3 hours of observation. CMA: Data will be used in retrieval in cloudy case JMA: The radiance data are assimilated operationally (1). The data are used for tropical cyclone analysis and maximum wind speed estimation (2). AMSR-E L2 data are also used for the operational SST and sea ice analysis (1). KMA: |
| Requirement Access and timeliness | Bureau: CMA: Data will be used in retrieval in cloudy case JMA: KMA: |
| - AQUA and TERRA MODIS polar winds |  |
| Availability | University of Wisconsin and more recently NESDIS produce cloud track winds from MODIS-TERRA/AQUA over Polar Regions. Direct broadcast winds are also received from the University of Wisconsin. |
| Usage and plans (by when) | Bureau: Used operationally CMA: JMA: The DB winds and NESDIS winds are assimilated operationally (1). KMA: AQUA and TERRA MODIS polar wind data through NESDIS satepsdist1e ftp server and WISCONSIN ftp sites are assimilated operationally. But we want to get that data through GTS or RMDCN directly for time saving |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get the data through GTS or RMDCN directly for time saving |
| - AURA: HIRDLS, MLS, OMI and TES |  |
| Availability | Provides information on various chemical constituents of the atmosphere. Some data available in near real time on internet from NASA DAAC. |
| Usage and plans (by when) | Bureau: OMI used operationally. Interest in MLS for research CMA: Data will be used for comparison with FY-3/TOU, SBUS data, ftp, daily. JMA: KMA: Plans to use data for research |
| Requirement Access and timeliness | Bureau: Interest in MLS for research CMA: ftp, daily. JMA: KMA: Plans to use data for research |
| - TERRA:MOPITT |  |
| Availability | The Version 4 data are available from a NASA FTP server (ftp://l4ftl01.larc.nasa.gov/MOPITT/MOP02.004) in one file per day with a delay of approximately 3 days. |
| Usage and plans (by when) | Bureau: CMA: JMA: The OMI data are used in the ozone monitoring and prediction. The other sensor data are currently investigated (1). The OMI data will be used by JMA/MRI for model validation and data assimilation of chemical species (2). KMA: Plans to use data for research |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Plans to use data for research |
| - TERRA and AQUA MODIS aerosol products |  |
| Availability | The offline data are available from a NASA FTP server (ftp://ladsftp.nascom.nasa.gov/allData/5/) with a delay of the order of 1 to 2 days. A NR version of the MODIS aerosol product is available from FTP server ftp://nanuk.eosdis.nasa.gov/pub/p[a]m1/MODIS/MOD04_L2 with a delay of around 3 hours. |
| Usage and plans (by when) | Bureau: CMA: JMA: MODIS aerosol products are used for the estimation of climatic value of aerosol distribution. The climatic value is used in the NWP model (1). The MODIS aerosol products will be used by JMA/MRI for model validation and data assimilation of chemical species (2). KMA: Plans to use data for research |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Plans to use data for research |
| - Windsat/CORIOLIS |  |
| Availability | Launched in Dec 2002. Data sent from NRL to Met Office in NRL format via internet and alternate datasets from NESDIS via dedicated link. Latter is 2 hours later than NRL dataset. |
| Usage and plans (by when) | Bureau: Will follow developments at Met Office. CMA: JMA: MODIS aerosol products are used for the estimation of climatic value of aerosol distribution. The climatic value is used in the NWP model (1). The MODIS aerosol products will be used by JMA/MRI for model validation and data assimilation of chemical species (2). KMA: Plan to assimilate that data. We are now receiving that data on binary form from NESDIS satepsanone ftp server and processing to convert to bufr form using Met Office WindSAT PP. |
| Requirement Access and timeliness | Bureau: Will follow developments at Met Office. CMA: JMA: KMA: |
| - COSMIC Radio-Occultation constellation |  |
| Availability | These data are now on the dedicated Washington/Exeter link (METO put them on GTS/RMDCN to other European users): timeliness has been nominal and stable. SAC-C is a JPL mission whose data we would definitely like to have in NRT (if possible as bending angles). |
| Usage and plans (by when) | Bureau: COSMIC to be used operationally within 6 months CMA: JMA: The COSMIC and GRAS refractivity data are assimilated operationally (1) and the bending angle data and excess path length data are currently investigated (2). KMA: 6 COSMIC RO data from NESDIS dds ftp server is assimilated operationally. But we want to get that data through GTS or RMDCN for time saving. |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get data through GTS or RMDCN for time saving. |
| - TRMM Data |  |
| Availability | Available via ftp level-1b TMI radiances and near-surface rainrates from the 2A12 TMI retrieval algorithm in NRT. This stream sould be maintained as is. |
| Usage and plans (by when) | Bureau: CMA: JMA: The TMI radiance data are assimilated operationally (1). The PR rain profiles are used for the model verification and rainfall retrieval algorithm development (3). The data are used for tropical cyclone analysis and maximum wind speed estimation (2). KMA: : Plans to use data for research |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - NPP Data |  |
| Availability | Request information on future availability of NPP datasets to European Met Services in real time to include VIIRS, CrIS and ATMS level 1B radiances. Information on both global data and local area direct readout services is requested to include timeliness of data, dataset descriptions, formats etc. Request to become formal collaborator in Cal/Val efforts with early access to simulated and real data. Also interest in receiving simulated NPP datasets. Launch planned for 2011 Interest in RARS system for ATMS/CrIS. |
| Usage and plans (by when) | Bureau: Monitor and assimilate once available. Will use NPP global GHRSST L2P SST products for operational SST analyses and ocean models. Required within 3 hours of observation. CMA: Data will be used in data assimilation and atmospheric condition retrieval JMA: : The ATMS and CrIS data are highly requested and the data are expected to be assimilated as soon as possible (1). The VIIRS data are also expected for the SST analysis (1). KMA: Yes, if/when available |
| Requirement Access and timeliness | Bureau: NPP global GHRSST L2P SST products required within 3 hours of observation. CMA: JMA: The ATMS and CrIS data are highly requested KMA: Yes, if/when available |
| - POLAR SATELLITES: MAPPED PRODUCTS (not orbit-by-orbit) - SSM/I Products - Ice/snow gridded field derived from SSM/I data |  |
| Availability | Received routinely via Washington-Exeter link. Contact point J. Tesmer (tesmerj@fnoc.navy.mil). |
| Usage and plans (by when) | Bureau: Interest to drive land data assimilation scheme CMA: JMA: KMA: Yes, Plan to monitor and compare with intrinsic data |
| Requirement Access and timeliness | Bureau: Interest to drive land data assimilation scheme CMA: JMA: KMA: Yes, Plan to monitor and compare with intrinsic data |
| - NCEP sea-ice analysis |  |
| Availability | Data received routinely via Washington-Exeter link. |
| Usage and plans (by when) | Bureau: Used operationally for NWP and SST analyses. CMA: JMA: : The data may be used for the model verification (3). KMA: Yes, Plan to monitor and compare with intrinsic data |
| Requirement Access and timeliness | Bureau: CMA: JMA: : The data may be used for the model verification (3). KMA: Yes, Plan to monitor and compare with intrinsic data |
| - AVHRR - AVHRR/MODIS products (eg vegetation indices, LAI, land use) |  |
| Availability | Products available from Internet or order from EOS data gateway. |
| Usage and plans (by when) | Bureau: Interest to drive land data assimilation scheme CMA: Data will be used for monitoring the vegetation growth and relevant research. Weekly data products are required. JMA: The data may be used for the model development (3). The AVHRR/MODIS products will be used by MRI for model validation and data assimilation of chemical species (2). KMA: : Interest to use for research |
| Requirement Access and timeliness | Bureau: CMA: Weekly data products are required. JMA: KMA: : Interest to use for research |
| - GEOSTATIONARY SATELLITES: DATA/PRODUCTS - GOES VIS/IR/WV atmospheric motion vectors |  |
| Availability | High density (HD) product on GTS in BUFR and SATOB from GOES EAST and WEST. Please advise when hourly data will become available. |
| Usage and plans (by when) | Bureau: HD BUFR winds used operationally. CMA: JMA: The AMV data are assimilated operationally (1). KMA: GOES AMV data through NESDIS satepsdist1e ftp server is assimilated operationally. But we want to get that data through GTS or RMDCN directly for time saving. Actually GOES AMV data is receiving through GTS but the amount is very insufficient |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get data through GTS or RMDCN directly for time saving. |
| - Cloud cleared radiances from GOES-E and GOES-W |  |
| Availability | Available from NESDIS and CIMSS. |
| Usage and plans (by when) | Receiving GOES-11 from NESDIS and GOES-12 from CIMSS via ftp. The pre-processing is not exactly the same but we can't see any obvious differences in our monitoring statistics. Contact persons at CIMSS are tony.schreiner@ssec.wisc.edu and tims@ssec.wisc.edu and at NESDIS Curtis.Holland@noaa.gov Bureau: Interest for research prior to MTSAT replacement CMA: Data will be used for data assimilation JMA: The radiance data are assimilated operationally (1). KMA: Plan to assimilate that data. We want to get that data on bufr form through GTS or RMDCN. |
| Requirement Access and timeliness | Bureau: Interest for research CMA: Data will be used for data assimilation JMA: The radiance data are assimilated operationally (1). KMA: Plan to assimilate that data. We want to get that data on bufr form through GTS or RMDCN |
| - Digital imagery from GOES-E, GOES-W and GOES-10(SA) |  |
| Availability | Not received from NESDIS. GOES-E, GOES-W retransmitted via METEOSAT 0E which satisfies current European requirements for imagery. No imagery from GOES-10 received. |
| Usage and plans (by when) | Bureau: CMA: JMA: The data are investigated for the SST analysis (2). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - Level 0 (GVAR) data from GOES-W and GOES-E |  |
| Availability | Currently received in NRT by MACC partner institution from UCAR Unidata (http://www.unidata.ucar.edu/) Also currently received in NRT by EUMETSAT, but suitability still to be verified |
| Usage and plans (by when) | Bureau: CMA: JMA: KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - NON-SATELLITE DATA - NCEP SST gridded field |  |
| Availability | Data received via GTS or via FTP once a day and used operationally. Current product resolution is 1 degree, 1/2 and ¼ degrees. Plans for 1/12 deg product. |
| Usage and plans (by when) | Bureau: CMA: JMA: The data may be used for the model verification (3). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - U.S. Profiler data |  |
| Availability | Received in BUFR. |
| Usage and plans (by when) | Bureau: Assimilated operationally CMA: JMA: The data are assimilated operationally (1). KMA: US Profiler data through GTS is assimilated operationally. But we want to get that data through GTS or RMDCN directly for time saving. |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get data through GTS or RMDCN directly for time saving |
| - MDCRS data (e.g. AMDAR etc) |  |
| Availability | Received and used operationally. |
| Usage and plans (by when) | Bureau: Assimilated operationally CMA: JMA: The data are assimilated operationally (1). KMA: MDCRS data through ESRL ftp server is assimilated operationally. But we want to get that data through GTS or RMDCN directly for time saving. |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: Desire to get data through GTS or RMDCN directly for time saving |
| - Tropical cyclone data |  |
| Availability | Received in tabular and BUFR form on GTS. |
| Usage and plans (by when) | Bureau: CMA: JMA: The data are used for the operational model verification (1) and tropical cyclone forecast verification (3). KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - Snow cover analysis for N. Hemisphere |  |
| Availability | Dataset available via Internet |
| Usage and plans (by when) | Digitised analysis of satellite and surface observations. Bureau: CMA: JMA: The data is expected to be used in the surface analysis (2). KMA: Yes, Plan to monitor and compare with intrinsic data |
| Requirement Access and timeliness | Bureau: CMA: JMA: The data is expected to be used in the surface analysis (2). KMA: Yes, Plan to monitor and compare with intrinsic data |
| - N. American hourly surface observations |  |
| Availability | Hourly METAR data are received. Station level is computed as station height is not known. |
| Usage and plans (by when) | Bureau: Interest for model validation and land surface assimilation. CMA: JMA: The six-hourly data are assimilated operationally (1) and the hourly data are investigated for the operational use (3). KMA: If possible to get, plan to monitor and research. |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: If possible to get, plan to monitor and research |
| - N. American soil temperature data on GTS |  |
| Availability | The Climate Prediction Center providing soil temperature data on server. ECMWF has been given access to these data, which are transferred routinely via FTP. |
| Usage and plans (by when) | Bureau: Interest for validation and assimilation CMA: JMA: The data may be used for the land surface analysis (3) KMA: |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: |
| - N. American ozone soundings on GTS |  |
| Availability | Few soundings are available on the GTS. Canadian soundings are now back on the GTS. |
| Usage and plans (by when) | Bureau: Interest for model validation CMA: : to be used for validation of FY-3/TOU, SBUS ozone products, ftp, daily JMA: The data are expected for the model verification (2). KMA: |
| Requirement Access and timeliness | Bureau: CMA: ftp, daily JMA: KMA: |
| - TAO/TRITON/PIRATA /RAMAarray data, XBT and PALACE and Argo floats |  |
| Availability | Available on the GTS in BUOY, BATHY and TESAC code, received and used in Europe. |
| Usage and plans (by when) | Bureau: Buoy SST data used operationally in SST analyses. XBT and Argo data assimilated operationally into ocean models and regional SST analysis. Required within 6 hours of observation. CMA: JMA: The data are used for the ocean analysis (1). KMA: the buoy data through GTS is assimilated operationally |
| Requirement Access and timeliness | Bureau: Required within 6 hours of observation. CMA: JMA: KMA: |
| - GPS total column water vapour over N. America |  |
| Availability | The SuomiNet data from UCAR (http://www.gst.ucar.edu/gpsrg/realtime.html) is a potential NRT source of data. Initial investigations have shown BUFR product is not in WMO approved template. One option being explored is to receive non-BUFR format and METO would put the data into WMO BUFR to place on GTS. Recently EUMETNET are also getting involved in helping out. Contact is Seth Gutman (NOAA-ESRL). |
| Usage and plans (by when) | Bureau: Expect to use for validation of assimilation system. CMA: JMA: The data are expected to be assimilated in the global NWP (1). KMA: If possible to get, plan to use after monitoring and research |
| Requirement Access and timeliness | Bureau: CMA: JMA: KMA: If possible to get, plan to use after monitoring and research |

- Table of Asia-Pacific Met Service Data Requirements from NOAA

|  |  | Bureau | CMA | JMA | KMA |
| --- | --- | --- | --- | --- | --- |
| E.1.1.1 | TOVS 1B | 3 |  | 1 |  |
| E.1.2.1 | ATOVS 1B | 1 |  | 1 |  |
| E.1.2.2 | ATOVS from HRPT | 1 |  | 1 |  |
| E.1.2.3 | AVHRR |  | 2 | 1 |  |
| E.1.2.4 | AVHRR L1 data | 2 | 1 | 1 |  |
| E.1.2.5 | AVHRR polar winds | 1 |  | 1 | 1 |
| E.1.3 | SBUV | 2 |  | 1 |  |
| E.1.4 | SeaWinds | 1 |  | 1 | 3 |
| E.1.5.1 | SSM/I BT SDR | 1 | 2 | 1 |  |
| E.1.5.2 | SSM/I products EDR | 2 |  | 3 |  |
| E.1.5.3 | SSM/IS | 1 | 2 | 1 | 1 |
| E.1.6.1 | Altimeter wind/wave |  |  | 1 | 3 |
| E.1.6.2 | Altimeter surface ht | 1 |  | 1 | 3 |
| E.1.7.1 | AIRS and AMSU-A data | 1 |  | 1 | 1 |
| E.1.7.2 | AMSR-E data | 1 | 2 | 1 |  |
| E.1.7.3 | MODIS polar winds | 1 |  | 1 | 1 |
| E.1.7.4 | AURA: HIRDLS, MLS, OMI and TES | 1 | 2 | 1 | 3 |
| E.1.7.6 | TERRA: MOPITT |  |  |  | 3 |
| E.1.7.7 | MODIS aerosol products |  |  | 1 | 1 |
| E.1.8 | Windsat/CORIOLIS | 1 | 2 | 1 | 1 |
| E.1.9 | COSMIC | 1 |  | 1 | 1 |
| E1.10 | TRMM | 2 | 2 | 1 | 3 |
| E1.11 | NPP (CrIS, ATMS, VIIRS) | 2 | 2 | 1 | 2 |
| E1.12 | OCO |  |  |  |  |
| E.2.1.1 | Mapped SSM/I |  | 2 | 3 | 2 |
| E.2.1.2 | NCEP sea-ice analyses | 1 | 2 | 3 | 2 |
| E.2.2.1 | AVHRR SST | 1 |  | 3 | 3 |
| E.2.2.2 | AVHRR/MODIS products |  | 2 | 3 |  |
| E.2.2.3 | AVHRR polar winds | 2 | 2 | 1 | 1 |
| E.3.1 | GOES VIS/IR/WV winds | 1 | 2 | 1 | 1 |
| E.3.2 | Cloud cleared radiances | 2 | 2 | 1 | 2 |
| E.3.3 | GOES/GMS imagery | 1 | 2 |  |  |
| E.3.4 | Other global imagery |  |  |  |  |
| E.4.1 | NCEP SST gridded field |  |  | 3 |  |
| E.4.2 | Profiler data | 1 |  | 1 | 1 |
| E.4.3 | MDCRS (AMDAR) data | 1 |  | 1 | 1 |
| E.4.4 | Tropical cyclones |  |  | 1 |  |
| E.4.5 | Snow analysis | 2 | 2 | 2 | 2 |
| E.4.6 | Hourly surface data | 2 |  | 1 | 2 |
| E.4.7 | Soil temperature | 2 |  | 3 |  |
| E.4.8 | Ozone soundings | 1 |  | 2 |  |
| E.4.9 | Ocean data | 1 |  | 1 |  |
| E.4.10 | GPS ZTD | 1 |  | 1 | 1 |

1 = High priority required for operations

2 = Medium priority to investigate possibility for future operational use

3 = Low priority for research

4 = For backup