FY-1 and FY-3 programmes

The Chinese FY-1 series  started in 1988.  The first two satellites (FY-1A and FY-1/B) were using the ITOS platform (see section 3.2 and Fig. 3.2.3), the next two (FY-1C and FY-1D) a new platform.  The FY-3 series is being developed and include 7 flight models.  All satellites are 3-axis stabilised, in sunsynchronous orbit.  Table 1 records the chronology of the FY-1 / FY-3 programme.

Table 1 – Chronology of the FY-1/FY-3 programme (in bolt the satellites active in Dec 2006)

| Satellite | Launch | End of service | Height | LST | Status (Dec 2006) | Instruments |
| --- | --- | --- | --- | --- | --- | --- |
| FY-1A | 7 Sep 1988 | 16 Oct 1988 | 900 km | 11.30 | Inactive | MVISR, SEM |
| FY-1B | 3 Sep 1990 | 5 Aug 1991 | 900 km | 16.00 | Inactive | MVISR, SEM |
| FY-1C | 10 May 1999 | 26 April 2004 | 862 km | 6.45 | Inactive | MVISR, SEM |
| FY-1D | 15 May 2002 | expected  2006 | 866 km | 6.50 | Operational | MVISR, SEM |
| FY-3A | 2007 | expected  2010 | 836 km | 10.00 | Being built | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3B | 2010 | expected  2013 | 836 km | 14.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3C | 2012 | expected  2015 | 836 km | 10.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3D | 2014 | expected  2017 | 836 km | 14.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3E | 2016 | expected  2019 | 836 km | 10.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3F | 2018 | expected  2021 | 836 km | 14.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |
| FY-3G | 2020 | expected  2023 | 836 km | 10.00 | Planned | VIRR, MERSI, MWRI, IRAS, MWTS, MWHS, TOU/SBUS, SEM |

Payload of FY-1

From April 2004, the only operational satellite is FY-1D, launched in 2002.  It embarks the following instruments:

- MVISR (Multichannel Visible and Infrared Scanning Radiometer), VIS/IR radiometer for multi-purpose imagery, resolution 1.1 km, swath 2800 km.  On FY-1A and FY-1B MVISR had 5 channels (0.48-0.53, 0.53-0.58, 0.58-0.68, 0.725-1.10 e 10.5-12.5 m).  On FY-1C and FY-1D there are 10 channels.

- SEM (Space Environment Monitoring) for in situ observation of charged particles in solar wind.

Data transmission from FY-1

Global data are stored on board and transmitted in S-band as:

- China Delayed Picture Transmission (CDPT): MVISR imagery with resolution reduced to 4 km; frequency 1708.5 MHz (backup 1695.5 MHz, bandwidth 5.6 MHz, data rate 1.33 Mbps.

As for direct read-out, there is:

- CHRPT (China High Resolution Picture Transmission), for the whole information at full resolution in digital form at S-band frequencies.  Main features:

    - frequencies: 1700.5 MHz (backup 1704.5 MHz); bandwidth: 5 MHz; polarisation: right-hand circular

    - antenna diameter ~ 2 m, G/T ~ 6.0 dB/K, data rate 1.33 kbps.

Payload of FY-3

- VIRR (Visible and Infra Red Radiometer), close to MVISR except that the water vapour channel at 932 nm is replaced by 1360 nm; 10-channel VIS/IR radiometer for multi-purpose imagery, resolution 1.1 km, swath 2800 km.

- MERSI (Medium Resolution Spectral Imager), 20-channel radiometer (19 in VIS/NIR/SWIR + one TIR at 10.0-12.5 m) for ocean colour and vegetation indexes; resolution 250 m for 4 VIS/NIR and the TIR channel, 1 km for all other channels; swath 2800 km.

- MWRI (Micro-Wave Radiation Imager), 6-frequencies / 12 channels (all frequencies in double polarisation) for multi-purpose MW imagery.  Conical-scanning radiometer, resolution 9.5 x 15 km at 90 GHz, 30 x 50 km at 19 GHz, swath 1400 km.

- IRAS (Infra Red Atmospheric Sounder), 26-channel IR radiometer (including one VIS) for temperature/humidity sounding, resolution 17 km, swath 2250 km.

- MWTS (Micro-Wave Temperature Sounder), 4-channel MW radiometer for nearly-all-weather temperature sounding, 54 GHz band, resolution 70 km, cross-track scanning, swath 2200 km.  MWHS (Micro-Wave Humidity Sounder), 4-frequency / 5-channel (one frequency in double polarisation) MW radiometer for nearly-all-weather humidity sounding, 183 GHz band, resolution 15 km, cross-track scanning, swath 2700 km.

- TOU/SBUS (Total Ozone Unit and Solar Backscatter Ultraviolet Sounder ), a suite of two UV spectro-radiometers, one (TOU) with 6 channels in the 308-360 nm range, resolution 50 km, swath 3000 km, for total ozone; the other one (SBUS) with 12 channels in the range 252-340 nm, resolution 200 km, nadir viewing, for ozone profile.

- SEM (Space Environment Monitoring) for in situ observation of charged particles in solar wind.

Data transmission from FY-3

The data rate of the MERSI instrument requires moving to X-band, both for global data recovery and for full information real-time transmission.  Global data stored on board are transmitted as:

- Delayed Picture Transmission (DPT): frequency: 8146 MHz; bandwidth: 149 MHz; data rate: 93 Mbps.

Direct read-out is provided according to two systems:

- MPT (Medium-resolution Picture Transmission), for full information of selected transmission area in X-band.  Main features:

    - frequency: 7775 MHz; bandwidth: 45 MHz; polarisation: right hand circular

    - antenna diameter ~ 3 m, G/T ~ 21.4 dB/K, data rate 18.7 Mbps;

- AHRPT (Advanced High Resolution Picture Transmission) for full information in S-band.  Main features:

    - frequency: in the range 1704.5 MHz; bandwidth: 6.8 MHz; polarisation: right hand circular

    - antenna diameter ~ 3 m, G/T ~ 6.8 dB/K, data rate 4.2 Mbps.

FY = Feng-Yun, “Wind and Cloud”.