The FY-2 and FY-4 Programmes

The Chinese series FY-2 (Feng-Yun-2) , operational since 1997, due to cover the 105°E position, is spin stabilised.  The next generation, FY-4, to take service around 2012, will be 3-axis stabilised.  Table 1 records the chronology of the FY-2/F-4 programme.

Table 1 - Chronology of the FY GEO Programme (in bold the satellites active in Dec.2006)

| Satellite | Launch | End of service | Position | Status (Sept 2005) | Instruments |
| --- | --- | --- | --- | --- | --- |
| FY-2A | 10 Jun 1997 | 08 April 1998 | 86.5°E | Inactive | S-VISSR, DCS, SEM |
| FY-2B | 25 Jun 2000 | Sept. 2004 | 123.5°E | Partial backup | S-VISSR, DCS, SEM |
| FY-2C | 19 Oct 2004 | expected  2009 | 105°E | Operational | S-VISSR (improved), DCS, SEM |
| FY-2D | 2006 | expected  2011 |  | Being built | S-VISSR (improved), DCS, SEM |
| FY-2E | 2009 | expected  2014 |  | Planned | S-VISSR (improved), DCS, SEM |
| FY-2F | 2011 | expected  2015 |  | Planned | S-VISSR (improved), DCS, SEM |
| FY-4A | 2012 | expected  2017 |  | Being defined | Imager, sounder, lightning |
| FY-4B | 2015 | expected  2020 |  | Being defined | MW radiometer |

FY-2A

Launched in June 1997 on the 105°E position, was moved to 86.5°E in July 2000 to leave the operational service to FY-2B.  It still has some capabilities that could be used in emergency.

FY-2B

Launched in June 2000, it exhibits now several operational limitation and serves as backup at 123.5°E.

FY-2C, 2D, 2E and 2F

FY-2C has been launched on 19 October 2004, and will be followed by FY-2D (2006), FY-2E (2009) and FY-2F (2011).  The payload has been improved in respect of that one of FY-2A and FY-2B.

Payload of FY-2

- S-VISSR (Stretched Visible and Infrared Spin Scan Radiometer) – The version of FY-2A/B had three VIS/IR channels (0.5-1.05 m, 6.3-7.6 m and 10.5-12.5 m) the improved version for FY-2 C/D/E/F splits the IR channel in two and adds a 3.5-4.0 m channel.  The resolution also is slightly improved: from 5.76 km (IR) and 1.44 km (VIS), to 5.0 km (IR) and 1.25 km (VIS).  The image cycle is 30 min.

- Data Collection Service (DCS) - Main features:

    - uplink: two bands, frequencies 402.0-402.1 MHz for international DCPs (33 channels of bandwidth 3 kHz), 401.1-401.4 MHz for regional DCPs (100 channels of bandwidth 3 kHz); data rate 100 kbps, polarisation right-hand circular.

- SEM (Space Environment Monitor).

Data transmission from FY-2

FY-2 data are transmitted in real time to the:

- Command and Data Acquisition Station (CDAS).  Main transmission characteristics:

    - frequency 1681.6 MHz, bandwidth 14 MHz, linear polarisation, data rate 14 Mbps.

Afterwards, data are re-transmitted to user stations.  The broadcast will comply with the HRIT and LRIT standards and initially, with WEFAX for continuity reasons:

- S-VISSR Data Transmission, compatible with MDUS acquisition stations. Main features:

    - frequency: 1687.5 MHz; bandwidth: 2.0 MHz; polarisation: linear

    - antenna diameter ~ 3 m, G/T ~ 12 dB/K, data rate 660 kbps.

- WEFAX from FY-2 A/B, LRIT (Low Rate Information Transmission) from FY-2 C/D/E, similar to MSG, GOES, MTSAT and GOMS-N2.  Main features of LRIT:

    - frequency: 1691.0 MHz; bandwidth: 260 kHz; polarisation: linear

    - antenna diameter ~ 1 m, G/T ~ 3 dB/K, data rate 150 kbps.

FY-4

A second generation geostationary series, FY-4, in being defined.  It will be based on a 3-axis stabilized platform, with much improved payload in respect of FY-2.  Currently, it is thought to have two types of missions:

- one (FY-4A) with an advanced VIS/IR imager, an IR sounding spectrometer and a lightning mapper; to be launched around 2012;

- one (FY-4B) with a MW radiometer for all-weather sounding and precipitation; to be launched around 2015.

Feng-Yun means “Wind and Cloud”.  The “2” series is geostationary, the “1” and “3” series sunsynchronous.