Thirteenth USA/Europe Data Exchange Meeting

UK Met. Office Telecommunications Status Report

Prepared By:

David Tinkler

Network Manager 				Tel: 		+44 (0)1344 856053

Room R302B 					Fax:		+44(0)1344-856099

The UK Met. Office, 				e-mail: 	dtinkler@meto.gov.uk

London Road, Bracknell,

Berkshire, RG12 2SZ.

1.	Communication Links

Figure 1:	UK Met. Office Telecommunications Links

0

Figure 1 shows some of the UK Met. Office (UKMO) telecommunications links. The message switches TROPICS and NETTLE send and receive data over the links while the routers UKMO1 and UKMO6 provide the connections to the circuits or managed network services.

The Regional Meteorological Data Communications Network (RMDCN) is a managed IP network service provided by Equant for the WMO National Meteorological Services (NMSs) in WMO region VI. RMDCN also supports X25 encapsulated in IP.

UKMO is making increased use of Frame Relay connections and has links to both British Telecom (Link to Australian BoM) and Worldcom (Link to NESDIS and CMC) Frame Relay services.

Since the last meeting the following changes have taken place:

A router has replaced the multiplexer on the link to NOAA-OSO and the speed of the GTS link has been increased to 38.4 Kbits/Sec from 19.2Kbits/Sec.

The speed of the Frame Relay link to NESDIS has been upgraded to 256Kbits/Sec access speed and 64Kbits/Sec Committed Information rate (CIR) at the NESDIS end.

The Meteo-France link has been moved from a leased line connection onto RMDCN

The link to Bureau of Meteorology (BoM) in Australia was moved onto Frame Relay and now only runs TCP/IP.

The following sections contain more detailed information on some of the links.

1.1	Links to NOAA

There are two links to NOAA, a Frame Relay link to NOAA-NESDIS and a 64KBits/sec leased line to the NOAA Office of Systems Operations (OSO).

1.2.1	Links to NOAA-OSO

The link to NOAA-OSO is a 64Kbits/Sec leased line. There are two separate X25 data streams described in Table 1. The X25 data is encapsulated into IP and sent over the leased line using a router. The link utilisation is shown in Appendix A.

Speed Bits/sec

Protocol

NOAA-OSO System

UKMO

Data

| 38.4K | X25 | NOAA-OSO Telecommunications Gateway | TROPICS | GTS |
| --- | --- | --- | --- | --- |
| 4.8K | X25 | NOAA-OSO Telecommunications Gateway | TROPICS | T4 Fax Products |

Table 1:	Details of the links between UKMO and NOAA-OSO

1.2.2	Links to NOAA-NESDIS

The Frame Relay link to NOAA-NESDIS is currently being used for the transfers of satellite sounding data. Details on the link is shown in Table 2, the utilisation is shown in Appendix A and the estimated daily volumes of data is shown in Table 4.

| Speed Kbits/sec | Protocol | NOAA-NESDIS System | UKMO | Data |
| --- | --- | --- | --- | --- |
| 256 Access/ 64 CIR from NESDIS | FTP | CEMSCS | NETLINK | Satellite Sounding data and products, see Table 4 |
| 128 Access/ 16 CIR to NESDIS | FTP | CEMSCS | NETLINK | ERS-2 Data |

Table 2:	Details of the links between UKMO and NOAA-NESDIS

The utilisation graph in Appendix A shows that the majority of the file transfers over the link take place at the access speed of the circuit rather than the Committed Information Rate (CIR).

1.2	Links to ECMWF

The 2MBits/Sec link to ECMWF is used to transmit files of data between UKMO and ECMWF using TCP/IP protocols. This data includes GTS Data, numerical modelling products and satellite data. UKMO users can also submit jobs to run on the ECMWF Cray and access data on ECMWF computer systems using their MARS and METVIEW software.

1.3	Links to RMDCN

The link from UKMO to the RMDCN network has an access speed of 384Kbits/Sec. Permanent Virtual Circuits (PVCs) are established across RMDCN to other National Meteorological Services (NMSs) in WMO region VI. The details of some of these are shown below:

1.3.1	Links to Meteo-France

A PVC with a speed of 128Kbits/Sec out (UKMO to Meteo-France) and 64Kbits/Sec in (Meteo-France to UKMO) has been established.

The link is used for:

Exchange of GTS and Radar data.

Sending GOES and GMS satellite data to Lannion. This data is then sent from Lannion to METEOSAT for rebroadcast.

Transmission of satellite sounding data such as the SSM/I and ATOVS data is sent to Meteo-France using this link.

1.3.2	Links to DWD

A PVC with a speed of 48Kbits/Sec in both directions has been established between the UKMO and DWD. Tests of file transfers using FTP between the DWD and UKMO message switches have only been partially successful. An apparent problem with the FTP software on the UKMO message switch has been passed to the suppliers for investigation. In order to remove the leased line between UKMO and DWD an interim solution has been agreed where the GTS data will be sent using X25 encapsulated in IP over RMDCN

1.3.3	Other RMDCN Links

The UKMO has the following PVCs through RMDCN to other NMSs

| National Meteorological Service | Protocol | PVC Speed Kbits/Sec | Status |
| --- | --- | --- | --- |
| Ireland (Dublin) | FTP | 32 Both ways | Running in Parallel |
| Netherlands (De Bilt) | TCP Sockets | 24 Both ways | Operational |
| Norway (Oslo) | TCP Sockets | 16 Both ways | Operational |
| Denmark | TCP Sockets | 16 Both ways | Operational |
| Italy (Rome) | FTP | 16 Both ways | Operational |
| Belgium (Brussels) | FTP | 16 Both ways | Operational |
| Portugal | X25 Over IP | 16 Both ways | Test |
| Spain | X25 Over IP | 32 Both ways | Not Started |
| Iceland (Reykjavik) | X25 Over IP | 16 Both ways | Test |

Table 3:	Details of RMDCN Links to other NMSs in WMO Region VI

1.4	Other International Links

In an effort to gain routine, reliable access to global imagery from geostationary satellites, the Met. Office has implemented links as described briefly below. The approach has generally sought to identify mutual benefit from the exchange of satellite data and, hence, sharing of the costs involved.

1.4.1	Australian Met. Service

A Frame Relay link has been established between the UK Met. Office and the Australian Bureau of Meteorology (BoM) in Melbourne. The access speed is 128Kbits/Sec at each end and the CIR is 16Kbits/Sec in both directions. The link is being used for the exchange of data in WMO bulletin format using TCP sockets with TROPICS and files of satellite data (METEOSAT from UK and GMS from Australia) with NETLINK.

1.4.2	Canadian Met. Service

A 32 Kbits/Sec Frame Relay link has been established between the UK Met. Office and the Canadian Met. Service (CMC) in Dorval. It is being used for the exchange of data in WMO bulletin format with TROPICS and files of satellite data (METEOSAT from UK and GOES from Canada) with NETLINK.

2.	Met. Office Communication Computer Systems

There are two computer systems at the Met. Office that are used to send data to customers. The TROPICS message switch handles all communications involving X25 protocols and WMO messages, it also has the ability to send WMO messages using FTP or TCP/IP sockets. The NETLINK system has the ability to send files using FTP.

In general TROPICS is used for the transfer of low volume data, typically from 100 Bytes to 50 Kbytes, where high speed data switching is important. NETLINK is used for the distribution of higher volume data at slower data switching speeds.

2.1	NETLINK

The NETLINK computer system is responsible for the operational transfer of files of data between computers on the Met. Office Central Data Network (CDN) and computers on external networks. It consists of two identical Compaq Alpha Computers running OpenVMS, the main system is called NETTLE and its backup is called TEAZLE.

The NETTLE system is a Compaq Alpha 1200/533 with 128 Mbytes of memory running OpenVMS V7.2 operating system. Currently the system is switching 5000 files per day, which is about 4.5 Gbytes of data.

Table 4 shows the data and its associated volumes sent to NETTLE over the Frame Relay link at present.

| Description | Daily Data Volume (Mbytes) |
| --- | --- |
| From NESDIS to UKMO |  |
| SSM/I Brightness temperatures | 231.0 |
| Level 1B TOVS Data | 150.0 |
| ATOVS Level 1B data | 141.5 |
| Ice/snow gridded field derived from SSM/I data | 80.0 |
| AVHRR Products on a HIRS Grid | 31.0 |
| AVHRR 100km and 50Km SSTs | 7.9 |
| AVHRR Mapped Global Area Composites | 6.3 |
| Ozone Retrieved profiles and/or total column from SBUV on NOAA | 1.1 |
| GOES cloud-tracked VIS/IR/WV Winds | 1.2 |
| Total from NESDIS to UKMO | 650.0 |

| Description | Daily Data Volume (Mbytes) |
| --- | --- |
| From UKMO to NESDIS |  |
| ERS-2 data | 9 |
| Total from UKMO to NESDIS | 9 |

Table 4:	Data volumes on the present UKMO-NESDIS Link

Table 5 shows the anticipated increase in data volumes over the next few years based on the data in the present European requirements list and the expected increase in link capacity required to be able to accommodate that data. It shows that the link speed will need upgrading in the near future to accommodate the Quikscat and NOAA-L data.

| Date | Data Volume (Mbytes) | Average Link Speed (Kbits/Sec) | Link CIR (Kbits/Sec) | Link Access Speed (Kbits/Sec) | Additional Data Requirement |
| --- | --- | --- | --- | --- | --- |
| 01/07/2000 | 650.0 | 60.2 | 64 | 256 |  |
| 01/10/2000 | 1003.9 | 93.0 | 128 | 512 | Quickscat, NOAA-L |
| 01/01/2001 | 1003.9 | 93.0 | 128 | 512 |  |
| 01/04/2001 | 1003.9 | 93.0 | 128 | 512 |  |
| 01/07/2001 | 1444.7 | 133.8 | 192 | 768 | SSM/IS and Airs |
| 01/10/2001 | 1444.7 | 133.8 | 192 | 768 |  |

Table 5	Future Data Volumes on the NESDIS Frame Relay Link.

2.2	TROPICS

The TROPICS system at Bracknell is a multi-computer Tandem K2000 Himalaya system; one computer system called TROPICS-A is used to meet our national and international message switching needs. As such it supports some communications links ranging from low speed asynchronous connections to 64KBits/s synchronous lines although increasing the connections are made through routers and data sent out via the Ethernet LAN interfaces.

The main messages are transferred using FTP, TCP sockets and X25 but there is also support for messaging via telex, ISO TP4 (over LAN) and X.400.

The TROPICS-A message switch is an eight processor system with 128MBytes of memory per processor running the non-stop Kernel D45 release of operating system. Currently the system is switching 1.7 million half-messages per day, which is about 3 Gbytes. Peak rates are around 45 half-messages per second, which is about 133 Kbytes/sec.

With regard to the "alpha-numeric observational" and GRIB channels between Bracknell and NOAA-OSO the following tables show the change in daily traffic rates since the last meeting: -

| Date | Washington to Bracknell |  | Bracknell to Washington |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| April 1996 | 9.2 | 14256 | 9.1 | 13651 |
| May 1997 | 9.6 | 15535 | 10.4 | 14662 |
| May 1998 | 13.2 | 16524 | 11.0 | 15884 |
| May 1999 | 17.5 | 18036 | 12.4 | 19199 |
| June 2000 | 21.3 | 22123 | 14.2 | 24359 |

Table 6 GTS Link Alpha-Numeric Channel

| Date | Washington to Bracknell |  | Bracknell to Washington |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| April 1996 | 72.8 | 19455 | 75.9 | 23463 |
| May 1997 | 75.6 | 19220 | 77.3 | 23007 |
| May 1998 | 80.1 | 19932 | 66.5 | 16259 |
| May 1999 | 78.7 | 19876 | 68.7 | 16343 |
| June 2000 | 83.1 | 20389 | 75.3 | 17763 |

Table 7 GTS Link GRIB Channel

Appendix A		Utilisation of UK Met. Office Links to NOAA

A1	Links to NOAA-OSO

A1.1	Utilisation of GTS Link between UKMO and NOAA-OSO

A1.2	Utilisation of the T4 Chart Link between UKMO and NOAA-OSO

A2	Frame Relay Link to NOAA-NESDIS