# NWS eMail Data Input System
(EDIS)

The NWS eMail Data Input System (EDIS) is a procedure that allows designated meteorological data providers the capability to input weather bulletins to the National Weather Service Telecommunication Gateway (NWSTG) - also known as the Regional Telecommunication Hub (RTH), via eMail.

### This capability requires prior establishment of procedural controls.
(Go to EDIS Access Request Information)

NOTICE:   This is an automatic process and every message submission will be disseminated world-wide as determined by the routing established for the WMO abbreviated heading you used. The submitting NMC or organization is responsible for the content of the bulletin(s).

### Guidance for using EDIS

General Rules

- All input is ALPHANUMERIC ONLY in the current version.

- The WMO Bulletin - (See WMO Message Structure for Full Details)

  - The eMail information submitted is the WMO bulletin

  - The content of the WMO bulletin is:

    - An abbreviated heading line

      - T1T2A1A2ii CCCC YYGGgg [BBB]

    - Message text

- The email is submitted to the RTH via EDIS using a predefined eMail address.

EDIS Submission Rules and Definitions

- Messages must be sent in ASCII (plain text or alphanumeric code form).

- Two options are available for sending messages:
    a. Messages may be sent as an attachment to the email (the attachment must have an extension .txt), or
    b. Messages may be sent in the body of an eMail.

- The body of an email shall remain empty if attachments are used.

- One to five attachments are allowed per eMail.

- An attachment or body may contain one or more messages.

- The "subject line" shall contain: EDIS Message Input .

- The WMO abbreviated heading line and message text are the only required data for this email input form.

- The WMO abbreviated heading is entered in capital letters.

- The format of the WMO Abbreviated Heading Line is: T1T2A1A2ii   CCCC   [NNNXXX] YYGGgg   (BBB) .

- The data time fields in the abbreviated heading must be current and meet the WMO standards of the Manual 386 Part II.

- The Pxx option of the BBB optional group is not allowed.

- The CCCC is the location identifier of origin of the "MESSAGE TEXT" being entered.

- The abbreviated heading T1T2A1A2ii CCCC must be previously defined at the RTH and will be validated against the NWS Switching Directory.

- No ASCII character control notations are required except for the report separator [ = ] signal.

- Each message must be ended with NNNN.

An Example of the Attachment or eMail Body:

SMMF01 TFFR 01000

AAXX 01124/

78890 36/// /0508 10244 20209 30120 40158 53016/ 333

10281 20228 58005=

NNNN

NTUS99 KNCF 031430

THIS IS A COMMUNICATIONS TEST MESSAGE ORGINATING FROM THE NMTR

SYSTEM IN SILVER SPRING  IT IS SET TO TRANSMIT EVERY MINUTE

24 HOURS PER DAY, SEVEN DAYS PER WEEK

VIA A CRON JOB ON THE DS1-NMTR.ER SERVER.

NNNN

EDIS Access Request Information:
(please copy this information into your returning eMail)

EDIS Access Request Form

1. Name of NMC or sender: ____________________________________________

2. From eMail Address: _______________________________________________

3. Name of Contact(s): _______________________     ______________________

4. Phone number(s):   _______________________     _______________________

5. Name of Shift Observer(s): ____________________     ____________________

6. Phone Number(s): _______________________     _______________________

7. Name of Chief Forecaster: __________________________________________

6. Phone Number(s): _______________________     _______________________

eMail this information to the Data Management Group 

Please include the text " EDIS INPUT REQUEST " in the subject line of your eMail.

Note: Please copy and paste the information above into the body of your return eMail.

You will receive a return eMail containing the eMail address to use for sending the data within 3 working days of receipt.

To establish access to the EDIS, an email exchange between the TOC Data Management Group (DM) and the requesting data provider is required. The exchange of information establishes a valid ID necessary to gain access to the system. The data provider submits the meteorological data via email which consists of a standard WMO bulletin (i.e. the WMO abbreviated heading and text or body of the bulletin). The email is sent to a designated Gateway mailbox. The email is automatically processed and built into a file. The file is then placed into AFS for ingest by the WDE (see the Weather Data Exchange (WDE) System documentation). The WDE stores and routes the bulletin to the NWS switching system for distribution. All email messages must be sent in plain ASCII text format. Each message is ended with NNNN.