Seventeenth NorthAmerica/Europe Data Exchange Meeting

UK Met Office Telecommunications Status Report 2004

Prepared By:

David Hennings

Message Switching Team  				Tel: 	+44 (0)1392 884092

Met Office   						Fax:	+44(0)1392-885681

Fitz roy Road 				            e-mail:david.hennings@metoffice.com

Exeter,  Devon

EX1 3PB, UK

1.	Communication Links

Figure 1	UK Met Office Telecommunication links

Figure 1 shows some of the UK Met Office telecommunications links. The message switches FROST, TROPICS and NETlink  send and receive data arriving over the links while the routers EXERT5 (and  EXERT6) provide the connections to the circuits or managed network services.

The TROPICS message switch has mostly been replaced by the new FROST message switch .

The Regional Meteorological Data Communications Network (RMDCN) is a managed IP network service provided by Equant for the WMO National Meteorological Services (NMSs) in WMO region VI. RMDCN also supports X25 encapsulated in IP (XOT).

The Met Office has Frame Relay links with both British Telecom (Link to Australia BoM, and NOAA-OSO) and MCI Worldcom (Link to NOAA-NESDIS and CMC).

The following sections contain more detailed information on some of the links.

1.1	Links to NOAA

There are two links to NOAA, one to NOAA-NESDIS and a second to the NOAA Office of Systems Operations (NOAA-OSO).

1.2.1	Links to NOAA-OSO

The link to NOAA-OSO is a Frame relay line. There are two separate sockets data streams described in Table 1. The link utilisation is shown in Appendix A.

| Protocol | NOAA-OSO System | Met Office | Data |
| --- | --- | --- | --- |
| TCP Sockets | NOAA-OSO Telecommunications Gateway | FROST | GTS |
| TCP Sockets | NOAA-OSO Telecommunications Gateway | FROST | T4 Fax Products |

Table 1	Details of the links between METO and NOAA-OSO

(The leased line between Bracknell and Washington has now been replaced with a Frame Relay connection operating from Exeter.)

1.2.2	Links to NOAA-NESDIS

The Frame Relay link to NOAA-NESDIS is used for the transfers of satellite sounding data. Details on the link is shown in Table 2, the utilisation is shown in Appendix A and the daily volumes of data is shown in Table 4.

| Speed Kbits/sec | Protocol | NOAA-NESDIS System | Met Office | Data |
| --- | --- | --- | --- | --- |
| 512 Access/ 128 CIR from NESDIS | FTP | CEMSCS | NETLINK | Satellite Sounding data and products, see Table 4 |
| 512 Access/32CIR to NESDIS | FTP | CEMSCS | NETLINK | ERS-2 Data |

Table 2	Details of the links between Met Office and NOAA-NESDIS

1.2	Links to ECMWF

The 2MBits/Sec link to ECMWF is used to transmit files of data between Met Office and ECMWF using TCP/IP protocols. This data includes GTS Data, numerical modelling products and satellite data. Met Office users can also submit jobs to run on the ECMWF Supercomputer and access data on ECMWF computer systems using their MARS and METVIEW software.

1.3	Links to RMDCN

The link from Met Office to the RMDCN network has an access speed of  512 Kbits/S (ISDN backup 384Kbits/S). Permanent Virtual Circuits (PVCs) are established across RMDCN to other National Meteorological Services (NMSs) in WMO region VI. The details of some of the most used these are described in this section.

1.3.1	Links to Météo-France

PVC with a speed of 256Kbits/Sec out (Met Office to Météo-France) with agreed upgrade to 384Kbits/S, and 128Kbits/Sec in (Météo-France to Met Office).

The link is used for:

- Exchange of GTS and Radar data.

- Sending GOES and GMS satellite data to Lannion. This data is then sent from Lannion to METEOSAT for rebroadcast.

- Transmission of Met Office NIMROD products.

- Transmission of NOAA satellite sounding data such as the SSM/I and ATOVS data .

- Receipt of SAF OCEAN/ SST and ICE (planned) products. Poorman’s Ensemble data and NWP Precpitiation

1.3.2	Links to DWD

A PVC with a speed of 64Kbits/Sec in both directions has been established between the Met Office and DWD to exchange GTS products.

1.3.3	Other RMDCN Links

The MET OFFICE has the following PVCs through RMDCN to other NMSs

| National Meteorological Service | Protocol | PVC Speed Kbits/Sec | Status |
| --- | --- | --- | --- |
| Belgium (Brussels) | FTP | 32 Both ways | Operational |
| Denmark | TCP Sockets | 32 Both ways | Operational |
| DWD | FTP | 64 Both ways | Operational |
| Iceland (Reykjavik) | FTP | 8 Both ways | Operational |
| Ireland (Dublin) | FTP | 64 Both ways | Operational |
| Italy (Rome) | FTP | 16 Both ways | Operational |
| Météo-France | FTP | 128 in 256 out [384*] | Operational |
| Russian Federation (Moscow) | FTP | 8 in/64 out* | Operational |
| Netherlands (De Bilt) | TCP Sockets | 64 Both ways | Operational |
| Norway (Oslo) | TCP Sockets | 32 Both ways | Operational |
| Portugal | FTP | 32 Both ways | Operational |
| Spain | TCP Sockets | 32 Both ways | Operational |

Table 3	Details of RMDCN Links to other NMSs in WMO Region VI

- United Kingdom

| PVC to | CIR (kbps) in | CIR (kbps) out | NRT (ms) specified | Service (%) availability |
| --- | --- | --- | --- | --- |
| Belgium | 32 | 32 | 110 | 99.5 |
| Denmark | 32 | 32 | 130 | 99.5 |
| France | 128 | 256 | 170 | 99.5 |
| Germany | 64 | 64 | 130 | 99.5 |
| Iceland | 8 | 8 | 170 | 99.5 |
| Ireland | 64 | 64 | 90 | 99.5 |
| Italy | 16 | 16 | 130 | 99.5 |
| Netherlands | 64 | 64 | 140 | 99.5 |
| Norway | 32 | 32 | 130 | 99.5 |
| Portugal | 32 | 32 | 220 | 99.5 |
| Russian Federation | 8 | 64 | - | 99.5 |
| Spain | 32 | 32 | 140 | 99.5 |

Table 4.  Details of RMDCN Links from http://rmdcn.ecmwf.int/RMDCN.html

1.4	Other International Links

In an effort to gain routine, reliable access to global imagery from geostationary satellites, the Met Office has implemented links as described briefly below. The approach has generally sought to identify mutual benefit from the exchange of satellite data and, hence, sharing of the costs involved.

1.4.1	Australian Met. Service

A Frame Relay link has been established between the Met Office and the Australian Bureau of Meteorology (BoM) in Melbourne. The access speed is 256Kbits/Sec at each end and the CIR is 64Kbits/Sec in both directions. The link is being used for the exchange of data in WMO bulletin format using TCP sockets with FROST, files of satellite imagery (METEOSAT from UK and GMS from Australia) and ATOVS (HIRS) level 1D data  with NETLINK.

1.4.2	Canadian Met. Service

A 32 Kbits/Sec Frame Relay link has been established between the Met. Office and the Canadian Met. Service (CMC) in Dorval. It is being used for the exchange of data in WMO bulletin format with FROST and files of satellite data (METEOSAT from UK and GOES from Canada) with NETLINK.

2.	Met. Office Communication Computer Systems

There are two main computer systems at the Met Office that are used to send data to customers. The HP-UNIX-based FROST message switch sends WMO messages using FTP or TCP/IP sockets. FROST is planned to have some X25 Over TCP/IP capability later this year though most remaining X25 customers are encouraged to migrate to FTP or TCP sockets.

The TROPICS system, which is shortly to be retired, handles communications using direct X25 links and X25 over TCP/IP (including GTS traffic to CMC).

The NETLINK system exchanges files using FTP. In general FROST is used for the transfer of low volume data, typically from 100 Bytes to 50 Kbytes, where high speed data switching is important. NETLINK is used for the distribution of higher volume data at slower data switching speeds.

2.1	NETLINK

The NETLINK computer system is responsible for the operational transfer of files of data between computers on the Met. Office Central Data Network (CDN) and computers on external networks. It consists of two identical Compaq Alpha Computers running OpenVMS, the main node is called NETTLE and its backup is called TEAZLE.

Each NETLINK node is a HP Alpha 1200/533 with 256 Mbytes of memory running OpenVMS V7.3-1 operating system. Currently the system is switching 44,000 files per day, which is about 28 Gbytes of data.

Planning is in its early stages to replace the functionality of NETLINK with a new platform which integrates better with the new FROST system sometime in 2006.

In the mean time, to handle anticipated loading from Meteosat Second Generation (MSG) and some of the new satellite product sets, the nodes will be replaced by more powerful OpenVMS (HP Alpha) platforms.

Table 5 below, shows the data and its associated volumes sent to NETTLE over the Frame Relay link at present.

| Requirements Description | Daily Data Volume (MBytes) |
| --- | --- |
| NOAA AMSU-A Level 1B | 92 |
| NOAA AMSU-ABLevel 1B | 330 |
| DSMP SSM/I Brightness temperatures | 220 |
| NOAA TOVS MSU Level 1B | 1 |
| NOAA TOVS SSU Level 1B | 13 |
| QuikScat Scatterometer data | 209 |
| NOAA AVHRR Products on a HIRS Grid. | 35 |
| NOAA AVHRR 100km and 50Km SSTs | 7.9 |
| NOAA AVHRR Mapped Global Area Composites | 6.3 |
| NOAA Ozone Retrieved profiles and/or total column from SBUV | 1.2 |
| AQUA AIRS Brightness temperatures | 300 |
| AQUA AMSU-A Brightness temperatures | 11.5 |
| NOAA TOVS HIRS Level 1B | 45.7 |
| NOAA ATOVS HIRS Level 1B | 206.7 |
| GOES cloud-tracked VIS/IR/WV Winds | 8.4 |
| TOTAL | 1487.7 |

Table 5	Data Volumes from NESDIS to Met Office

Table 7 shows the anticipated increase in data volumes over the next year based on the data in the present European requirements list.

The expected increase in link capacity required to be able to accommodate that data is also shown.

The present link runs well below the frame relay access speed of 512Kb/s.

Currently the average is ~10% greater than the committed information rate (CIR) of 128 Kb/s, and will rise to +30% above CIR later this year with SSM/IS.

This speed could drop to the CIR if the supplier’s network becomes congested.

It has been previously proposed that the link is operated at a CIR similar to the average daily data rate with an access speed four times that value.

As long at the network continues to operate at the access speed there should be sufficient bandwidth and if the network becomes congested and the data rate drops we can pay to increase the CIR (up to 80% of the access speed).

At an earlier meeting of this Data Exchange group, a document containing options for backing up the data on the NESDIS link was prepared for discussion. The option selected was to routinely copy high priority data (defined as data crucial to operations) from the CEMSCS system at NESDIS to the NOAA-OSO FTP Server. In the event of a problem with the NESDIS link MET OFFICE would pull the data from this server over the NOAA-OSO GTS link and then pass it to the NETLINK system for processing.

The European Met Services provided information on the priority for the data sets on the dedicated link shown in Table . From the information provided it was decided to proceed with backing up the ATOVS level 1B data sets. Access to files on the NOAA-OSO server from Bracknell over the NOAA-OSO GTS link was successfully tested (in 2002). There is a requirement to reacquire the knowledge as to how this procedure works by Met Office staff working from Exeter.

| Data Set | MET OFFICE | ECMWF | Météo-France |
| --- | --- | --- | --- |
| ATOVS Level 1b | 1 | 1 | 1 |
| AVHRR 100/50 km SST | 3 | 4 | 3 |
| AVHRR Mapped Images | 2 | 4 | 3 |
| AVHRR products for HIRS | 3 | 3 | 3 |
| GOES AMVs | 1 | 1 | 3 |
| SSM/I Sea Ice | 4 | 1 | 4 |
| TOVS Level 1B | 4 | 1 | 4 |
| SBUV Ozone | 3 | 2 | 3 |
| Quikscatt | 1 | 1 | 2 |
| SSM/I BTs | 1 | 1 | 2 |

Table 6 Priorities for Data Sets on Exeter-NESDIS dedicated link

Categories are:

- High Priority (crucial for operations)

- Medium Priority (operational or pre-operational use but not critical

- Low Priority (research datasets)

- No Requirements

| Date | Data Volume (Mbytes) | Average Link Speed (Kbits/Sec) | Link CIR (Kbits/Sec) | Link Access Speed (Kbits/Sec) | Additional Data Requirement |
| --- | --- | --- | --- | --- | --- |
| 01/07/2000 | 650.0 | 61.6 | 64 | 256 |  |
| 01/10/2000 | 862.3 | 81.7 | 64 | 256 | NOAA-L |
| 01/01/2001 | 862.3 | 81.7 | 64 | 256 |  |
| 01/04/2001 | 862.3 | 81.7 | 64 | 256 |  |
| 01/07/2001 | 1043.5 | 98.9 | 128 | 512 | Quickscatt NOAA-L AVHRR |
| 01/10/2001 | 1073.5 | 101.8 | 128 | 512 | NOAA-L AVHRR |
| 01/04/2002 | 1073.5 | 101.8 | 128 | 512 |  |
| 01/07/2002 | 1473.3 | 139.7 | 128 | 512 | NOAA-M AMSU & AVHRR |
| 01/03/2004 | 1420 | 134.6 | 128 | 512 | AIRS and AMSU on AQUA |
| 01/08/2004 | 1728 | 163.8 | 128 | 512 | SSM/IS, |

Table 7.  Data Volumes on the NESDIS Frame Relay Link.

2.2	FROST

The new Met Office FROST system is based upon a pair of HP L3000 servers running HP-UX Operating System software configured to run as a main system and a hot standby. There is a separate HP A500 Test & Development server.

A dedicated gigabit Ethernet link between the systems is used for management traffic and data synchronisation.

Application software is based upon the Messir-Comm message switching package for WMO message switching supporting FTP and TCP sockets. This is integrated with Nexor Mailer/Mixer software for general message switching using X.400 and SMTP. Legacy X.25 communications is supported by a pair of CISCO 3640 routers hot-switchable between servers. Operations and management of the system is performed using client software running on standard desktop PCs augmented by integration of the system with existing Met Office Tivoli and OpenView management platforms.

When migrating the Washington link from TROPICS to FROST, protocols were changed to use TCP sockets rather than X25.

The data volumes between Washington and Exeter are shown in Tables 4; 5 & 6 from the Frost Message Switch.

| Date | Washington to Exeter |  | Exeter to Washington |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| April 1996 | 9.2 | 14256 | 9.1 | 13651 |
| May 1997 | 9.6 | 15535 | 10.4 | 14662 |
| May 1998 | 13.2 | 16524 | 11.0 | 15884 |
| May 1999 | 17.5 | 18036 | 12.4 | 19199 |
| June 2000 | 21.3 | 22123 | 14.2 | 24359 |
| April 2001 | 32.7 | 23011 | 16.4 | 28925 |
| June 2002 | 41 | 31354 | 18 | 32247 |
| April 2003 | 35.6 | 28259 | 37.3 | 29612 |
| May 2004 | 44.6 | 36961 | 17.1 | 29268 |

Table 8 GTS Link Alpha-Numeric Channel

| Date | Washington to Exeter |  | Exeter to Washington |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| April 1996 | 72.8 | 19455 | 75.9 | 23463 |
| May 1997 | 75.6 | 19220 | 77.3 | 23007 |
| May 1998 | 80.1 | 19932 | 66.5 | 16259 |
| May 1999 | 78.7 | 19876 | 68.7 | 16343 |
| June 2000 | 83.1 | 20389 | 75.3 | 17763 |
| April 2001 | 86.1 | 20740 | 86.6 | 20006 |
| June 2002 | 163 | 36186 | 119 | 24961 |
| April 2003 | 182 | 38834 | 130 | 27687 |
| May 2004 | 181.4 | 40803 | 164.3 | 29302 |

Table 59GTS Link GRIB Channel

| Date | Washington to Exeter |  | Exeter to Washington |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| April 2001 | 5.4 | 59 | 5.4 | 60 |
| June 2002 | 3 | 33 | 6 | 68 |
| April 2003 | 3.3 | 33 | 7.5 | 74 |
| May 2004 | 3.4 | 38 | 6.7 | 74 |

Table 10 GTS Link T4 Channel

The data volumes between Toulouse and Exeter are shown in Tables 7; 8 & 9 from the Frost Message Switch.

| Date | Toulouse to Exeter |  | Exeter to Toulouse |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| May 2004 | 39.6 | 16785 | 71.3 | 47490 |

Table 11 GTS Link Alpha-Numeric Channel

| Date | Toulouse to Exeter |  | Exeter to Toulouse |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| May 2004 | GRIB received on alpha channel |  | 98.2 | 27897 |

Table 12 GTS Link GRIB Channel

| Date | Toulouse to Exeter |  | Toulouse to Toulouse |  |
| --- | --- | --- | --- | --- |
|  | Data (Mb) | Msgs | Data (Mb) | Msgs |
| May 2004 | 18 | 1203 | 57.7 | 1476 |

Table 13 GTS Link T4 Channel

The data volumes between Washington and Bracknell are shown in Table7, Table8

& Table9.They show a large increase in GRIB data transferred, this is due to the addition of GRIB data exchange from model runs at 06Z and 18Z in the last year. The graph of link utilisation in Appendix A shows there are now daily four peaks which is the GRIB data exchange.

3	Met Office Relocation

The Met Office has moved to Exeter in the South West of the UK, around 170 miles from the previous location in Bracknell.

At the new site there are two separate IT Halls. Diversely routed communication ducts to each hall enable highly resilient communication circuits.

Migration of computer systems and circuits is complete with the present building in Bracknell having closed.

Appendix A.	Utilisation of UK Met. Office Links to NOAA

Utilisation of UK Met. Office Links

Following are utilisation graphs for the following links:

- Washingtom – NOAA OSO

- RMDCN

- Dorval – CMC

- ECMWF

Note on the graphs which follow:

We are currently in the process of investigating new network monitoring tools, as the previous product has been dropped by the supplier. The following graphs have been captured from a basic OpenView monitoring package and as such are not ideal.

A1	Links to NOAA-OSO

A1.1	Utilisation of GTS Link from MET OFFICE from NOAA-OSO

CIR marked as yellow horizontal line

A1.2	Utilisation of GTS Link from MET OFFICE to NOAA-OSO

CIR marked as yellow horizontal line.

A2.1	Frame Relay Link to NOAA-NESDIS

CIR 128 Kbits/s  (32K Octets)     [Access: 512Kbits/s]

The graph shows the input utilisation of the Frame Relay link to NESDIS relative to a bandwidth of 512Kbits/Sec which is the access speed of the frame relay circuit in April 2004. The peak usage exceeds the CIR of 32K octets/sec by a factor of two.

A2.1	Frame Relay Link from  NOAA-NESDIS

Well within the CIR of 32K octets/sec

A3	RMDCN

A4	Dorval CMC

A5	ECMWF (Rx)