STATUS OF NWP REQUIREMENTS AS RECORDED IN CEOS-WMO DATABASE

Response to Action on E.1.2.2  ATOVS level 1b data 
from 19th North-America Europe Data Exchange Meeting

Action: WMO will provide parties with the latest status of requirements and the appropriate references.

As indicated at the 19th meeting, the CEOS-WMO database on user requirements is available on line through the WMO Space Programme homepage within the WMO website.

The currently recorded requirements for temperature and humidity sounding for global and regional NWP are as follows  (extract from the table circulated at the 19th meeting):

| Requirement | Hor Res | Vert Res | Obs Cycle | Accuracy | Delay | Confidence | Use |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Atmospheric temperature profile - Higher stratosphere & mesosphere (HS & M) | 50 km / 500 km | 1 km / 3 km | 1 h / 12 h | 0.5 K / 5 K | 1 h / 4 h | Reasonable | Global NWP |
| Atmospheric temperature profile - Higher troposphere (HT) | 10 km / 500 km | 1 km / 3 km | 0.5 h / 12 h | 0.5 K / 3 K | 0.5 h / 2 h | Firm | Regional NWP |
| Atmospheric temperature profile - Higher troposphere (HT) | 50 km / 500 km | 1 km / 3 km | 1 h / 12 h | 0.5 K / 3 K | 1 h / 4 h | Firm | Global NWP |
| Atmospheric temperature profile - Lower stratosphere (LS) | 10 km / 500 km | 1 km / 3 km | 0.5 h / 12 h | 0.5 K / 3 K | 0.5 h / 2 h | Firm | Regional NWP |
| Atmospheric temperature profile - Lower stratosphere (LS) | 50 km / 500 km | 1 km / 3 km | 1 h / 12 h | 0.5 K / 3 K | 1 h / 4 h | Firm | Global NWP |
| Atmospheric temperature profile - Lower troposphere (LT) | 10 km / 500 km | 0.3 km / 3 km | 0.5 h / 12 h | 0.5 K / 3 K | 0.5 h / 2 h | Firm | Regional NWP |
| Atmospheric temperature profile - Lower troposphere (LT) | 50 km / 500 km | 0.3 km / 3 km | 1 h / 12 h | 0.5 K / 3 K | 1 h / 4 h | Firm | Global NWP |
| Specific humidity profile - Higher troposphere (HT) | 10 km / 100 km | 1 km / 3 km | 0.5 h / 12 h | 5 % / 20 % | 0.5 h / 2 h | Firm | Regional NWP |
| Specific humidity profile - Higher troposphere (HT) | 50 km / 250 km | 1 km / 3 km | 1 h / 12 h | 5 % / 20 % | 1 h / 4 h | Firm | Global NWP |
| Specific humidity profile - Lower troposphere (LT) | 10 km / 100 km | 0.4 km / 2 km | 0.5 h / 12 h | 5 % / 20 % | 0.5 h / 2 h | Firm | Regional NWP |
| Specific humidity profile - Lower troposphere (LT) | 50 km / 250 km | 0.4 km / 2 km | 1 h / 12 h | 5 % / 20 % | 1 h / 4 h | Firm | Global NWP |
| Specific humidity profile - Total column | 50 km / 500 km | / | 1 h / 12 h | 1 kg/m2 / 5 kg/m2 | 1 h / 4 h | Firm | Global NWP |

Requirements are subject to a regular Rolling Requirement Review process, which is led by the CBS/OPAG IOS Expert Team on the Evolution of the Global Observing System (ET-EGOS), currently chaired by Dr John Eyre (U.K.)

In the table above each criterion contains 2 values: a goal, and a threshold. This requirements database has just been modified to incorporate the concept of breakthrough, which will appear as an intermediate value between goal and threshold. This new breakthrough value needs to be determined by experts like the other two.

A call for update was recently sent to focal points of each application area, with the aim to implement the updates in advance of the 3rd ET-EGOS meeting on 9-13 July 2007.

Regarding Regional and Global NWP requirements for temperature and humidity profiles, and according to discussions with relevant experts during the past year, it is anticipated that through the updating process the “goal” delay value will be divided by 2 , i.e. becoming 30 mn for global NWP and 15 mn for Regional NWP.