Precipitation Processing System (PPS) Product Format Description

Digital Hybrid Scan Reflectivity (DHR)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 256-level digital |  |
| Time generated | Once every volume scan |  |
| Product size | Fixed at 85668 |  |
| Compression | Yes |  |
| Resolution | 1-degree x 1-km |  |
| Reflectivity values | -32.0 and +94.5 dBZ, in 0.5dBZ increments |  |
| Data level codes | 0 – below threshold 1 – range folded 2-255 – reflectivity values |  |
| AWIPS ID | WSRDHRxxx | xxx is site ID for originating WFO |
| Description/Purpose: The reflectivity values are assembled from the Hybrid Scan array, which is generated on the fly. The Product Symbology block contains two layers: Layer 1 (packet code 16) • Reflectivity data – a fixed 230 x 360 array Layer 2 (packet code 1) • Precip Status Message values (CHAR) • Adaptable parameters (CHAR) • Supplemental data (CHAR) • Bias Table (CHAR) |  |  |

The following table provides a detailed specification of the DHR product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a half-word (INT*2) is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 32 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 85668 | 1 | CCR#NA04-32201 |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD)

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 32 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 | N/A | 0 | N/A |  |
| 28 | Not used | INT*2 | N/A | 0 | N/A |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Minimum DHR data level | INT*2 | dBZ | -32.0 | 0.1 |  |
| 32 | Data level increment | INT*2 | dBZ | 0.5 | 0.1 |  |
| 33 | Number of data levels | INT*2 | N/A | 256 | 1 |  |
| 34 | Not used | INT*2 | N/A | 0 | N/A |  |
| 35 | Not used | INT*2 | N/A | 0 | N/A |  |
| 36 | Not used | INT*2 | N/A | 0 | N/A |  |
| 37 | Not used | INT*2 | N/A | 0 | N/A |  |
| 38 | Not used | INT*2 | N/A | 0 | N/A |  |
| 39 | Not used | INT*2 | N/A | 0 | N/A |  |
| 40 | Not used | INT*2 | N/A | 0 | N/A |  |
| 41 | Not used | INT*2 | N/A | 0 | N/A |  |
| 42 | Not used | INT*2 | N/A | 0 | N/A |  |
| 43 | Not used | INT*2 | N/A | 0 | N/A |  |
| 44 | Not used | INT*2 | N/A | 0 | N/A |  |
| 45 | Not used | INT*2 | N/A | 0 | N/A |  |
| 46 | Not used | INT*2 | N/A | 0 | N/A |  |
| 47 | Maximum Reflectivity | INT*2 | dBZ | 95 | 1 |  |
| 48 | Date of Scan | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 49 | Average Time of Hybrid Scan | INT*2 | Minutes | 1 – 1439 | 1 |  |
| 50 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 51 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Product Symbology block | INT*4 | Half-words | 60 | 1 |  |
| 57-58 | Offset to Graphic Attributes block | INT*4 | Half-words | 0 | 1 |  |
| 59-60 | Offset to Graphic Alphanumeric block | INT*4 | Half-words | 0 | 1 |  |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” (Sheet 8)

Fig 3-8b “Text and Special Symbol Packets”

Fig 3-11c “Digital Radial Data Array Packet”

2620003 (ICD for Product Specification):

Appendix C, Format VIII “DHR and ….”

| HALF-WORD | FIELDNAME | TYPE |  | UNITS |  | VALUE |  | PRECISION/ ACCURACY |  | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 62 | Block ID | INT*2 |  | N/A |  | 1 |  | N/A |  |  |
| 63-64 | Length of Block | INT*4 |  | Bytes |  | 85548 |  | 1 |  | CCR#NA04-32201 |
| 65 | Number of Layers | INT*2 |  | N/A |  | 2 |  | 1 |  |  |
| 66 | Layer Divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 67-68 | Length of Data Layer not including layer divider and layer length | INT*4 |  | N/A |  | 84974 |  | 1 |  |  |
|  | BEGINNING OF THE DHR REFLECTIVITY DATA LAYER |  |  |  |  |  |  |  |  |  |
| 69 | Packet Code | INT*2 |  | N/A |  | 16 |  | N/A |  |  |
| 70 | Index of First Range Bin | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 71 | Number of Range Bins | INT*2 |  | N/A |  | 230 |  | 1 |  |  |
| 72 | I Center of Sweep | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 73 | J Center of Sweep | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 74 | Range Scale Factor (230/#Bins) | Scaled Integer |  | N/A |  | 1.0 |  | 0.001 |  |  |
| 75 | Number of Radials | INT*2 |  | N/A |  | 360 |  | 1 |  |  |
| 76 | Number of Bytes in Radial | INT*2 |  | N/A |  | 230 |  | 1 |  | }Repeat }for |
| 77 | Radial Start Angle | Scaled Integer |  | Degrees |  | 0.0 to 359.9 |  | 0.1 |  | } |
| 78 | Radial Delta Angle | Scaled Integer |  | Degrees |  | 1.0 |  | 0.1 |  | }Each |
| 79 | Level 0 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level 1 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
| 80 | Level 2 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level 3 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
| … | … | … |  | … |  | … |  | … |  | } |
| … | … | … |  | … |  | … |  | … |  | } |
| 193 | Level (N-1) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level (N) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }Row |
| … | … | … |  | … |  | … |  | … |  |  |
| … | … | … |  | … |  | … |  | … |  | }Between |
| … | … | … |  | … |  | … |  | … |  | } |
| … | … | … |  | … |  | … |  | … |  | } |
| 42555 | Level(N-1) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }Radial 2 and |
|  | Level(N) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }Radial 360 |
|  | BEGINNING OF THE DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42556 | Text layer divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 42557-42558 | Layer length not including layer divider and layer length | INT*4 |  | N/A |  | 552 |  | 1 |  | CCR#NA04-32201 |
| 42559 | Text layer packet code | INT*2 |  | N/A |  | 1 |  | N/A |  |  |
| 42560 | Length of text layer in bytes | INT*2 |  | N/A |  | 548 |  | 1 |  | CCR#NA04-32201 |
| 42561 | I Starting Point | INT*2 |  | Km/4 or Pixels |  | 0 |  | 1 |  |  |
| 42562 | J Starting Point | INT*2 |  | Km/4 or Pixels |  | 0 |  | 1 |  |  |
|  | BEGINNING OF PRECIP STATUS MESSAGE SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42563-42566 | Precip Status Message (PSM) header | CHAR*8 |  | N/A |  | “PSM( 6)” (space padded) |  | N/A |  | Corrections to original description (no format change) |
| 42567-42570 | Current Date Precip Function Ran | CHAR*8 |  | JULIAN DAYS |  | 0 – 99999 |  | 1 |  |  |
| 42571-42574 | Current Time Precip Function Ran | CHAR*8 |  | SEC |  | 0 – 86399 |  | 1 |  |  |
| 42575-42578 | Last Date Precip Detected | CHAR*8 |  | JULIAN DAYS |  | 0 – 99999 |  | 1 |  |  |
| 42579-42582 | Last Time Precip Detected | CHAR*8 |  | SEC |  | 0 – 86399 |  | 1 |  |  |
| 42583-42586 | Current Precip Category | CHAR*8 |  | N/A |  | 0 to 2 |  | 1 |  | Correction to original description (no format change) |
| 42587-42590 | Previous Precip Category | CHAR*8 |  | N/A |  | 0 to 2 |  | 1 |  | Correction to original description (no format change) |
|  | BEGINNING OF THE EPRE ADAPTATION DATA SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42591-42594 | Adaptation data header | CHAR*8 |  | N/A |  | “ADAP(32)” |  | N/A |  | CCR#NA04-32201 |
| 42595-42598 | Width of radar beam | CHAR*8 |  | Deg |  | “XXXXX.XX” (space padded) Range: 0.80 to 1.00 Default: 0.90 |  | 0.01 |  |  |
| 42599-42602 | Blockage Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 42603-42606 | Clutter Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 42607-42610 | Weight Threshold | CHAR*8 |  | % |  | “XXXXX.XX” Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 42611-42614 | Full Hybrid Scan Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 90.00 to 100.00 Default: 99.70 |  | 0.01 |  |  |
| 42615-42618 | Low Reflectivity Threshold | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: -40.00 to -20.00 Default: -32.00 |  | 0.01 |  |  |
| 42619-42622 | Rain Detection Reflectivity Threshold | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 20.00 |  | 0.01 |  |  |
| 42623-42626 | Rain Detection Area Threshold | CHAR*8 |  | Km2 |  | “XXXXX.XX” (space padded) Range: 0.00 to 82800.00 Default: 80.00 |  | 0.01 |  |  |
| 42627-42630 | Rain Detection Time Threshold | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 1440.00 Default: 60.00 |  | 0.01 |  |  |
| 42631-42634 | Z-R Mult. Coef. | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 50.00 to 500.00 Default: 300.00 |  | 0.01 |  |  |
| 42635-42638 | Z-R Power Coef. | CHAR*8 |  | N/A |  | “XXXXX.XX” Range: 1.00 to 2.00 Default: 1.40 |  | 0.01 |  |  |
| 42639-42642 | Min. Refl. to Convert to Rate | CHAR*8 |  | dBZ |  | “XXXXX.XX” Range: -32.00 to +20.00 Default: 0.00 |  | 0.01 |  |  |
| 42643-42646 | Max. Refl. to Convert to Rate | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: 50.00 to 90.00 Default: 70.00 |  | 0.01 |  |  |
| 42647-42650 | Number of Exclusion Zones | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 0.00 to 20.00 Default: 0.00 |  | 1.00 |  | Correction to original description (no format change) |
|  | BEGINNING OF THE RATE ALGORITHM ADAPTATION DATA SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42651-42654 | Max Storm Speed | CHAR*8 |  | M/S |  | “XXXXX.XX” (space padded) Range: 10.00 to 40.00 Default: 25.00 |  | 0.01 |  | CCR#NA04-32201 |
| 42655-42658 | Thresh. Max Time Difference | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 15.00 |  | 0.01 |  | CCR#NA04-32201 |
| 42659-42662 | MINS. Area Time Continuity | CHAR*8 |  | KM**2 |  | “XXXXX.XX” (space padded) Range: 50.00 to 1000.00 Default: 200.00 |  | 0.01 |  | CCR#NA04-32201 |
| 42663-42666 | Time Continuity Parameter #1 | CHAR*8 |  | 1/HR |  | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 24.00 |  | 0.01 |  | CCR#NA04-32201 |
| 42667-42670 | Time Continuity Parameter #2 | CHAR*8 |  | 1/HR |  | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 13.20 |  | 0.01 |  | CCR#NA04-32201 |
| 42671-42674 | Max. Rate Echo Area Change | CHAR*8 |  | KM**2/HR |  | “XXXXX.XX” (space padded) Range: 20.00 to 700.00 Default: 200.00 |  | 0.01 |  | CCR#NA04-32201 |
| Note: as a result of implementing this CCR the halfword addresses of all remaining fields have changed |  |  |  |  |  |  |  |  |  |  |
| 42651-42654 | Range Cut-Off | CHAR*8 |  | KM |  | “XXXXX.XX” (space padded) Range: 0.00 to 230.00 Default: 230.00 |  | 1.00 |  | Correction to original description (no format change) |
| 42655-42658 | Range Effect Coeff. #1 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 0.00 to 3.00 Default: 0.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 42659-42662 | Range Coeff. Coeff. #2 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 1.00 to 10.00 Default: 1.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 42663-42666 | Range Coeff. Coeff. #3 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 0.00 to 1.00 Default: 0.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 42667-42670 | Min. Precip. Rate for inclusion | CHAR*8 |  | MM/HR |  | “XXXXX.XX” (space padded) Range: 0.00 to 10.00 Default: 0.00 |  | 0.10 |  | Correction to original description (no format change) |
| 42671-42674 | Max Precip. Rate allowed | CHAR*8 |  | MM/HR |  | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 103.80 |  | 0.10 |  | Corrections to original description (no format change) |
|  | BEGINNING OF THE ACCUM ALGORITHM ADAPTATION DATA SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42675-42678 | Thresh. Elapsed Time to Restart | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 45.00 to 60.00 Default: 60.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42679-42682 | Max. Time for Interpolation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 15.00 to 60.00 Default: 30.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42683-42686 | Min. Time in Hourly Period | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 60.00 Default: 54.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42687-42690 | Threshold Hourly Outlier | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 50.00 to 800.00 Default: 400.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42691-42694 | End Time Gage Accumulation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 59.00 Default: 0.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42695-42698 | Max Period Accum Value | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 50.00 to 400.00 Default: 400.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42699-42702 | Max Hourly Accum Value | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 800.00 |  | 1.00 |  | Corrections to original description (no format change) |
|  | BEGINNING OF THE ADJUSTMENT ALGORITHM ADAPTATION DATA SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42703-42706 | Time Bias Estimation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 50.00 to 59.00 Default: 50.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42707-42710 | Thresh. No. Gage-Radar Pairs | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 6.00 to 30.00 Default: 10.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42711-42714 | Reset Bias Value | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 0.50 to 2.00 Default: 1.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 42715-42718 | Longest Allowable Lag | CHAR*8 |  | HRS |  | “XXXXX.XX” (space padded) Range: 100.00 to 1000.00 Default: 168.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 42719-42722 | Bias Applied Flag | CHAR*8 |  | N/A |  | “XXXXXXXX” (space padded) Range: T or F Default: F |  | N/A |  |  |
|  | BEGINNING OF THE SUPPLEMENTAL SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42723-42726 | Supplemental data header |  | CHAR*8 |  | N/A |  | “SUPL(15)” |  | N/A | Correction to original description (no format change) |
| 42727-42730 | Average Scan Date |  | CHAR*8 |  | Julian Dalys |  | 0-99999 |  | 1 |  |
| 42731-42734 | Average Scan Time |  | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |
| 42735-42738 | Flag Zero Hybrid |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 42739-42742 | Rain Detection Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 42743-42746 | Reset STP Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 42747-42750 | Precip. Begin Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 42751-42754 | Last Date Rain |  | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |
| 42755-42758 | Last Time Rain |  | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |
| 42759-42762 | Rejected Blockage Counter |  | CHAR*8 |  | N/A |  | 0-82800 |  | 1 |  |
| 42763-42766 | Rejected AP/Clutter Counter |  | CHAR*8 |  | N/A |  | 0-82800 |  | 1 |  |
| 42767-42770 | Total Bins Smooth |  | CHAR*8 |  | N/A |  | 0 |  | N/A | Not implemented |
| 42771-42774 | Pecentage Filled Hybrid Scan |  | CHAR*8 |  | N/A |  | 0.00 to 100.00 |  | 0.01 |  |
| 42775-42778 | Highest Elevation Angle |  | CHAR*8 |  | N/A |  | 0.50 to 19.50 |  | 0.01 |  |
| 42779-42782 | Rain Summation Area |  | CHAR*8 |  | N/A |  | 0.0-166190.3 |  | 0.1 | CCR #NA04-27811 |
| 42783-42786 | Volume Spot Blank |  | CHAR*8 |  | N/A |  | 0 to 1 |  | 1 |  |
|  | BEGINNING OF THE BIAS TABLE SUB-LAYER OF DHR ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 42787-42790 | Bias Table data header | CHAR*8 |  | N/A |  | “BIAS(11)” |  | N/A |  | Correction to original description (no format change) |
| 42791-42794 | Time Last Update of Local Bias Value | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 42795-42798 | Date Last Update of Local Bias Value | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 42799-42802 | Time of Last Update of Local Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 42803-42806 | Date of Last Update of Local Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 42807-42810 | Observation Time of Latest Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 42811-42814 | Observation Date of Latest Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 42815-42818 | Generation Time of Latest Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 42819-42822 | Generation Date of Latest Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 42823-42826 | Mean-Field Bias Estimate | CHAR*8 |  | N/A |  | .01-100. |  | .01 |  |  |
| 42827-42830 | Effective G-R Pair | CHAR*8 |  | N/A |  | 0.00-999.99 |  | .01 |  |  |
| 42831-42834 | Memory Span | CHAR*8 |  | N/A |  | 0.001-10.**7 |  | .001 |  |  |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT USED BY DHR]

[TABULAR ALPHANUMERIC BLOCK IS NOT USED BY DHR]

The following is an example of the alphanumeric layer (layer 2) of the DHR product, as rendered by CODE cvt.

*** ORPG DATABASE PRODUCT LOAD UTILITY ***

-> Number of Products Available=5417

-> Message ID=5394

-> Product Info: LBuffer# 057 MSGLEN 026141 VOLNUM 158 ELEV 07

-> Set Processing ONLY for Layer Number 2

packet code 1 found

Packet 1: Write Text (No Value) Summary Information

Length of Data Block (in bytes) = 548

I Starting Point:                 0

J Starting Point:                 0

Message to follow:

PSM ( 6)   12335   48300   12355   48300       1       1 ADAP(32)   0.90   50.00

50.00   50.00   99.70  -32.00   20.00   80.00   60.00  300.00    1.40    0.00

70.00    3.00  230.00    0.00    1.00    0.00    0.00  103.80   60.00    0.00

54.00  400.00    0.00  400.00  800.00   50.00   10.00    1.00  168.00       T

SUPL(15)   12335   48448       0       1       0       0   12355   48448       0

12576       0   99.86    9.90 14772.4       0BIAS(11)   58368   10461       0

0   72000   11695   75453   11695  1.2550   13.49    168.

program complete