Precipitation Processing System (PPS) Product Format Description

Hourly Digital Precip Array (DPA)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 256-level digital |  |
| Time generated | Once every volume scan |  |
| Hourly Accumulation Data | 1-hr digital data 131 x 131 array |  |
| Rate Scan Data | ¼ LFM, 13 x 13 arrays | Number of rate scan layers may vary from 1 to 16 with number of volumes scans in the hour |
| Alphanumeric data | Adaptable parameters Bias Table Supplemental data |  |
| Compression | None |  |
| AWIPS ID | WSRDPAxxx | xxx is site ID for originating WFO |
| Description/Purpose The hourly accumulation data are compacted in a 1/40-LFM grid with a maximum of 256 levels, and the scan rate data are compacted in a ¼ LFM grid with a maximum of 8 levels. The 8 levels are defined as follows: 8-Level Code; Display; Range (in/hr). 0; 0.0; 0.0<in/hr<0.1. 1; 0.1; 0.1<in/hr<0.3. 2; 0.3; 0.3<in/hr<0.5. 3; 0.5; 0.5<in/hr<1.0. 4; 1.0; 1.0<in/hr<2.0. 5; 2.0; 2.0<in/hr<4.0. 6; 4.0; 4.0<in/hr. 7; ND. |

The following table provides a detailed specification of the DPA product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a half-word (INT*2) is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 81 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | Bytes | 3400 to 42266 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent HALFWORD Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

“Digital Precipitation Array Product Format”, published by the WSR-88D Operational Support Facility, March 18 1996

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 81 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 | N/A | 0 | N/A |  |
| 28 | Not used | INT*2 | N/A | 0 | N/A |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Minimum DPA data level | INT*2 | dBA | -6.0 | 0.1 |  |
| 32 | Data level increment | INT*2 | dBA | .125 | 0.001 |  |
| 33 | Number of data levels | INT*2 | N/A | 256 | 1 |  |
| 34 | Not used | INT*2 | N/A | 0 | N/A |  |
| 35 | Not used | INT*2 | N/A | 0 | N/A |  |
| 36 | Not used | INT*2 | N/A | 0 | N/A |  |
| 37 | Not used | INT*2 | N/A | 0 | N/A |  |
| 38 | Not used | INT*2 | N/A | 0 | N/A |  |
| 39 | Not used | INT*2 | N/A | 0 | N/A |  |
| 40 | Not used | INT*2 | N/A | 0 | N/A |  |
| 41 | Not used | INT*2 | N/A | 0 | N/A |  |
| 42 | Not used | INT*2 | N/A | 0 | N/A |  |
| 43 | Not used | INT*2 | N/A | 0 | N/A |  |
| 44 | Not used | INT*2 | N/A | 0 | N/A |  |
| 45 | Not used | INT*2 | N/A | 0 | N/A |  |
| 46 | Not used | INT*2 | N/A | 0 | N/A |  |
| 47 | Maximum Rainfall accum | INT*2 | dBA | 0; -6.0 to 25.625 | 0.125 |  |
| 48 | Mean field bias | INT*2 | N/A | .01 to 99.99 | 0.01 |  |
| 49 | Effective Number G-R pairs (sample size) | INT*2 | N/A | .00 to 9999.99 | 0.01 |  |
| 50 | Hourly accum End Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 51 | Hourly accum End Time | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 0 to 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Product Symbology block | INT*4 | Half-words | 60 | 1 |  |
| 57-58 | Offset to Graphic Alphanumeric block | INT*4 | Half-words | 0 | 1 |  |
| 59-60 | Offset to Tabular Alphanumeric block | INT*4 | Half-words | 0 | 1 |  |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” (Sheet 8)

Fig 3-8b “Text and Special Symbol Packets”

Fig 3-11a “Digital Precip Data Array Packet”

Fig 3-11b “Precip Rate Data Array Packet”

2620003 (ICD for Product Specification):

Appendix C, Format VII “DPA”

“Decoding of DPA Products”, published by OHD/HSEB, updated as of AWIPS Release OB3 (1/8/04)

“Digital Precipitation Array Product Format”, published by the WSR-88D Operational Support Facility, March 18 1996

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Block ID | INT*2 | N/A | 1 | N/A |  |
| 63-64 | Length of Block | INT*4 | Bytes | 3280 to 42146 | 1 |  |
| 65 | Number of Layers | INT*2 | N/A | 3 to 18 | 1 |  |
|  | BEGINNING OF THE DPA HOURLY DATA LAYER |  |  |  |  |  |
| 66 | Digital Data Layer Divider | INT*2 | N/A | -1 | N/A |  |
| 67-68 | Length of Data Layer in bytes not including layer divider and layer length | INT*4 | Bytes | 534 to 34594 | 1 |  |
| 69 | Packet Code | INT*2 | N/A | 17 | N/A |  |
| 70 | Spare | INT*2 | N/A | 0 | N/A |  |
| 71 | Spare | INT*2 | N/A | 0 | N/A |  |
| 72 | Number of 1/40 LFM boxes in row | INT*2 | N/A | 131 | 1 |  |
| 73 | Number of rows | INT*2 | N/A | 131 | 1 |  |
|  | Following is a fixed 131 x 131 array of INT*2 elements, run-length encoded. Run/data level byte values are set to 0 when any runs are not used. 0 is for no accumulation; 255 is for data outside the coverage area. Data values may be converted to rainfall amount by: DBA = -6.125 + (data level code) *0.125 RAINFALL (mm) = 10 ** (0.1*DBA) |  |  |  |  |  |
| 74 | Row 1: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 |  |
| 75 | Row 1: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 1: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| (74 + N) | Row 1: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 1: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 |  |
| (74 + N+1) | Row 2: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 |  |
| (74 + N+2) | Row 2: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 2: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 2: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 2: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 131: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 |  |
|  | Row 131: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 131: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 131: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 |  |
|  | Row 131: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 |  |
|  | BEGINNING OF THE DPA RATE SCAN LAYERS (The number of layers will be variable as per the number of rate scans in the hour) |  |  |  |  |  |
| O-Rate | Rate Scan layer #1 divider | INT*2 | N/A | -1 | N/A |  |
| O-Rate +1 to +2 | Rate Scan Layer #1 length in bytes not including layer divider and layer length | INT*4 | Bytes | 62 to 218 | 1 |  |
| O-Rate +3 | Rate Scan packet code | INT*2 | N/A | 18 | N/A |  |
| O-Rate +4 | Spare | INT*2 | N/A | 0 | N/A |  |
| O-Rate +5 | Spare | INT*2 | N/A | 0 | N/A |  |
| O-Rate +6 | Number of 1/4 LFM boxes in row | INT*2 | N/A | 13 | 1 |  |
| O-Rate +7 | Number of rows | INT*2 | N/A | 13 | 1 |  |
| O-Rate +8 | Row N: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 – 14 | 1 | } repeat |
| O-Rate +9 | Run Length Code (1) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (1) | 4-bit | N/A | 0 – 15 | 1 | } for |
|  | Run Length Code (2) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (2) | 4-bit | N/A | 0 – 15 | 1 | } each |
| O-Rate+10 | Run Length Code (3) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (3) | 4-bit | N/A | 0 – 15 | 1 | } of |
|  | … | …. | … | … | … | } |
|  | … | … | … | … | … | } |
|  | … | … | … | … | … | } |
|  | Run Length Code (N) (N = 1 to 13) | 4-bit | N/A | 1 – 13 | 1 | } N = 1 to 13 |
|  | Data Level Code (N) (N = 1 to 13) | 4-bit | N/A | 0 – 15 | 1 | } |
|  | NULL (only nec. if #) | 4-bit | N/A | 0000 | N/A | } rows |
|  | NULL (runs (N)=odd) | 4-bit | N/A | 0000 | N/A | } |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
| O-Rate (L) | Rate Scan layer #L divider (L = 1 to 16) | INT*2 | N/A | -1 | N/A |  |
| O-Rate (L) +1 to +2 | Rate Scan Layer #L length in bytes not including layer divider and layer length | INT*4 | Bytes | 62 to 218 | 1 |  |
| O-Rate (L) +3 | Rate Scan packet code | INT*2 | N/A | 18 | N/A |  |
| O-Rate (L) +4 | Spare | INT*2 | N/A | 0 | N/A |  |
| O-Rate (L) +5 | Spare | INT*2 | N/A | 0 | N/A |  |
| O-Rate (L) +6 | Number of 1/4 LFM boxes in row | INT*2 | N/A | 13 | 1 |  |
| O-Rate (L) +7 | Number of rows | INT*2 | N/A | 13 | 1 |  |
| O-Rate (L) +8 | Row N: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 – 14 | 1 | } repeat |
| O-Rate (L) +9 | Run Length Code (1) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (1) | 4-bit | N/A | 0 – 15 | 1 | } for |
|  | Run Length Code (2) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (2) | 4-bit | N/A | 0 – 15 | 1 | } each |
| O-Rate (L) +10 | Run Length Code (3) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (3) | 4-bit | N/A | 0 – 15 | 1 | } of |
|  | … | …. | … | … | … | } |
|  | … | … | … | … | … | } |
|  | … | … | … | … | … | } |
|  | Run Length Code (N) (N = 1 to 13) | 4-bit | N/A | 1 – 13 | 1 | } N = 1 to 13 |
|  | Data Level Code (N) (N = 1 to 13) | 4-bit | N/A | 0 – 15 | 1 | } |
|  | NULL (only nec. if #) | 4-bit | N/A | 0000 | N/A | } rows |
|  | NULL (runs (N)=odd) | 4-bit | N/A | 0000 | N/A | } |
|  | BEGINNING OF THE DPA ALPHANUMERIC LAYER (Note: Offset [0] represents the last NULL byte of Rate Scan layers) |  |  |  |  |  |
| Offset[1] | Text layer divider | INT*2 | N/A | -1 | N/A |  |
| Offset[2] – Offset[3] | Layer length not including layer divider and layer length | INT*4 | Bytes | 2656 to 3856 | 1 |  |
| Offset[4] | Text layer packet code | INT*2 | N/A | 1 | N/A |  |
| Offset[5] | Length of text layer in bytes | INT*2 | Bytes | 2652 to 3852 | 1 |  |
| Offset[6] | I Starting Point | INT*2 | Km/4 or Pixels | 0 | 1 |  |
| Offset[7] | J Starting Point | INT*2 | Km/4 or Pixels | 0 | 1 |  |
|  | BEGINNING OF THE EPRE ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[8] - Offset[11] | Adaptation data header | CHAR*8 | N/A | “ADAP(32)” | N/A | CCR#NA04-32201 |
| Offset[12 - Offset[15] | Width of radar beam | CHAR*8 | Deg | “XXXXX.XX” (space padded) Range: 0.80 to 1.00 Default: 0.90 | 0.01 |  |
| Offset[16] - Offset[19] | Blockage Threshold | CHAR*8 | % | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.01 |  |
| Offset[20] - Offset[23] | Clutter Threshold | CHAR*8 | % | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.01 |  |
| Offset[24] - Offset[27] | Weight Threshold | CHAR*8 | % | “XXXXX.XX” Range: 0.00 to 100.00 Default: 50.00 | 0.01 |  |
| Offset[28] - Offset[31] | Full Hybrid Scan Threshold | CHAR*8 | % | “XXXXX.XX” (space padded) Range: 90.00 to 100.00 Default: 99.70 | 0.01 |  |
| Offset[32] - Offset[35] | Low Reflectivity Threshold | CHAR*8 | dBZ | “XXXXX.XX” (space padded) Range: -40.00 to -20.00 Default: -32.00 | 0.01 |  |
| Offset[36] - Offset[39] | Rain Detection Reflectivity Threshold | CHAR*8 | dBZ | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 20.00 | 0.01 |  |
| Offset[40 - Offset[43] | Rain Detection Area Threshold | CHAR*8 | Km2 | “XXXXX.XX” (space padded) Range: 0.00 to 82800.00 Default: 80.00 | 0.01 |  |
| Offset[44] - Offset[47] | Rain Detection Time Threshold | CHAR*8 | Min | “XXXXX.XX” (space padded) Range: 0.00 to 1440.00 Default: 60.00 | 0.01 |  |
| Offset[48] - Offset[51] | Z-R Mult. Coef. | CHAR*8 | N/A | “XXXXX.XX” (space padded) Range: 50.00 to 500.00 Default: 300.00 | 0.01 |  |
| Offset[52] - Offset[55] | Z-R Power Coef. | CHAR*8 | N/A | “XXXXX.XX” Range: 1.00 to 2.00 Default: 1.40 | 0.01 |  |
| Offset[56] - Offset[59] | Min. Refl. to Convert to Rate | CHAR*8 | dBZ | “XXXXX.XX” Range: -32.00 to +20.00 Default: 0.00 | 0.01 |  |
| Offset[60 - Offset[63] | Max. Refl. to Convert to Rate | CHAR*8 | dBZ | “XXXXX.XX” (space padded) Range: 50.00 to 90.00 Default: 70.00 | 0.01 |  |
| Offset[64] - Offset[67] | Number of Exclusion Zones | CHAR*8 | N/A | “XXXXX.XX” (space padded) Range: 0.00 to 20.00 Default: 0.00 | 1.00 | Correction to original description (no format change) |
|  | BEGINNING OF THE RATE ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[68] - Offset[71] | Max Storm Speed | CHAR*8 | M/S | “XXXXX.XX” (space padded) Range: 10.00 to 40.00 Default: 25.00 | 0.01 | CCR#NA04-32201 |
| Offset[72] - Offset[75] | Thresh. Max Time Difference | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 15.00 | 0.01 | CCR#NA04-32201 |
| Offset[76] - Offset[79] | Min. Area Time Continuity | CHAR*8 | KM**2 | “XXXXX.XX” (space padded) Range: 50.00 to 1000.00 Default: 200.00 | 0.01 | CCR#NA04-32201 |
| Offset[80] - Offset[83] | Time Continuity Parameter #1 | CHAR*8 | 1/HR | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 24.00 | 0.01 | CCR#NA04-32201 |
| Offset[84] - Offset[87] | Time Continuity Parameter #2 | CHAR*8 | 1/HR | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 13.20 | 0.01 | CCR#NA04-32201 |
| Offset[88] - Offset[91] | Max. Rate Echo Area Change | CHAR*8 | KM**2/HR | “XXXXX.XX” (space padded) Range: 20.00 to 700.00 Default: 200.00 | 0.01 | CCR#NA04-32201 |
| Note: as a result of implementing this CCR the halfword addresses of all remaining fields have changed |  |  |  |  |  |  |
| Offset[68] - Offset[71] | Range Cut-Off | CHAR*8 | KM | “XXXXX.XX” (space padded) Range: 0.00 to 230.00 Default: 230.00 | 1.00 | Correction to original description (no format change) |
| Offset[72] - Offset[75] | Range Effect Coeff. #1 | CHAR*8 | dBR | “XXXXX.XX” (space padded) Range: 0.00 to 3.00 Default: 0.00 | 0.10 | Correction to original description (no format change) |
| Offset[76] - Offset[79] | Range Coeff. Coeff. #2 | CHAR*8 | dBR | “XXXXX.XX” (space padded) Range: 1.00 to 10.00 Default: 1.00 | 0.10 | Correction to original description (no format change) |
| Offset[80] - Offset[83] | Range Coeff. Coeff. #3 | CHAR*8 | dBR | “XXXXX.XX” (space padded) Range: 0.00 to 1.00 Default: 0.00 | 0.10 | Correction to original description (no format change) |
| Offset[84] - Offset[87] | Min Precip. Rate for inclusion | CHAR*8 | MM/HR | “XXXXX.XX” (space padded) Range: 0.00 to 10.00 Default: 0.00 | 0.10 | Correction to original description (no format change) |
| Offset[88] - Offset[91] | Max Precip. Rate allowed | CHAR*8 | MM/HR | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 103.80 | 0.10 | Correction to original description (no format change) |
|  | BEGINNING OF THE ACCUM ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[92] - Offset[95] | Thresh. Elapsed Time to Restart | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 45.00 to 60.00 Default: 60.00 | 1.00 | Correction to original description (no format change) |
| Offset[96] - Offset[99] | Max. Time for Interpolation | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 15.00 to 60.00 Default: 30.00 | 1.00 | Correction to original description (no format change) |
| Offset[100] - Offset[103] | Min. Time in Hourly Period | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 0.00 to 60.00 Default: 54.00 | 1.00 | Corrections to original description (no format change) |
| Offset[104] - Offset[107] | Threshold Hourly Outlier | CHAR*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 800.00 Default: 400.00 | 1.00 | Correction to original description (no format change) |
| Offset[108] - Offset[111] | End Time Gage Accumulation | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 0.00 to 59.00 Default: 0.00 | 1.00 | Correction to original description (no format change) |
| Offset[112] - Offset[115] | Max Period Accum Value | CHAR*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 400.00 Default: 400.00 | 1.00 | Correction to original description (no format change) |
| Offset[116] - Offset[119] | Max Hourly Accum Value | CHAR*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 800.00 | 1.00 | Correction to original description (no format change) |
|  | BEGINNING OF THE ADJUSTMENT ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[120] - Offset[123] | Time Bias Estimation | CHAR*8 | MIN | “XXXXX.XX” (space padded) Range: 50.00 to 59.00 Default: 50.00 | 1.00 | Correction to original description (no format change) |
| Offset[124] - Offset[127] | Thresh. No. Gage-Radar Pairs | CHAR*8 | N/A | “XXXXX.XX” (space padded) Range: 6.00 to 30.00 Default: 10.00 | 1.00 | Correction to original description (no format change) |
| Offset[128] - Offset[131] | Reset Bias Value | CHAR*8 | N/A | “XXXXX.XX” (space padded) Range: 0.50 to 2.00 Default: 1.00 | 0.10 | Correction to original description (no format change) |
| Offset[132] - Offset[135] | Longest Allowable Lag | CHAR*8 | HRS | “XXXXX.XX” (space padded) Range: 100.00 to 1000.00 Default: 168.00 | 1.00 | Correction to original description (no format change) |
| Offset[136] - Offset[139] | Bias Applied Flag | CHAR*8 | N/A | “XXXXXXXX” (space padded) Range: T or F Default: F | N/A |  |
| Offset[140] – Offset[163] | Blank | CHAR*24 | N/A | N/A | N/A | Space left after removal of six fields for TCT Removal (CCR#NA04-32201) |
|  | BEGINNING OF THE BIAS TABLE SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[164] - Offset[167] | Bias Table data header | CHAR*8 | N/A | “BIAS(13)” | N/A | Correction to original description (no format change) |
| Offset[168] - Offset[207] | Bias Table Title Line 1 | CHAR*80 | N/A | “GAGE-RADAR MEAN FIELD BIAS TABLE “ | N/A |  |
| Offset[208] - Offset[249] | Bias Table Title Line 2 | CHAR*80 | N/A | “LAST BIAS UPDATE TIME: MM/DD/YY HH:MM BIAS APPLIED? XXX” Range: “YES” or “ NO “ Default: NO | N/A | MM/DD/YY and HH:MM filled in appropriately |
| Offset[248] - Offset[287] | Bias Table Title Line 3 | CHAR*80 | N/A | “MSPAN (HRS) NO. G_R PAIRS AVG. GAGE(MM) AVG. RADAR(MM) MEAN FLD BIAS“ | N/A |  |
| Offset[288] - Offset[307] | Row 1: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 1: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 1: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 1: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 1: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[308] - Offset[327] | Row 2: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 2: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 2: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 2: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 2: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[328] - Offset[347] | Row 3: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 3: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 3: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 3: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 3: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[348] - Offset[367] | Row 4: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 4: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 4: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 4: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 4: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[368] - Offset[387] | Row 5: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 5: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 5: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 5: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 5: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[388] - Offset[407] | Row 6: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 6: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 6: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 6: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 6: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[408] - Offset[427] | Row 7: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 7: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 7: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 7: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 7: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[428] - Offset[447] | Row 8: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 8: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 8: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 8: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 8: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[448] - Offset[467] | Row 9: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 9: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 9: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 9: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 9: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| Offset[468] - Offset[487] | Row 10: MEMORY SPAN (HOURS) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 10: NO. G-R PAIRS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 10: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 10: AVG. RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 10: MEAN-FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
|  | BEGINNING OF THE SUPPLEMENTAL DATA SUB-LAYER OF THE DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[488] - Offset[491] | Supplemental data header | CHAR*8 | N/A | “SUPL(nn)” | N/A | nn is variable that depends on the houly rate scan |
| Offset[492] - Offset[531] | Rate Scan 1 | CHAR*80 | N/A | “RATE SCAN 1 DATE: dddd TIME: ttttt” | N/A | Date format is “adjusted” Julian date (number of days since Jan 1 1970) Time format is number of seconds |
|  | … | … | … | … | … |  |
|  | … | … | … | … | … |  |
| Offset[532]- Offset[571] | Rate Scan nn (Note: nn can range from 1 to 16) | CHAR*80 | N/A | “RATE SCAN nn DATE: dddd TIME: ttttt” | N/A | Date format is “adjusted” Julian date (number of days since Jan 1 1970) Time format is number of seconds |
| Offset[572]- Offset[611] | Hourly Accumulation End Date | CHAR*80 | N/A | “HOURLY ACCUMULATION END DATE…: dddd” | 1 | Date format is “adjusted” Julian date (number of days since Jan 1 1970) |
| Offset[612]- Offset[651] | Hourly Accumulation End Time | CHAR*80 | N/A | “HOURLY ACCUMULATION END TIME…: ttttt” | 1 | Time format is number of seconds |
| Offset[652]- Offset[691] | Total No. of Blockage Bins Rejected | CHAR*80 | N/A | “TOTAL NO. OF BLOCKAGE BINS REJECTED: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[692]- Offset[731] | Total No. of Clutter Bins Rejected | CHAR*80 | N/A | “TOTAL NO. OF CLUTTER BINS REJECTED.: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[732d]- Offset[771] | Number of Bins Smoothed | CHAR*80 | N/A | “NUMBER OF BINS SMOOTHED……..: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[772]- Offset[811] | Percent of Hybrid Scan Bins Filled | CHAR*80 | % | “PERCENT OF HYBRID SCANS FILLED.: xx.xx” Range: 90.00 to 100.00 Default: 99.70 | 0.01 | Number is padded with spaces Correction to original description (no format change) |
| Offset[812]- Offset[851] | Highest Elevation Angle Used in Hybrid Scan | CHAR*80 | Deg | “HIGHEST ELEVATION ANGLE USED IN HYBSCAN: xxx.xx” Range: 0.00 to 19.50 Default: 3.50 | 0.01 | Number is padded with spaces |
| Offset[852]- Offset[891] | Total Hybrid Scan Rain Area | CHAR*80 | Km2 | “TOTAL HYBRID SCAN RAIN AREA……..: xxxxxx.x” Range: 0.0 to 166190.3 Default: 80.0 | 0.1 | Number is padded with spaces CCR #NA04-27811 |
| Offset[892]- Offset[931] | Number of Bad Scans in Hour | CHAR*80 | N/A | “NUMBER OF BAD SCANS IN HOUR…….: xx” Range: 0 to 16 Default: 0 | 1 | Number is padded with spaces |
| Offset[932]- Offset[971] | Bias Estimate | CHAR*80 | N/A | “BIAS ESTIMATE…………….: x.xx” Range: 0.01 to 100.00 Default : 1.00 | 0.01 | Number is padded with spaces |
| Offset[972]- Offset[1011] | Effective No. Gage-Radar Pairs | CHAR*80 | N/A | “EFFECTIVE # G/R PAIRS…….: x.xx” Range: 6.00 to 30.00 Default: 10.00 | 0.01 | Number is padded with spaces |
| Offset[1012]- Offset[1051] | Memory Span (Hours) | CHAR*80 | N/A | “MEMORY SPAN (HOURS)………..: xx.xxx” Range: 0.001 to 10.**7 Default: 10.00 | 0.01 | Number is padded with spaces |
| Offset[1052]- Offset[1091] | Current Volume Coverage Pattern | CHAR*80 | N/A | “CURRENT VOLUME COVERAGE PATTERN….: xxx” Range: 11,12, 21, 121, etc. Default: 11 | 1 | Number is padded with spaces |
| Offset[1092]- Offset[1131] | Current Operational (Weather) Mode | CHAR*80 | N/A | “CURRENT OPERATIONAL (WEATHER) MODE.: x” Range: A or B Default: A | N/A |  |
| Offset[1132]- Offset[1171] | No Missing Periods in Current Hour | CHAR*80 | N/A | “NO MISSING PERIODS IN CURRENT HOUR” | N/A | Correction to original description (no format change) |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT USED BY DPA]

[TABULAR ALPHANUMERIC BLOCK IS NOT USED BY DPA]

The following is an example of the alphanumeric layer of the DPA product, as rendered by CODE cvt.

ADAP(32)    0.90   50.00   50.00   50.00   99.70  -32.00   20.00   80.00   60.00

300.00    1.40    0.00   70.00    3.00  230.00    0.00    1.00    0.00    0.00

103.80   60.00   30.00   54.00  400.00    0.00  400.00  800.00   50.00   10.00

1.00  168.00       T                                                BIAS(13)

GAGE-RADAR MEAN FIELD BIAS TABLE

LAST BIAS UPDATE TIME:  10/09/03 00:03                      BIAS APPLIED ?  YES

MSPAN (HRS)    NO. G_R PAIRS   AVG. GAGE(MM)   AVG. RADAR(MM)   MEAN FLD BIAS

0.001           0.000           1.016           1.090           0.932

1.000           0.000           1.863           1.538           1.211

2.000           0.000           2.648           1.933           1.370

3.001           0.000           2.825           2.032           1.391

4.998           0.000           2.908           2.089           1.392

10.004           0.000           2.935           2.118           1.386

168.006          13.494           2.627           2.094           1.255

719.819         126.212           2.417           2.104           1.149

2160.295         212.154           2.381           2.109           1.129

9999044.000         277.982           2.365           2.112           1.120

SUPL(31)RATE SCAN  1 DATE:  12335 TIME: 3520

RATE SCAN  2 DATE:  12335 TIME: 3776

RATE SCAN  3 DATE:  12335 TIME: 4032

RATE SCAN  4 DATE:  12335 TIME: 4288

RATE SCAN  5 DATE:  12335 TIME: 4544

RATE SCAN  6 DATE:  12335 TIME: 4800

RATE SCAN  7 DATE:  12335 TIME: 5120

RATE SCAN  8 DATE:  12335 TIME: 5376

RATE SCAN  9 DATE:  12335 TIME: 5632

RATE SCAN 10 DATE:  12335 TIME: 5888

RATE SCAN 11 DATE:  12335 TIME: 6144

RATE SCAN 12 DATE:  12335 TIME: 6400

RATE SCAN 13 DATE:  12335 TIME: 6656

RATE SCAN 14 DATE:  12335 TIME: 6912

RATE SCAN 15 DATE:  12335 TIME: 7168

RATE SCAN 16 DATE:  12335 TIME: 7424

HOURLY ACCUMULATION END DATE.......:   12335

HOURLY ACCUMULATION END TIME.......:    7200

TOTAL NO. OF BLOCKAGE BINS REJECTED:       0

TOTAL NO. OF CLUTTER BINS REJECTED.:    8578

NUMBER OF BINS SMOOTHED............:       0

PERCENT OF HYBRID SCAN BINS FILLED.:   99.88

HIGHEST ELEV. ANGLE USED IN HYBSCAN:    2.40

TOTAL HYBRID SCAN RAIN AREA........: 42459.1

NUMBER OF BAD SCANS IN HOUR........:       0

BIAS ESTIMATE......................:    1.25

EFFECTIVE # G/R PAIR...............:   13.49

MEMORY SPAN (HOURS)................:  168.01

CURRENT VOLUME COVERAGE PATTERN....:      12

CURRENT OPERATIONAL (WEATHER) MODE.:       2

NO MISSING PERIODS IN CURRENT HOUR