Precipitation Processing System (PPS) Product Format Description

Hourly Digital Precip Array (DPA)

DPA is a digital product, not directly displayable. It is generated every volume scan. The Product Symbology block contains several layers:

- Hourly accumulation data – 1 Hr. digital data – 131 x 131 array (approx. 4 x 4 km2 grid)

- Rate Scan data (1/4 LFM) 13 x 13 arrays (approx. 40 x 40 km2 grids)  (note: number of rate scan layers may vary from 1 to 16 with number of volume scans in the hour)

- Alphanumeric data (ASCII)0 to 16)??????

  - Adaptable parameters

  - Bias Table

  - Supplemental data

The hourly accumulation data are compacted in a 1/40-LFM grid with a maximum of 256 levels, and the scan rate data are compacted in a 1/4 LFM grid with a maximum of 8 levels.  The 8 levels are defined as follows:

| 8-Level Code | Display | Range (in/hr) |
| --- | --- | --- |
| 0 | 0.0 | 0.0<in/hr<0.1 |
| 1 | 0.1 | 0.1<in/hr<0.3 |
| 2 | 0.3 | 0.3<in/hr<0.5 |
| 3 | 0.5 | 0.5<in/hr<1.0 |
| 4 | 1.0 | 1.0<in/hr<2.0 |
| 5 | 2.0 | 2.0<in/hr<4.0 |
| 6 | 4.0 | 4.0<in/hr |
| 7 | ND |  |

This product is not compressed.

The following table provides a detailed specification of the DPA product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a half-word (INT*2) is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 81 | N/A | From Fig 3-3 |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 | From Fig 3-3 |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 | From Fig 3-3 |
| 05-06 | Length of Message | INT*4 | Bytes | 3400 to 42266 | 1 | From Fig 3-3 |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 | From Fig 3-3 |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 | From Fig 3-3 |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 | From Fig 3-3 |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent HALFWORD Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

“Digital Precipitation Array Product Format”, published by the WSR-88D Operational Support Facility, March 18 1996

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A | From Fig 3-6 Sheet 6 |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 | From Fig 3-6 Sheet 6 |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 | From Fig 3-6 Sheet 6 |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 | From Fig 3-6 Sheet 6 |
| 16 | Product Code | INT*2 | N/A | 81 |  | From Table III |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A | From Fig 3-6 Sheet 6 |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A | From Fig 3-6 Sheet 6 |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 | From Fig 3-6 Sheet 6 |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 | From Fig 3-6 Sheet 6 |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 | From Fig 3-6 Sheet 6 |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 | From Fig 3-6 Sheet 6 |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 | From Fig 3-6 Sheet 6 |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 | From Fig 3-6 Sheet 6 |
| 27 | Not used |  |  | 0 |  | From OSF doc |
| 28 | Not used |  |  | 0 |  | From OSF doc |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 | From Fig 3-6 Sheet 6 |
| 30 | Not used |  |  | 0 |  | From OSF doc |
| 31 | Minimum DPA data level | INT*2 | dBA | -6.0 | 0.1 | From OSF doc |
| 32 | Data level increment | INT*2 | dBA | .125 | 0.001 | From OSF doc |
| 33 | Number of data levels | INT*2 | N/A | 256 | 1 | From OSF doc |
| 34 | Not used |  |  | 0 |  | From OSF doc |
| 35 | Not used |  |  | 0 |  | From OSF doc |
| 36 | Not used |  |  | 0 |  | From OSF doc |
| 37 | Not used |  |  | 0 |  | From OSF doc |
| 38 | Not used |  |  | 0 |  | From OSF doc |
| 39 | Not used |  |  | 0 |  | From OSF doc |
| 40 | Not used |  |  | 0 |  | From OSF doc |
| 41 | Not used |  |  | 0 |  | From OSF doc |
| 42 | Not used |  |  | 0 |  | From OSF doc |
| 43 | Not used |  |  | 0 |  | From OSF doc |
| 44 | Not used |  |  | 0 |  | From OSF doc |
| 45 | Not used |  |  | 0 |  | From OSF doc |
| 46 | Not used |  |  | 0 |  | From OSF doc |
| 47 | Maximum Rainfall accum | INT*2 | dBA | 0; -6.0 to 25.625 | 0.125 | DOORS version of Class I Users ICD |
| 48 | Mean field bias | INT*2 | N/A | .01 to 99.99 | 0.01 | DOORS version of Class I Users ICD |
| 49 | Effective Number G-R pairs (sample size) | INT*2 | N/A | .00 to 9999.99 | 0.01 | DOORS version of Class I Users ICD |
| 50 | Hourly accum End Date | INT*2 | Julian Date | 1 to 32767 | 1 | DOORS version of Class I Users ICD |
| 51 | Hourly accum End Time | INT*2 | Minutes | 0 to 1439 | 1 | DOORS version of Class I Users ICD |
| 52 | Not used | INT*2 | N/A | 0 | N/A | From OSF doc |
| 53 | Not used | INT*2 | N/A | 0 | N/A | From OSF doc |
| 54 | Version | INT*1 | N/A | 0 to 2 | 1 | From Sheet 7, Note 2 - DOORS version of Class I Users ICD |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 | From Fig 3-6 Sheet 6 |
| 55-56 | Offset to Product Symbology block | INT*4 | Half-words | 60 | 1 | From OSF doc |
| 57-58 | Offset to Graphic Alphanumeric block | INT*4 | Half-words | 0 | 1 | From OSF doc |
| 59-60 | Offset to Tabular Alphanumeric block | INT*4 | Half-words | 0 | 1 | From OSF doc |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” (Sheet 8)

Fig 3-8b “Text and Special Symbol Packets”

Fig 3-11a “Digital Precip Data Array Packet”

Fig 3-11b “Precip Rate Data Array Packet”

2620003 (ICD for Product Specification):

Appendix C, Format VII “DPA”

“Decoding of DPA Products”, published by OHD/HSEB, updated as of AWIPS Release OB3 (1/8/04)

“Digital Precipitation Array Product Format”, published by the WSR-88D Operational Support Facility, March 18 1996

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A | From Fig 3-6 Sheet 8 |
| 62 | Block ID | INT*2 | N/A | 1 | N/A | From Fig 3-6 Sheet 8 |
| 63-64 | Length of Block | INT*4 | Bytes | 3280 to 42146 | 1 | From Fig 3-6 Sheet 8 |
| 65 | Number of Layers | INT*2 | N/A | 3 to 18 | 1 | DOORS version of Class I Users ICD |
|  | BEGINNING OF THE DPA HOURLY DATA LAYER |  |  |  |  |  |
| 66 | Digital Data Layer Divider | INT*2 | N/A | -1 | N/A | From Fig 3-6 Sheet 8 |
| 67-68 | Length of Data Layer in bytes not including layer divider and layer length | INT*4 | Bytes | 534 to 34594 | 1 | From Fig 3-6 Sheet 8 |
| 69 | Packet Code | INT*2 | N/A | 17 | N/A | From Fig 3-11a |
| 70 | Spare | INT*2 | N/A | 0 | N/A | From Fig 3-11a |
| 71 | Spare | INT*2 | N/A | 0 | N/A | From Fig 3-11a |
| 72 | Number of 1/40 LFM boxes in row | INT*2 | N/A | 131 | 1 | From Fig 3-11a |
| 73 | Number of rows | INT*2 | N/A | 131 | 1 | From Fig 3-11a |
|  | Following is a fixed 131 x 131 array of INT*2 elements, run-length encoded. Run/data level byte values are set to 0 when any runs are not used. 0 is for no accumulation; 255 is for data outside the coverage area. Data values may be converted to rainfall amount by: DBA = -6.125 + (data level code) *0.125 RAINFALL (mm) = 10 ** (0.1*DBA) |  |  |  |  |  |
| 74 | Row 1: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 | Fig 3-11a |
| 75 | Row 1: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 1: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| (74 + N) | Row 1: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 1: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
| (74 + N+1) | Row 2: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 | Fig 3-11a |
| (74 + N+2) | Row 2: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 2: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 2: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 2: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 131: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 to 262 | 1 | Fig 3-11a |
|  | Row 131: Run Length Code (1) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 131: Data Level Code (1) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
| … | … | … | … | … | … |  |
|  | Row 131: Run Length Code N (N = 1 to 131) | INT*1 | N/A | 1 to 131 | 1 | Fig 3-11a |
|  | Row 131: Data Level Code N (N = 1 to 131) | INT*1 | N/A | 0 to 255 | 1 | Fig 3-11a |
|  | BEGINNING OF THE DPA RATE SCAN LAYERS (The number of layers will be variable as per the number of rate scans in the hour) |  |  |  |  |  |
| O-Rate | Rate Scan layer #1 divider | INT*2 | N/A | -1 | N/A | OSF doc |
| O-Rate +1 to +2 | Rate Scan Layer #1 length in bytes not including layer divider and layer length | INT*4 | Bytes | 62 to 218 | 1 | OSF doc |
| O-Rate +3 | Rate Scan packet code | INT*2 | N/A | 18 | N/A | Fig 3-11b |
| O-Rate +4 | Spare | INT*2 | N/A | 0 | N/A | Fig 3-11b |
| O-Rate +5 | Spare | INT*2 | N/A | 0 | N/A | Fig 3-11b |
| O-Rate +6 | Number of 1/4 LFM boxes in row | INT*2 | N/A | 13 | 1 | Fig 3-11b |
| O-Rate +7 | Number of rows | INT*2 | N/A | 13 | 1 | Fig 3-11b |
| O-Rate +8 | Row N: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 – 14 | 1 | } repeat |
| O-Rate +9 | Run Length Code (1) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (1) | 4-bit | N/A | 0 – 15 | 1 | } for |
|  | Run Length Code (2) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (2) | 4-bit | N/A | 0 – 15 | 1 | } each |
| O-Rate+10 | Run Length Code (3) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (3) | 4-bit | N/A | 0 – 15 | 1 | } of |
|  | … | …. | … | … | … | } |
|  | … | … | … | … | … | } |
|  | … | … | … | … | … | } |
|  | Run Length Code (N) (N = 1 to 13) | 4-bit | N/A | 1 – 13 | 1 | } N = 1 to 13 |
|  | Data Level Code (N) (N = 1 to 13) | 4-bit | N/A | 0 – 15 | 1 | } |
|  | NULL (only nec. if #) | 4-bit | N/A | 0000 | N/A | } rows |
|  | NULL (runs (N)=odd) | 4-bit | N/A | 0000 | N/A | } |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
|  | … | …. | … | … | … |  |
| O-Rate (L) | Rate Scan layer #L divider (L = 1 to 16) | INT*2 | N/A | -1 | N/A | OSF doc |
| O-Rate (L) +1 to +2 | Rate Scan Layer #L length in bytes not including layer divider and layer length | INT*4 | Bytes | 62 to 218 | 1 | OSF doc |
| O-Rate (L) +3 | Rate Scan packet code | INT*2 | N/A | 18 | N/A | Fig 3-11b |
| O-Rate (L) +4 | Spare | INT*2 | N/A | 0 | N/A | Fig 3-11b |
| O-Rate (L) +5 | Spare | INT*2 | N/A | 0 | N/A | Fig 3-11b |
| O-Rate (L) +6 | Number of 1/4 LFM boxes in row | INT*2 | N/A | 13 | 1 | Fig 3-11b |
| O-Rate (L) +7 | Number of rows | INT*2 | N/A | 13 | 1 | Fig 3-11b |
| O-Rate (L) +8 | Row N: Number of RLE bytes in row not including this field | INT*2 | Bytes | 2 – 14 | 1 | } repeat |
| O-Rate (L) +9 | Run Length Code (1) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (1) | 4-bit | N/A | 0 – 15 | 1 | } for |
|  | Run Length Code (2) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (2) | 4-bit | N/A | 0 – 15 | 1 | } each |
| O-Rate (L) +10 | Run Length Code (3) | 4-bit | N/A | 1 – 13 | 1 | } |
|  | Data Level Code (3) | 4-bit | N/A | 0 – 15 | 1 | } of |
|  | … | …. | … | … | … | } |
|  | … | … | … | … | … | } |
|  | … | … | … | … | … | } |
|  | Run Length Code (N) (N = 1 to 13) | 4-bit | N/A | 1 – 13 | 1 | } N = 1 to 13 |
|  | Data Level Code (N) (N = 1 to 13) | 4-bit | N/A | 0 – 15 | 1 | } |
|  | NULL (only nec. if #) | 4-bit | N/A | 0000 | N/A | } rows |
|  | NULL (runs (N)=odd) | 4-bit | N/A | 0000 | N/A | } |
|  | BEGINNING OF THE DPA ALPHANUMERIC LAYER (Note: Offset [0] represents the last NULL byte of Rate Scan layers) |  |  |  |  |  |
| Offset[1] | Text layer divider | INT*2 | N/A | -1 | N/A | OSF doc |
| Offset[2] – Offset[3] | Layer length not including layer divider and layer length | INT*4 | Bytes | 2656 to 3856 | 1 | OSF doc |
| Offset[4] | Text layer packet code | INT*2 | N/A | 1 | N/A | Fig 3-8b |
| Offset[5] | Length of text layer in bytes | INT*2 | Bytes | 2652 to 3852 | 1 | Fig 3-8b |
| Offset[6] | I Starting Point | INT*2 | Km/4 or Pixels | 0 | 1 | Fig 3-8b |
| Offset[7] | J Starting Point | INT*2 | Km/4 or Pixels | 0 | 1 | Fig 3-8b |
|  | BEGINNING OF THE EPRE ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[8] - Offset[11] | Adaptation data header | ASCII*8 | N/A | “ADAP(nn)” | N/A | Product Spec Format VII Note 1; nn (=38) is the number of adaptable params contained in this sub-layer as of ORPG Build 5 |
| Offset[12 - Offset[15] | Width of radar beam | ASCII*8 | Deg | “XXXXX.XX” (space padded) Range: 0.80 to 1.00 Default: 0.90 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[16] - Offset[19] | Blockage Threshold | ASCII*8 | % | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[20] - Offset[23] | Clutter Threshold | ASCII*8 | % | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.01 | Class I Users ICD Format VII Note 1; discussions on REC performance Nov 2003 |
| Offset[24] - Offset[27] | Weight Threshold | ASCII*8 | % | “XXXXX.XX” Range: 0.00 to 100.00 Default: 50.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[28] - Offset[31] | Full Hybrid Scan Threshold | ASCII*8 | % | “XXXXX.XX” (space padded) Range: 90.00 to 100.00 Default: 99.70 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[32] - Offset[35] | Low Reflectivity Threshold | ASCII*8 | dBZ | “XXXXX.XX” (space padded) Range: -40.00 to -20.00 Default: -32.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[36] - Offset[39] | Rain Detection Reflectivity Threshold | ASCII*8 | dBZ | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 20.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[40 - Offset[43] | Rain Detection Area Threshold | ASCII*8 | Km2 | “XXXXX.XX” (space padded) Range: 0.00 to 82800.00 Default: 80.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[44] - Offset[47] | Rain Detection Time Threshold | ASCII*8 | Min | “XXXXX.XX” (space padded) Range: 0.00 to 1440.00 Default: 60.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
| Offset[48] - Offset[51] | Z-R Mult. Coef. | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 50.00 to 500.00 Default: 300.00 | 0.01 | Class I Users ICD Format VII Note 1 |
| Offset[52] - Offset[55] | Z-R Power Coef. | ASCII*8 | N/A | “XXXXX.XX” Range: 1.00 to 2.00 Default: 1.40 | 0.01 | Class I Users ICD Format VII Note 1 |
| Offset[56] - Offset[59] | Min. Refl. to Convert to Rate | ASCII*8 | dBZ | “XXXXX.XX” Range: -32.00 to +20.00 Default: 0.00 | 0.01 | Class I Users ICD Format VII Note 1 |
| Offset[60 - Offset[63] | Max. Refl. to Convert to Rate | ASCII*8 | dBZ | “XXXXX.XX” (space padded) Range: 50.00 to 90.00 Default: 70.00 | 0.01 | Class I Users ICD Format VII Note 1 |
| Offset[64] - Offset[67] | Number of Exclusion Zones | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 0.00 to 20.00 Default: 0.00 | 0.01 | Class I Users ICD Format VII Note 1; Dennis’s email to Cheryl on “Final” Adaptation settings 8/28/03 |
|  | BEGINNING OF THE RATE ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[68] - Offset[71] | Max Storm Speed | ASCII*8 | M/S | “XXXXX.XX” (space padded) Range: 10.00 to 40.00 Default: 25.00 | 0.01 |  |
| Offset[72] - Offset[75] | Thresh. Max Time Difference | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 15.00 | 0.01 |  |
| Offset[76] - Offset[79] | Min. Area Time Continuity | ASCII*8 | KM**2 | “XXXXX.XX” (space padded) Range: 50.00 to 1000.00 Default: 200.00 | 0.01 |  |
| Offset[80] - Offset[83] | Time Continuity Parameter #1 | ASCII*8 | 1/HR | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 24.00 | 0.01 |  |
| Offset[84] - Offset[87] | Time Continuity Parameter #2 | ASCII*8 | 1/HR | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 13.20 | 0.01 |  |
| Offset[88] - Offset[91] | Max. Rate Echo Area Change | ASCII*8 | KM**2/HR | “XXXXX.XX” (space padded) Range: 20.00 to 700.00 Default: 200.00 | 0.01 |  |
| Offset[92] - Offset[95] | Range Cut-Off | ASCII*8 | KM | “XXXXX.XX” (space padded) Range: 0.00 to 230.00 Default: 230.00 | 0.01 |  |
| Offset[96] - Offset[99] | Range Effect Coeff. #1 | ASCII*8 | dBR | “XXXXX.XX” (space padded) Range: 0.00 to 3.00 Default: 0.00 | 0.01 |  |
| Offset[100] - Offset[103] | Range Coeff. Coeff. #2 | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 1.00 to 10.00 Default: 1.00 | 0.01 |  |
| Offset[104] - Offset[107] | Range Coeff. Coeff. #3 | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 0.00 to 1.00 Default: 0.00 | 0.01 |  |
| Offset[108] - Offset[111] | Min Precip. Rate for inclusion | ASCII*8 | MM/HR | “XXXXX.XX” (space padded) Range: 0.00 to 10.00 Default: 0.00 | 0.01 |  |
| Offset[112] - Offset[115] | Max Precip. Rate allowed | ASCII*8 | MM/HR | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 103.80 | 0.01 |  |
|  | BEGINNING OF THE ACCUM ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[116] - Offset[119] | Thresh. Elapsed Time to Restart | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 45.00 to 60.00 Default: 60.00 | 0.01 |  |
| Offset[120] - Offset[123] | Max. Time for Interpolation | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 15.00 to 60.00 Default: 30.00 | 0.01 |  |
| Offset[124] - Offset[127] | Min. Time in Hourly Period | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 45.00 to 60.00 Default: 54.00 | 0.01 |  |
| Offset[128] - Offset[131] | Threshold Hourly Outlier | ASCII*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 800.00 Default: 400.00 | 0.01 |  |
| Offset[132] - Offset[135] | End Time Gage Accumulation | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 0.00 to 59.00 Default: 0.00 | 0.01 |  |
| Offset[136] - Offset[139] | Max Period Accum Value | ASCII*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 400.00 Default: 400.00 | 0.01 |  |
| Offset[140] - Offset[143] | Max Hourly Accum Value | ASCII*8 | MM | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 800.00 | 0.01 |  |
|  | BEGINNING OF THE ADJUSTMENT ALGORITHM ADAPTATION DATA SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[144] - Offset[147] | Time Bias Estimation | ASCII*8 | MIN | “XXXXX.XX” (space padded) Range: 50.00 to 59.00 Default: 50.00 | 0.01 |  |
| Offset[148] - Offset[151] | Thresh. No. Gage-Radar Pairs | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 6.00 to 30.00 Default: 10.00 | 0.01 |  |
| Offset[152] - Offset[155] | Reset Bias Value | ASCII*8 | N/A | “XXXXX.XX” (space padded) Range: 0.50 to 2.00 Default: 1.00 | 0.01 |  |
| Offset[156] - Offset[159] | Longest Allowable Lag | ASCII*8 | HOURS | “XXXXX.XX” (space padded) Range: 100.00 to 1000.00 Default: 168.00 | 0.01 |  |
| Offset[160] - Offset[163] | Bias Applied Flag | ASCII*8 | N/A | “XXXXXXXX” (space padded) Range: T or F Default: F | N/A |  |
|  | BEGINNING OF THE BIAS TABLE SUB-LAYER OF DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[164] - Offset[167] | Bias Table data header | ASCII*8 | N/A | “BIAS(nn)” | N/A | From HSEB DPA Decoder doc; nn (=13) is the number of lines of information including header and data lines contained in this sub-layer as of ORPG Build 5 |
| Offset[168] - Offset[207] | Bias Table Title Line 1 | ASCII*80 | N/A | “GAGE-RADAR MEAN FIELD BIAS TABLE “ | N/A |  |
| Offset[208] - Offset[249] | Bias Table Title Line 2 | ASCII*80 | N/A | “LAST BIAS UPDATE TIME: MM/DD/YY HH:MM BIAS APPLIED? XXX” Range: “YES” or “ NO “ Default: NO | N/A | MM/DD/YY and HH:MM filled in appropriately |
| Offset[248] - Offset[287] | Bias Table Title Line 3 | ASCII*80 | N/A | “MSPAN (HRS) NO. G_R PAIRS AVG. GAGE(MM) AVG. RADAR(MM) MEAN FLD BIAS“ | N/A |  |
| Offset[288] - Offset[307] | Row 1: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 0.1 | 0.001 |  |
|  | Row 1: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 1: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 1: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 1: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.01 to 100.0 Default: 1.000 | 0.001 |  |
| Offset[308] - Offset[327] | Row 2: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 1.000 | 0.001 |  |
|  | Row 2: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 2: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 2: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 2: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.01 to 100. Default: 1.000 | 0.001 |  |
| Offset[328] - Offset[347] | Row 3: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 2.000 | 0.001 |  |
|  | Row 3: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 3: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 3: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 3: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[348] - Offset[367] | Row 4: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 3.000 | 0.001 |  |
|  | Row 4: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 4: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 4: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 4: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[368] - Offset[387] | Row 5: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 5.000 | 0.001 |  |
|  | Row 5: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 5: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 5: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 5: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[388] - Offset[407] | Row 6: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 10.000 | 0.001 |  |
|  | Row 6: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 6: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 6: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 6: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[408] - Offset[427] | Row 7: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 168.000 | 0.001 |  |
|  | Row 7: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 7: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 7: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 7: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[428] - Offset[447] | Row 8: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 720.000 | 0.001 |  |
|  | Row 8: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 8: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 8: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 8: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[448] - Offset[467] | Row 9: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 2160.000 | 0.001 |  |
|  | Row 9: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 9: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 9: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 9: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
| Offset[468] - Offset[487] | Row 10: MEMORY SPAN (HOURS) | ASCII*11 |  | “XXXXXXX.XXX” (space padded) Range: 0.001 to 10.**7 Default: 10.**7 | 0.001 |  |
|  | Row 10: NO. G-R PAIRS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 999.999 Default: 0.000 | 0.001 |  |
|  | Row 10: AVG. GAGE VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 10: AVG. RADAR VALUE (MM) | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 400.000 Default: 0.000 | 0.001 |  |
|  | Row 10: MEAN-FIELD BIAS | ASCII*7 |  | “XXX.XXX” (space padded) Range: 0.000 to 100. Default: 1.000 | 0.001 |  |
|  | BEGINNING OF THE SUPPLEMENTAL DATA SUB-LAYER OF THE DPA ALPHANUMERIC LAYER |  |  |  |  |  |
| Offset[488] - Offset[491] | Supplemental data header | ASCII*8 |  | “SUPL(LL)” | N/A | From HSEB DPA Decoder doc; LL is the number of elements of information which follow in the sub- layer (15 + nn as of ORPG Build 5) |
| Offset[492] - Offset[531] | Rate Scan 1 | ASCII*80 | N/A | “RATE SCAN 1 DATE: dddd TIME: ttttt” | N/A | Date format is “adjusted” Julian date (number of days since Jan 1 1970) Time format is number of seconds |
|  | … | … | … | … | … |  |
|  | … | … | … | … | … |  |
| Offset[532]- Offset[571] | Rate Scan nn (Note: nn can range from 1 to 16) | ASCII*80 | N/A | “RATE SCAN nn DATE: dddd TIME: ttttt” | N/A | Date format is “adjusted” Julian date (number of days since Jan 1 1970) Time format is number of seconds |
| Offset[572]- Offset[611] | Hourly Accumulation End Date | ASCII*80 | N/A | “HOURLY ACCUMULATION END DATE…: dddd” | 1 | Date format is “adjusted” Julian date (number of days since Jan 1 19770) |
| Offset[612]- Offset[651] | Hourly Accumulation End Time | ASCII*80 | N/A | “HOURLY ACCUMULATION END TIME…: ttttt” | 1 | Time format is number of seconds |
| Offset[652]- Offset[691] | Total No. of Blockage Bins Rejected | ASCII*80 | N/A | “TOTAL NO. OF BLOCKAGE BINS REJECTED: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[692]- Offset[731] | Total No. of Clutter Bins Rejected | ASCII*80 | N/A | “TOTAL NO. OF CLUTTER BINS REJECTED.: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[732d]- Offset[771] | Number of Bins Smoothed | ASCII*80 | N/A | “NUMBER OF BINS SMOOTHED……..: xxxxxx” Range: 0 to 82800 Default: 0 | 1 | Number is padded with spaces |
| Offset[772]- Offset[811] | Percent of Hybrid Scan Bins Filled | ASCII*80 | % | “PERCENT OF HYBRID SCANS FILLED.: xx.xx” Range: 90.00 to 100.00 Default: 99.75 | 0.01 | Number is padded with spaces |
| Offset[812]- Offset[851] | Highest Elevation Angle Used in Hybrid Scan | ASCII*80 | Deg | “HIGHEST ELEVATION ANGLE USED IN HYBSCAN: xxx.xx” Range: 0.00 to 19.50 Default: 3.50 | 0.01 | Number is padded with spaces |
| Offset[852]- Offset[891] | Total Hybrid Scan Rain Area | ASCII*80 | Km2 | “TOTAL HYBRID SCAN RAIN AREA……..: xxxxx.xx” Range: 0.00 to 82800.00 Default: 80.00 | 0.01 | Number is padded with spaces |
| Offset[892]- Offset[931] | Number of Bad Scans in Hour | ASCII*80 | N/A | “NUMBER OF BAD SCANS IN HOUR…….: xx” Range: 0 to 16 Default: 0 | 1 | Number is padded with spaces |
| Offset[932]- Offset[971] | Bias Estimate | ASCII*80 | N/A | “BIAS ESTIMATE…………….: x.xx” Range: 0.01 to 100.00 Default : 1.00 | 0.01 | Number is padded with spaces |
| Offset[972]- Offset[1011] | Effective No. Gage-Radar Pairs | ASCII*80 | N/A | “EFFECTIVE # G/R PAIRS…….: x.xx” Range: 6.00 to 30.00 Default: 10.00 | 0.01 | Number is padded with spaces |
| Offset[1012]- Offset[1051] | Memory Span (Hours) | ASCII*80 | N/A | “MEMORY SPAN (HOURS)………..: xx.xxx” Range: 0.001 to 10.**7 Default: 10.00 | 0.01 | Number is padded with spaces |
| Offset[1052]- Offset[1091] | Current Volume Coverage Pattern | ASCII*80 | N/A | “CURRENT VOLUME COVERAGE PATTERN….: xxx” Range: 11,12, 21, 121, etc. Default: 11 | 1 | Number is padded with spaces |
| Offset[1092]- Offset[1131] | Current Operational (Weather) Mode | ASCII*80 | N/A | “CURRENT OPERATIONAL (WEATHER) MODE.: x” Range: A or B Default: A | N/A |  |
| Offset[1132]- Offset[1171] | No Missing Periods in Current Hour | ASCII*80 | N/A | “NO MISSING PERIODS IN CURRENT HOUR” |  |  |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT USED BY DPA]

[TABULAR ALPHANUMERIC BLOCK IS NOT USED BY DPA]

References

“Decoding of DPA Products”, published by OHD/HSEB, updated as of AWIPS Release OB3 (1/8/04)

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| (n) – (n+29) | Blank | ASCII | N/A | “ “ | N/A |  |
| (n+30) – (n+…..) | Nulls | INT*2 | N/A | 0 | N/A | Variable number of Nulls |

The following is an example of the alphanumeric layer of the DPA product, as rendered by CODE cvt.

ADAP(38)    0.90   50.00   50.00   50.00   99.70  -32.00   20.00   80.00   60.00  300.00    1.40    0.00   70.00    0.00      25.00   15.00  200.00   24.00   13.20  200.00  230.00    0.00    1.00    0.00    0.00  103.80   60.00   30.00   54.00  400.00    0.00  400.00  800.00   50.00   10.00    1.00  168.00       FBIAS(13)

GAGE-RADAR MEAN FIELD BIAS TABLE

LAST BIAS UPDATE TIME:  12/31/** 00:00                      BIAS APPLIED ?   NO

MSPAN (HRS)    NO. G_R PAIRS   AVG. GAGE(MM)   AVG. RADAR(MM)   MEAN FLD BIAS

0.000            	0.000           	0.000           		 0.000		   0.000

0.000           	0.000           	0.000          		 0.000  	         	   0.000

0.000           	0.000           	0.000           		 0.000          	   0.000

0.000           	0.000           	0.000           		 0.000           	   0.000

0.000           	0.000           	0.000           		 0.000           	   0.000

0.000           	0.000           	0.000           		 0.000           	   0.000

0.000           	0.000           	0.000           		 0.000           	   0.000

0.000           	0.000           	0.000           		 0.000                      0.000

0.000           	0.000           	0.000           		 0.000                      0.000

0.000          	0.000           	0.000           		 0.000           	   0.000

SUPL(27) RATE SCAN  1 DATE:   9534 TIME:27072

RATE SCAN  2 DATE:   9534 TIME:27392

RATE SCAN  3 DATE:   9534 TIME:27776

RATE SCAN  4 DATE:   9534 TIME:28160

RATE SCAN  5 DATE:   9534 TIME:28480

RATE SCAN  6 DATE:   9534 TIME:28864

RATE SCAN  7 DATE:   9534 TIME:29248

RATE SCAN  8 DATE:   9534 TIME:29568

RATE SCAN  9 DATE:   9534 TIME:29952

RATE SCAN 10 DATE:   9534 TIME:30272

RATE SCAN 11 DATE:   9534 TIME:30592

RATE SCAN 12 DATE:   9534 TIME:30976

HOURLY ACCUMULATION END DATE.......:    9534

HOURLY ACCUMULATION END TIME.......:   30976

TOTAL NO. OF BLOCKAGE BINS REJECTED:28305

TOTAL NO. OF CLUTTER BINS REJECTED.:     2024

NUMBER OF BINS SMOOTHED............:                    0

PERCENT OF HYBRID SCAN BINS FILLED.:     99.88

HIGHEST ELEV. ANGLE USED IN HYBSCAN:    3.40

TOTAL HYBRID SCAN RAIN AREA........:     18304.99

NUMBER OF BAD SCANS IN HOUR........:                 0

BIAS ESTIMATE......................:                                1.00

EFFECTIVE # G/R PAIR...............:                           0.00

MEMORY SPAN (HOURS)................:                      0.00

CURRENT VOLUME COVERAGE PATTERN....:      21

CURRENT OPERATIONAL (WEATHER) MODE.:     2

NO MISSING PERIODS IN CURRENT HOUR