Precipitation Processing System (PPS) Product Format Description

Digital Storm-Total Precipitation (DSP) Product

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 256-data level |  |
| Time generated | Once every volume scan |  |
| Product size | 44628 |  |
| Compression | Yes |  |
| Resolution | 2-km x 1-deg polar grid |  |
| AWIPS ID | WSRDSPxxx | xxx is site ID for originating WFO |
| Description/Purpose The basic output units are of .01" resolution, scaled as a function of the maximum data value in the output grid as follows: if the max data value is less than or equal to 2.55", the scaling factor (or increment between data values) is .01"; if the max data value exceeds 2.55" but is less than or equal to 5.10", the scaling factor is .02"; if the max data value exceeds (2 x 2.55") but is less than or equal to (3 x 2.55"), the scaling factor is .03"; etc. Level code 0 will correspond to no accumulation and level code 255 will indicate missing data. Level codes 1 through 250 will indicate accumulations, with a minimal step of 0.01 inches. For every multiple of 2.50 inches exceeded, the scale will increment by a corresponding integer multiple. The DSP product will also generate an appended layer of ASCII data containing ancillary information such as present adaptation data settings and supplemental results determined during the algorithm processing. |  |  |

The following table provides a detailed specification of the DSP product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: (INT*2) is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 138 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 44628 | 1 | CCR#NA04-32201 |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD)

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 138 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Scan Beginning Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 28 | Scan Beginning Time | INT*2 | Seconds | 0 to 86399 | 1 |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Mean-Field Bias | INT*2 | N/A | .01-100. | 0.01 |  |
| 31 | Minimum DSP data level | INT*2 | N/A | 0 | 1 |  |
| 32 | Data Level Scaled Factor | INT*2 | N/A | 0.01 to 1.29 | 0.01 |  |
| 33 | Number of data levels | INT*2 | N/A | 256 | 1 |  |
| 34 | Not used | INT*2 | N/A | 0 | N/A |  |
| 35 | Not used | INT*2 | N/A | 0 | N/A |  |
| 36 | Not used | INT*2 | N/A | 0 | N/A |  |
| 37 | Not used | INT*2 | N/A | 0 | N/A |  |
| 38 | Not used | INT*2 | N/A | 0 | N/A |  |
| 39 | Not used | INT*2 | N/A | 0 | N/A |  |
| 40 | Not used | INT*2 | N/A | 0 | N/A |  |
| 41 | Not used | INT*2 | N/A | 0 | N/A |  |
| 42 | Not used | INT*2 | N/A | 0 | N/A |  |
| 43 | Not used | INT*2 | N/A | 0 | N/A |  |
| 44 | Not used | INT*2 | N/A | 0 | N/A |  |
| 45 | Not used | INT*2 | N/A | 0 | N/A |  |
| 46 | Not used | INT*2 | N/A | 0 | N/A |  |
| 47 | Maximum Precip. Value | INT*2 | Inch | 0.0 to 327.6 | 0.1 | Correction to original description (no format change) |
| 48 | Scan Ending Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 49 | Scan Ending Time | INT*2 | Minutes | 1 – 1439 | 1 |  |
| 50 | Effective Number G-R pairs (sample size) | INT*2 | N/A | .00 to 9999.99 | 0.01 |  |
| 51 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Product Symbology block | INT*4 | Half-words | 60 | 1 |  |
| 57-58 | Offset to Graphic Attributes block | INT*4 | Half-words | 0 | 1 |  |
| 59-60 | Offset to Graphic Alphanumeric block | INT*4 | Half-words | 0 | 1 |  |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” (Sheet 8)

Fig 3-8b “Text and Special Symbol Packets”

Fig 3-11c  “Digital Radial Data Array Packet”

2620003 (ICD for Product Specification):

Appendix C, Format VIII “DSP and ….”

| HALF-WORD | FIELDNAME | TYPE |  | UNITS |  | VALUE |  | PRECISION/ ACCURACY |  | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 62 | Block ID | INT*2 |  | N/A |  | 1 |  | N/A |  |  |
| 63-64 | Length of Block | INT*4 |  | Bytes |  | 44508 |  | 1 |  | CCR#NA04-32201 |
| 65 | Number of Layers | INT*2 |  | N/A |  | 2 |  | 1 |  |  |
| 66 | Layer Divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 67-68 | Length of Data Layer not including layer divider and layer length | INT*4 |  | N/A |  | 43934 |  | 1 |  |  |
|  | BEGINNING OF THE DSP DATA LAYER |  |  |  |  |  |  |  |  |  |
| 69 | Packet Code | INT*2 |  | N/A |  | 16 |  | N/A |  |  |
| 70 | Index of First Range Bin | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 71 | Number of Range Bins | INT*2 |  | N/A |  | 116 |  | 1 |  |  |
| 72 | I Center of Sweep | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 73 | J Center of Sweep | INT*2 |  | N/A |  | 0 |  | 1 |  |  |
| 74 | Range Scale Factor (116/#Bins) | Scaled Integer |  | N/A |  | 2.0 |  | 0.001 |  |  |
| 75 | Number of Radials | INT*2 |  | N/A |  | 360 |  | 1 |  |  |
| 76 | Number of Bytes in Radial | INT*2 |  | N/A |  | 116 |  | 1 |  | }Repeat }for |
| 77 | Radial Start Angle | Scaled Integer |  | Degrees |  | 0.0 to 359.9 |  | 0.1 |  | } |
| 78 | Radial Delta Angle | Scaled Integer |  | Degrees |  | 1.0 |  | 0.1 |  | }Each } |
| 79 | Level 0 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level 1 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
| 80 | Level 2 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level 3 | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
| … | … | … |  | … |  | … |  | … |  | } |
| … | … | … |  | … |  | … |  | … |  | } |
| 136 | Level (N-1) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | } |
|  | Level (N) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }Row |
| … | … | … |  | … |  | … |  | … |  |  |
| … | … | … |  | … |  | … |  | … |  | }Between |
| … | … | … |  | … |  | … |  | … |  | } |
| … | … | … |  | … |  | … |  | … |  | }Radial 2 |
| 22035 | Level(N-1) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }And |
|  | Level(N) | INT*1 |  | N/A |  | 0 to 255 |  | 1 |  | }Radial 360 |
|  | BEGINNING OF THE DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22036 | Text layer divider | INT*2 |  | N/A |  | -1 |  | N/A |  |  |
| 22037-22038 | Layer length not including layer divider and layer length | INT*4 |  | N/A |  | 552 |  | 1 |  | CCR#NA04-32201 |
| 22039 | Text layer packet code | INT*2 |  | N/A |  | 1 |  | N/A |  |  |
| 22040 | Length of text layer in bytes | INT*2 |  | N/A |  | 548 |  | 1 |  | CCR#NA04-32201 |
| 22041 | I Starting Point | INT*2 |  | Km/4 or Pixels |  | 0 |  | 1 |  |  |
| 22042 | J Starting Point | INT*2 |  | Km/4 or Pixels |  | 0 |  | 1 |  |  |
|  | BEGINNING OF PRECIP STATUS MESSAGE SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22043-22046 | Precip Status Message (PSM) header | CHAR*8 |  | N/A |  | “PSM( 6)” (space padded) |  | N/A |  | Correction to original description (no format change) |
| 22047-22050 | Current Date Precip Function Ran | CHAR*8 |  | JULIAN DAYS |  | 0 – 99999 |  | 1 |  |  |
| 22051-22054 | Current Time Precip Function Ran | CHAR*8 |  | SEC |  | 0 – 86399 |  | 1 |  |  |
| 22055-22058 | Last Date Precip Detected | CHAR*8 |  | JULIAN DAYS |  | 0 – 99999 |  | 1 |  |  |
| 22059-22062 | Last Time Precip Detected | CHAR*8 |  | SEC |  | 0 – 86399 |  | 1 |  |  |
| 22063-22066 | Current Precip Category | CHAR*8 |  | N/A |  | 0 to 2 |  | 1 |  | Correction to original description (no format change) |
| 22067-22070 | Previous Precip Category | CHAR*8 |  | N/A |  | 0 to 2 |  | 1 |  | Correction to original description (no format change) |
|  | BEGINNING OF THE EPRE ADAPTATION DATA SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22071-22074 | Adaptation data header | CHAR*8 |  | N/A |  | “ADAP(32)” |  | N/A |  | CCR#NA04-32201 |
| 22075-22078 | Width of radar beam | CHAR*8 |  | Deg |  | “XXXXX.XX” (space padded) Range: 0.80 to 1.00 Default: 0.90 |  | 0.01 |  |  |
| 22079-22082 | Blockage Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 22083-22086 | Clutter Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 22087-22090 | Weight Threshold | CHAR*8 |  | % |  | “XXXXX.XX” Range: 0.00 to 100.00 Default: 50.00 |  | 0.01 |  |  |
| 22091-22094 | Full Hybrid Scan Threshold | CHAR*8 |  | % |  | “XXXXX.XX” (space padded) Range: 90.00 to 100.00 Default: 99.70 |  | 0.01 |  |  |
| 22095-22098 | Low Reflectivity Threshold | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: -40.00 to -20.00 Default: -32.00 |  | 0.01 |  |  |
| 22099-22102 | Rain Detection Reflectivity Threshold | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 20.00 |  | 0.01 |  |  |
| 22103-22106 | Rain Detection Area Threshold | CHAR*8 |  | Km2 |  | “XXXXX.XX” (space padded) Range: 0.00 to 82800.00 Default: 80.00 |  | 0.01 |  |  |
| 22107-22110 | Rain Detection Time Threshold | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 1440.00 Default: 60.00 |  | 0.01 |  | Correction to original description (no format change) |
| 22111-22114 | Z-R Mult. Coef. | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 50.00 to 500.00 Default: 300.00 |  | 0.01 |  |  |
| 22115-22118 | Z-R Power Coef. | CHAR*8 |  | N/A |  | “XXXXX.XX” Range: 1.00 to 2.00 Default: 1.40 |  | 0.01 |  |  |
| 22119-22122 | Min. Refl. To Convert to Rate | CHAR*8 |  | dBZ |  | “XXXXX.XX” Range: -32.00 to +20.00 Default: 0.00 |  | 0.01 |  |  |
| 22123-22126 | Max. Refl. to Convert to Rate | CHAR*8 |  | dBZ |  | “XXXXX.XX” (space padded) Range: 50.00 to 90.00 Default: 70.00 |  | 0.01 |  |  |
| 22127-22130 | Number of Exclusion Zones | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 0.00 to 20.00 Default: 0.00 |  | 1.00 |  | Correction to original description (no format change) |
|  | BEGINNING OF THE RATE ALGORITHM ADAPTATION DATA SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22131-22134 | Max Storm Speed | CHAR*8 |  | M/S |  | “XXXXX.XX” (space padded) Range: 10.00 to 40.00 Default: 25.00 |  | 0.01 |  | CCR#NA04-32201 |
| 22135-22138 | Thresh. Max Time Difference | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 15.00 |  | 0.01 |  | CCR#NA04-32201 |
| 22139-22142 | Min. Area Time Continuity | CHAR*8 |  | KM**2 |  | “XXXXX.XX” (space padded) Range: 50.00 to 1000.00 Default: 200.00 |  | 0.01 |  | CCR#NA04-32201 |
| 22143-22146 | Time Continuity Parameter #1 | CHAR*8 |  | 1/HR |  | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 24.00 |  | 0.01 |  | CCR#NA04-32201 |
| 22147-22150 | Time Continuity Parameter #2 | CHAR*8 |  | 1/HR |  | “XXXXX.XX” (space padded) Range: 0.10 to 99.90 Default: 13.20 |  | 0.01 |  | CCR#NA04-32201 |
| 22151-22154 | Max. Rate Echo Area Change | CHAR*8 |  | KM**2/HR |  | “XXXXX.XX” (space padded) Range: 20.00 to 700.00 Default: 200.00 |  | 0.01 |  | CCR#NA04-32201 |
| Note: as a result of implementing this CCR the halfword addresses of all remaining fields have changed |  |  |  |  |  |  |  |  |  |  |
| 22131-22134 | Range Cut-Off | CHAR*8 |  | KM |  | “XXXXX.XX” (space padded) Range: 0.00 to 230.00 Default: 230.00 |  | 1.00 |  | Correction to original description (no format change) |
| 22135-22138 | Range Effect Coeff. #1 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 0.00 to 3.00 Default: 0.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 22139-22142 | Range Coeff. Coeff. #2 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 1.00 to 10.00 Default: 1.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 22143-22146 | Range Coeff. Coeff. #3 | CHAR*8 |  | dBR |  | “XXXXX.XX” (space padded) Range: 0.00 to 1.00 Default: 0.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 22147-22150 | Min Precip. Rate for inclusion | CHAR*8 |  | MM/HR |  | “XXXXX.XX” (space padded) Range: 0.00 to 10.00 Default: 0.00 |  | 0.10 |  | Corrections to original description (no format change) |
| 22151-22154 | Max Precip. Rate allowed | CHAR*8 |  | MM/HR |  | “XXXXX.XX” (space padded) Range: 50.00 to 1600.00 Default: 103.80 |  | 0.10 |  | Corrections to original description (no format change) |
|  | BEGINNING OF THE ACCUM ALGORITHM ADAPTATION DATA SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22155-22158 | Thresh. Elapsed Time to Restart | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 45.00 to 60.00 Default: 60.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22159-22162 | Max. Time for Interpolation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 15.00 to 60.00 Default: 30.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22163-22166 | Min. Time in Hourly Period | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 60.00 Default: 54.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22167-22170 | Threshold Hourly Outlier | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 50.00 to 800.00 Default: 400.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22171-22174 | End Time Gage Accumulation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 0.00 to 59.00 Default: 0.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22175-22178 | Max Period Accum Value | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 50.00 to 400.00 Default: 400.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22179-22182 | Max Hourly Accum Value | CHAR*8 |  | MM |  | “XXXXX.XX” (space padded) Range: 200.00 to 1600.00 Default: 800.00 |  | 1.00 |  | Correction to original description (no format change) |
|  | BEGINNING OF THE ADJUSTMENT ALGORITHM ADAPTATION DATA SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22183-22186 | Time Bias Estimation | CHAR*8 |  | MINS |  | “XXXXX.XX” (space padded) Range: 50.00 to 59.00 Default: 50.00 |  | 1.00 |  | Corrections to original description (no format change) |
| 22187-22190 | Thresh. No. Gage-Radar Pairs | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 6.00 to 30.00 Default: 10.00 |  | 1.00 |  | Correction to original description (no format change) |
| 22191-22194 | Reset Bias Value | CHAR*8 |  | N/A |  | “XXXXX.XX” (space padded) Range: 0.50 to 2.00 Default: 1.00 |  | 0.10 |  | Correction to original description (no format change) |
| 22195-22198 | Longest Allowable Lag | CHAR*8 |  | HRS |  | “XXXXX.XX” (space padded) Range: 100.00 to 1000.00 Default: 168.00 |  | 1.00 |  | Correction to original description (no format change) |
| 22199-22202 | Bias Applied Flag | CHAR*8 |  | N/A |  | “XXXXXXXX” (space padded) Range: T or F Default: F |  | N/A |  |  |
|  | BEGINNING OF THE SUPPLEMENTAL SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22203-22206 | Supplemental data header |  | CHAR*8 |  | N/A |  | “SUPL(15)” |  | N/A | Correction to original description (no format change) |
| 22207-22210 | Average Scan Date |  | CHAR*8 |  | Julian Dalys |  | 0-99999 |  | 1 |  |
| 22211-22214 | Average Scan Time |  | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |
| 22215-22218 | Flag Zero Hybrid |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 22219-22222 | Rain Detection Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 22223-22226 | Reset STP Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 22227-22230 | Precip. Begin Flag |  | CHAR*8 |  | N/A |  | 0 or 1 |  | 1 |  |
| 22231-22234 | Last Date Rain |  | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |
| 22235-22238 | Last Time Rain |  | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |
| 22239-22242 | Rejected Blockage Counter |  | CHAR*8 |  | N/A |  | 0-82800 |  | 1 |  |
| 22243-22246 | Rejected AP/Clutter Counter |  | CHAR*8 |  | N/A |  | 0-82800 |  | 1 |  |
| 22247-22250 | Total Bins Smooth |  | CHAR*8 |  | N/A |  | 0 |  | N/A | Not implemented |
| 22251-22254 | Percentage Filled Hybrid Scan |  | CHAR*8 |  | N/A |  | 0.00 to 100.00 |  | 0.01 |  |
| 22255-22258 | Highest Elevation Angle |  | CHAR*8 |  | N/A |  | 0.50 to 19.50 |  | 0.01 |  |
| 22259-22262 | Rain Summation Area |  | CHAR*8 |  | N/A |  | 0.0-166190.3 |  | 0.1 | CCR #NA04-27811 Corrections to original description (no format change) |
| 22263-22266 | Volume Spot Blank |  | CHAR*8 |  | N/A |  | 0 to 1 |  | 1 |  |
|  | BEGINNING OF THE BIAS TABLE SUB-LAYER OF DSP ALPHANUMERIC LAYER |  |  |  |  |  |  |  |  |  |
| 22267-22270 | Bias Table data header | CHAR*8 |  | N/A |  | “BIAS(11)” |  | N/A |  | Correction to original description (no format change) |
| 22271-22274 | Time Last Update of Local Bias Value | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 22275-22278 | Date Last Update of Local Bias Value | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 22279-22282 | Time of Last Update of Local Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 22283-22286 | Date of Last Update of Local Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 22287-22290 | Observation Time of Latest Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 22291-22294 | Observation Date of Latest Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 22295-22298 | Generation Time of Latest Bias Table | CHAR*8 |  | Sec |  | 0-86399 |  | 1 |  |  |
| 22299-22302 | Generation Date of Latest Bias Table | CHAR*8 |  | Julian Days |  | 0-99999 |  | 1 |  |  |
| 22303-22306 | Mean-Field Bias Estimate | CHAR*8 |  | N/A |  | .01-100. |  | .01 |  |  |
| 22307-22310 | Effective G-R Pair | CHAR*8 |  | N/A |  | 0.00-999.99 |  | .01 |  |  |
| 22311-22314 | Memory Span | CHAR*8 |  | N/A |  | 0.001-10.**7 |  | .001 |  |  |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT USED BY DSP]

[TABULAR ALPHANUMERIC BLOCK IS NOT USED BY DSP]

The following is an example of the alphanumeric layer (layer 2) of the DSP product, as rendered by CODE cvt.

CVT using variable CV_ORPG_BUILD to set ORPG Build to 6

*** ORPG DATABASE PRODUCT LOAD UTILITY ***

-> Number of Products Available=5417

-> Message ID=5398

-> Product Info: LBuffer# 138 MSGLEN 044724 VOLNUM 158 ELEV 07

-> Set Processing ONLY for Layer Number 2

packet code 1 found

Packet 1: Write Text (No Value) Summary Information

Length of Data Block (in bytes) = 548

I Starting Point:                 0

J Starting Point:                 0

Message to follow:

PSM ( 6)   12335   48300   12335   48300       1       1ADAP(32)    0.90   50.00

50.00   50.00   99.70  -32.00   20.00   80.00   60.00  300.00    1.40    0.00

70.00    3.00  230.00    0.00    1.00    0.00    0.00  103.80   60.00   30.00

54.00  400.00    0.00  400.00  800.00   50.00   10.00    1.00  168.00       T

SUPL(15)   12335   48448       0       1       0       0   12335   48448       0

12576       0   99.86    9.90 14772.4       0BIAS(11)   58368   10461       0

0   72000   11695   75453   11695  1.2550   13.49    168.

program complete