Precipitation Processing System (PPS) Product Format Description

Hybrid Scan Reflectivity (HSR)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/31/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 16-level |  |
| Time generated | Once every volume scan |  |
| Product size | N/A |  |
| Compression | None |  |
| Range of data level values (dBZ) | Variable with operation mode, area climatology, and season, and with NEXRAD system adaptation data |  |
| Range of reflectivity | -32 to +95 dBZ |  |
| AWIPS ID | WSRHSRxxx | xxx is site ID for originating WFO |
| Description/Purpose: HSR’s Symbology block only has one layer. |  |  |

The following table provides a detailed specification of the HSR product.

[Note: a half-word (INT*2) is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 33 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD):

1-2 Color Level Code Table

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 33 |  |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 | N/A | 0 | N/A |  |
| 28 | Not used | INT*2 | N/A | 0 | N/A |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Data Level Threshold 1 | INT*2 | dBZ | ND | N/A |  |
| 32 | Data Level Threshold 2 | INT*2 | dBZ | 5 | N/A |  |
| 33 | Data Level Threshold 3 | INT*2 | dBZ | 10 | N/A |  |
| 34 | Data Level Threshold 4 | INT*2 | dBZ | 15 | N/A |  |
| 35 | Data Level Threshold 5 | INT*2 | dBZ | 20 | N/A |  |
| 36 | Data Level Threshold 6 | INT*2 | dBZ | 25 | N/A |  |
| 37 | Data Level Threshold 7 | INT*2 | dBZ | 30 | N/A |  |
| 38 | Data Level Threshold 8 | INT*2 | dBZ | 35 | N/A |  |
| 39 | Data Level Threshold 9 | INT*2 | dBZ | 40 | N/A |  |
| 40 | Data Level Threshold 10 | INT*2 | dBZ | 45 | N/A |  |
| 41 | Data Level Threshold 11 | INT*2 | dBZ | 50 | N/A |  |
| 42 | Data Level Threshold | INT*2 | dBZ | 55 | N/A |  |
| 43 | Data Level Threshold 13 | INT*2 | dBZ | 60 | N/A |  |
| 44 | Data Level Threshold 14 | INT*2 | dBZ | 65 | N/A |  |
| 45 | Data Level Threshold 15 | INT*2 | dBZ | 70 | N/A |  |
| 46 | Data Level Threshold 16 | INT*2 | dBZ | 75 | N/A |  |
| 47 | Maximum Reflectivity | INT*2 | dBZ | 95 | 1 |  |
| 48 | Date of Scan | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 49 | Average Time of Hybrid Scan | INT*2 | Minutes | 1 – 1439 | 1 |  |
| 50 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 51 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 0 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Product Symbology block | INT*4 | Half-words | 60 | 1 |  |
| 57-58 | Offset to Graphic Attributes block | INT*4 | Half-words | 0 | 1 |  |
| 59-60 | Offset to Graphic Alphanumeric block | INT*4 | Half-words | 0 | 1 |  |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” (Sheet 8)

Fig 3-8b “Text and Special Symbol Packets”

Fig 3-10 “Radial Data Packet”

2620003 (ICD for Product Specification):

33-1 HSR Product Description

| HALF-WORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Block ID | INT*2 | N/A | 1 | N/A |  |
| 63-64 | Length of Block | INT*4 | Bytes | 1 to 400000 | 1 |  |
| 65 | Number of Layers | INT*2 | N/A | 1 | 1 |  |
| 66 | Layer Divider | INT*2 | N/A | -1 | N/A |  |
| 67-68 | Length of Data Layer not including layer divider and layer length | INT*4 | N/A | 1 to 400000 | 1 |  |
| 69 | Packet Code | INT*2 | N/A | AF1F(Hex) | N/A |  |
| 70 | Index of First Range Bin | INT*2 | N/A | 0 | 1 |  |
| 71 | Number of Range Bins | INT*2 | N/A | 230 | 1 |  |
| 72 | I Center of Sweep | INT*2 | Km/4 | 256 | 1 |  |
| 73 | J Center of Sweep | INT*2 | Km/4 | 280 | 1 |  |
| 74 | Scale Factor | Scaled Integer | Pixels | 1.0 | 0.001 |  |
| 75 | Number of Radials | INT*2 | N/A | 360 | 1 |  |
| 76 | Number of RLE Halfword in Radial | INT*2 | N/A | 1 to 230 | 1 | }Repeat |
| 77 | Radial Start Angle | Scaled Integer | Degrees | 0.0 to 359.0 | 0.1 | } Each } of } |
| 78 | Radial Angle Delta | Scaled Integer | Degrees | 1.0 to 2.0 | 0.1 | } Radial } } |
| 79 | Run (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Run (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| … | … | … | … | … | … | } |
|  | Run (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
|  | Color Code (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT USED BY HSR]

[TABULAR ALPHANUMERIC BLOCK IS NOT USED BY HSR]