Precipitation Processing System (PPS) Product Format Description

One Hour Surface Rainfall Accumulation (OHP)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 16-level |  |
| Time generated | Once every volume scan | It generates the blank images before it reaches the first hour. The image of Rainfall accumulation changes with every hour |
| Product size | N/A |  |
| Compression | None |  |
| Resolution | 1.1-Nmi x 1-deg |  |
| Range | 124 |  |
| AWIPS ID | WSROHPxxx | xxx is site ID for originating WFO |
| Description/Purpose: Rainfall accumulation maps displayed as an image. |  |  |

The following table provides a detailed specification of the OHP product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a Halfword is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 78 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 4 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 78 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 | N/A | 0 | N/A |  |
| 28 | Not used | INT*2 | N/A | 0 | N/A |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Data Level 1 Threshold | INT*2 | Inches | Code for ND (i.e. 0) | 1 |  |
| 32 | Data Level 2 Threshold | INT*2 | Inches | Code for > 0.00 | 1 |  |
| 33 | Data Level 3 Threshold | INT*2 | Inches | Code for > 0.10 | 1 |  |
| 34 | Data Level 4 Threshold | INT*2 | Inches | Code for > 0.25 | 1 |  |
| 35 | Data Level 5 Threshold | INT*2 | Inches | Code for > 0.50 | 1 |  |
| 36 | Data Level 6 Threshold | INT*2 | Inches | Code for > 0.75 | 1 |  |
| 37 | Data Level 7 Threshold | INT*2 | Inches | Code for > 1.00 | 1 |  |
| 38 | Data Level 8 Threshold | INT*2 | Inches | Code for > 1.25 | 1 |  |
| 39 | Data Level 9 Threshold | INT*2 | Inches | Code for > 1.50 | 1 |  |
| 40 | Data Level 10 Threshold | INT*2 | Inches | Code for > 1.75 | 1 |  |
| 41 | Data Level 11 Threshold | INT*2 | Inches | Code for > 2.00 | 1 |  |
| 42 | Data Level 12 Threshold | INT*2 | Inches | Code for > 2.50 | 1 |  |
| 43 | Data Level 13 Threshold | INT*2 | Inches | Code for > 3.00 | 1 |  |
| 44 | Data Level 14 Threshold | INT*2 | Inches | Code for > 4.00 | 1 |  |
| 45 | Data Level 15 Threshold | INT*2 | Inches | Code for > 6.00 | 1 |  |
| 46 | Data Level 16 Threshold | INT*2 | Inches | Code for > 8.00 | 1 |  |
| 47 | Max rainfall | INT*2 | Inches | 0.0 to 189.0 | 0.1 |  |
| 48 | Mean-field Bias | INT*2 | N/A | 0.01 to 99.99 | 0.01 |  |
| 49 | Effective No. G-R Pairs (Sample Size) | INT*2 | N/A | 0.00 to 9999.99 | 0.01 |  |
| 50 | Rainfall End Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 51 | Rainfall End Time | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 1 to 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Symbology | INT*4 | Halfwords | 60 | 1 |  |
| 57-58 | Offset to Graphic | INT*4 | Halfwords | 0 | 1 |  |
| 59-60 | Offset to Tabular | INT*4 | Halfwords | 0 to 400000 | 1 | Varies, depending on size of Symbology block |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” Sheet 3 and Sheet 8

Fig 3-10 “Radial Data Packet”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Block ID | INT*2 | N/A | 1 | N/A |  |
| 63-64 | Length of Block | INT*4 | Bytes | 1 to 400000 | 1 |  |
| 65 | Number of Layers | INT*2 | N/A | 1 | 1 |  |
| 66 | Layer Divider | INT*2 | N/A | -1 | N/A |  |
| 67-68 | Length of Data Layer | INT*4 | N/A | 1 to 400000 | 1 |  |
| 69 | Packet Code | INT*2 | N/A | AF1F (Hex) | N/A |  |
| 70 | Index of First Range Bin | INT*2 | N/A | 0 | 1 |  |
| 71 | Number of Range Bins | INT*2 | N/A | 115 | 1 |  |
| 72 | I Center of Sweep | INT*2 | Km/4 | 256 | 1 |  |
| 73 | J Center of Sweep | INT*2 | Km/4 | 280 | 1 |  |
| 74 | Scale Factor | Scaled Integer | Pixels | 2.0 | 0.001 |  |
| 75 | Number of Radials | INT*2 | N/A | 360 | 1 |  |
| 76 | Number of RLE Halfwords in Radial | INT*2 | Halfword | 1 to 116 | 1 | }Repeat |
| 77 | Radial Start Angle | Scaled Integer (INT*2) | Degrees | 0.0 to 359.0 | 0.1 | } Each } of } |
| 78 | Radial Angle Delta | Scaled Integer (INT*2) | Degrees | 1.0 to 2.0 | 0.1 | } Radial } } |
| 79 | Run (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Run (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| … | … | … | … | … | … | } |
| … | … | … | … | … | … | } } |
| … | … | … | … | … | … | } |
|  | Run (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
|  | Color Code (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT APPLICABLE FOR OHP]

TABULAR ALPHANUMERIC BLOCK – CONTAINS OHP PAIRED ALPHANUMERIC PRODUCT

References

2620001F:

Section 3.2.1.4 “Tabular Alphanumeric Block”

Fig 3-3 “Message Header”

Fig 3-6 “Graphic Product Message” Sheet 7 and Sheet 10

Fig 3-10 “Radial Data Packet”

From Table V “Product Dependent Halfword Definition for Product Description Block”

26200003F:

Appendix C, Format IX “Precipitation Adaptation Data”, Sheets 1-4

ICD for the RPG to Class I User:

3-28 (Sheet 7)

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Offset+1 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 2 | Block ID | INT*2 | N/A | 3 | N/A |  |
| 3-4 | Length of Block | INT*4 | Bytes | 1 to 65535 | 1 |  |
| 5 | Message Code | INT*2 | N/A | 107 | N/A |  |
| 6 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 7-8 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 9-10 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 11 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 12 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 13 | Number of Blocks | INT*2 | N/A | 4 | 1 |  |
| 14 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 15-16 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 17-18 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 19 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 20 | Product Code | INT*2 | N/A | 78 | N/A |  |
| 21 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 22 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 23 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 24 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 25 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 26-27 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 28 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 29-30 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 31 | Not Used | INT*2 | NA | 0 | N/A |  |
| 32 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 33 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 34 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 35 | Data Level 1 Threshold | INT*2 | Inches | Code for ND (i.e. 0) | 1 |  |
| 36 | Data Level 2 Threshold | INT*2 | Inches | Code for >0.00 | 1 |  |
| 37 | Data Level 3 Threshold | INT*2 | Inches | Code for >0.10 | 1 |  |
| 38 | Data Level 4 Threshold | INT*2 | Inches | Code for >0.25 | 1 |  |
| 39 | Data Level 5 Threshold | INT*2 | Inches | Code for >0.50 | 1 |  |
| 40 | Data Level 6 Threshold | INT*2 | Inches | Code for >0.75 | 1 |  |
| 41 | Data Level 7 Threshold | INT*2 | Inches | Code for >1.00 | 1 |  |
| 42 | Data Level 8 Threshold | INT*2 | Inches | Code for >1.25 | 1 |  |
| 43 | Data Level 9 Threshold | INT*2 | Inches | Code for >1.50 | 1 |  |
| 44 | Data Level 10 Threshold | INT*2 | Inches | Code for >1.75 | 1 |  |
| 45 | Data Level 11 Threshold | INT*2 | Inches | Code for >2.00 | 1 |  |
| 46 | Data Level 12 Threshold | INT*2 | Inches | Code for >2.50 | 1 |  |
| 47 | Data Level 13 Threshold | INT*2 | Inches | Code for >3.00 | 1 |  |
| 48 | Data Level 14 Threshold | INT*2 | Inches | Code for > 4.00 | 1 |  |
| 49 | Data Level 15 Threshold | INT*2 | Inches | Code for >6.00 | 1 |  |
| 50 | Data Level 16 Threshold | INT*2 | Inches | Code for >8.00 | 1 |  |
| 51 | Max rainfall | INT*2 | Inches | 0.0 to 189.0 | 0.1 |  |
| 52 | Mean-field Bias | INT*2 | N/A | 0.01 to 99.99 | 0.01 |  |
| 53 | Effective No. G-R Pairs (Sample Size) | INT*2 | N/A | 0.00 to 9999.99 | 0.01 |  |
| 54 | Rainfall End Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 55 | Rainfall End Time | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 56 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 57 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 58 | Version | INT*1 | N/A | 1 to 2 | 1 |  |
| 58 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 59-60 | Offset to Symbology | INT*4 | Halfwords | 60 | 1 |  |
| 61-62 | Offset to Graphic | INT*4 | Halfwords | 0 | 1 |  |
| 63-64 | Offset to Tabular | INT*4 | Halfwords | 0 to 400000 | 1 |  |
| 65 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 66 | Number of Pages | INT*2 | N/A | 5 | 1 |  |
| 67 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 68-107 | Title | CHAR*80 | 8 Bit ASCII | 1-HOUR PRECIPITATION ACCUMULATION MM/DD/YYHH:MM (space padded) | N/A | Corrections to original description (no format change |
| 108 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 109-148 | Blank Line | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A | Correction to original description (no format change |
| 149 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 150-189 | Blank Line | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A | Correction to original description (no format change |
| 190 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 191-230 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | GAGE/RADAR BIAS ESTIMATE......................... ”XXXX.XXX” (space padded) Range:0.001 to 100.000 Default:1.255 | 0.001 | Correction to original description (no format change |
| 231 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 232-271 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | SAMPLE SIZE (EFFECTIVE NO. GAGE/RADAR PAIRS)..... ”XXXX.XXX” (space padded) Range:0.000 to 9999.999 Default:13.494 | 0.001 | Correction to original description (no format change |
| 272 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 273-312 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MEMORY SPAN (HOURS) OVER WHICH BIAS DETERMINED ... ”XXXX.XXX” (space padded) Range:0.001 to 10.**3 Default:168.006 | 0.001 | Correction to original description (no format change |
| 313 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 314-353 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | PRODUCT ADJUSTED BY BIAS ESTIMATE? ............... ”XXX” (space padded) Range:Yes to No Default:No | N/A |  |
| 354 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |
| 355 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 356-395 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | RADAR HALF POWER BEAM WIDTH. . . . . . . . . . . . . . . . . . . . . . . . . . “XXXXX.XX” DEG” (space padded) Range: 0.80 to 1.00 Default:0.90 | 0.10 | Correction to original description (no format change |
| 396 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 397-436 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAXIMUM ALLOWABLE PERCENT OF BEAM BLOCKAGE . . . . . . . . . . . . . .”XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.10 | Correction to original description (no format change |
| 437 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 438-477 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAXIMUM ALLOWABLE PERCENT LIKELIHOOD OF CLUTTER . ……………... ..”XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.10 | Correction to original description (no format change |
| 478 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 479-518 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | PERCENT OF BEAM REQUIRED TO COMPUTE AVERAGE POWER……”XXXXX.XX” (space padded) Range: 0.00 to 100.00 Default: 50.00 | 0.10 | Correction to original description (no format change |
| 519 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 520-559 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | PERCENT OF HYBRID SCAN NEEDED TO BE CONSIDERED FULL…..”XXXXX.XX” (space padded) Range: 90.00 to 100.00 Default: 99.70 | 0.10 | Correction to original description (no format change |
| 560 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 561-600 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | LOW REFLECTIVITY THRESHOLD (dBZ) FOR BASE DATA. . . . . . . . . . . . . . . . . . . . . . . .”XXXXX.XX DBZ” (space padded) Range: -40.00 to -20.00 Default: -32.00 | 0.10 | Correction to original description (no format change |
| 601 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 602-641 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | REFLECTIVITY (dBZ) REPRESENTING SIGNIFICANT RAIN. . . . . . . . . “XXXXX.XX” (space padded) Range: 10.00 to 30.00 Default: 20.00 | 0.10 | Correction to original description (no format change |
| 642 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 643-682 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | AREA WITH REFLECTIVITY EXCEEDING SIGNIFICANT RAIN THRESHOLD. . . . . . . .”XXXXX.XX” KM**2” (space padded) Range: 0.00 to 82800.00 Default:80.00 | 1.00 | Correction to original description (no format change |
| 683 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 684-723 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | THRESHOLD TIME WITHOUT RAIN FOR RESETTING STP . . . . . . . . . . . . .”XXXXX.XX” MINUTES” (space padded) Range:0.00 to 1440.00 Defaults: 60.00 | 1.00 | Correction to original description (no format change |
| 724 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 725-764 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | REFLECT-TO-PRECIP RATE CONVERSION MULTIPLICATIVE COEFFICIENT. ….”XXXXX.XX” DBZ” (space padded) Range: 30.00 to 3000.00 Defaults: 300.00 | 0.10 | Correction to original description (no format change |
| 765 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 766-805 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | REFLECT-TO-PRECIP RATE CONVERSION POWER COEFFICIENT …”XXXXX.XX” DBZ” (space padded) Range: 1.00 to 2.50 Default: 1.40 | 0.10 | Correction to original description (no format change |
| 806 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 807-846 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MIN DBZ FOR CONVERTING TO PRECIP RATE (VIA TABLE LOOKUP)…”XXXXX.XX” (space padded) Range: -32.00 to 20.00 Default: 0.00 | 0.10 | Correction to original description (no format change |
| 847 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 848-887 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAX DBZ FOR CONVERTING TO PRECIP RATE (VIA TABLE LOOKUP)…”XXXXX.XX” (space padded) Range: 50.00 to 90.00 Default: 70.00 | 0.10 | Correction to original description (no format change |
| 888 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 889-928 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | NUMBER OF EXCLUSION ZONE…”XXXXX.XX” Range: 0.00 to 20.00 (space padded) Default: 3.00 | 1.00 | Correction to original description (no format change |
| 929 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |
| 930 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 931-970 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | MAX STORM SPEED (m/sec)…”XXXXX M/Sec” Range: 10.00 to 40.00 Default: 25.00 | N/A | CCR#NA04-32201 |
| 971 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 972-1011 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | MAX SCAN-TO-SCAN TIME DIFFERENCE FOR TIME CONTINUITY TESTS…”XXXXX MINUTES” Range: 10.00 to 30.00 Default:15.00 | N/A | CCR#NA04-32201 |
| 1012 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 1013-1052 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | MIN PRECIP-AREA FOR PERFORMING TIME CONTINUITY TESTS…”XXXXX KM**2” Range:50 to 1000 Default: 200.00 | N/A | CCR#NA04-32201 |
| 1053 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 1054-1093 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | RATE OF CHANGE: VOLUM ETRIC PRECIP RATE, MIN ECHO AREA . . . . . . . . .”XXXXX 1/Hr” Range:0.1 to 99.9 Default:24.00 | N/A | CCR#NA04-32201 |
| 1094 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 1095-1134 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | RATE OF CHANGE: VOLUM ETRIC PRECIP RATE, Full ECHO UMBRELLA . . . . . . . . .”XXXXX 1/Hr” Range:0.1 to 99.9 Default:13.20 | N/A | CCR#NA04-32201 |
| 1135 | Number of Characters | INT*2 | N/A | 80 | 1 | CCR#NA04-32201 |
| 1136-1175 | Site Adaptable Parameters | CHAR | 8 Bit ASCII | MAX ECHO-AREA RATE OF CHANGE…”XXXXX KM**2/Hr” Range:20.00 to 700.00 Default:200.00 | N/A | CCR#NA04-32201 |
| Note: as a result of implementing this CCR the halfword addresses of all remaining fields have changed |  |  |  |  |  |  |
| 930 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 931-970 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | RANGE BEYOND WHICH TO APPLY RANGE-EFFECT FUNCTION…”XXXXX.XX” KM (space padded) Range:0.00 to 230.00 Default:230.00 | 1.00 | Correction to original description (no format change |
| 971 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 972-1011 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | 1ST COEFFICIENT OF RANGE-EFFECT FUNCTION…”XXXXX.XX DBR” (space padded) Range:0.00 to 3.00 Default:0.00 | 0.10 | Correction to original description (no format change |
| 1012 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1013-1052 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | 2ND COEFFICIENT OF RANGE-EFFECT FUNCTION…”XXXXX.XX DBR” (space padded) Range:1.00 to 10.00 Default:1.00 | 0.10 | Correction to original description (no format change |
| 1053 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1054-1093 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | 3RD COEFFICIENT OF RANGE-EFFECT FUNCTION…”XXXXX.XX DBR” (space padded) Range:0.00 to 1.00 Default:0.00 | 0.10 | Correction to original description (no format change |
| 1094 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1095-1134 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MIN RATE SIGNIFYING PRECIPITATION…”XXXXX.XX MM/Hr” (space padded) Range:0.00 to 10.00 Default:0.00 | 0.10 | Correction to original description (no format change |
| 1135 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1136-1175 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAX PRECIPITATION RATE…”XXXXX.XX MM/Hr” (space padded) Range:50.00 to 1600.00 Default:103.80 | 0.10 | Correction to original description (no format change |
| 1176 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |
| 1177 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1178-1217 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | REINITIALIZATION TIME LAPSE THRESHOLD (FOR ACCUM PROCESS)…”XXXXX.XX MINUTES” (space padded) Range:45.00 to 60.00 Default:60.00 | 1.00 | Correction to original description (no format change |
| 1218 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1219-1258 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAX TIME DIFFERENCE BETWEEN SCANS FOR INTERPOLATION…”XXXXX.XX MINUTES” (space padded) Range:15.00 to 60.00 Default:30.00 | 1.00 | Correction to original description (no format change |
| 1259 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1260-1299 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MIN TIME NEEDED TO ACCUMULATE HOURLY TOTALS…”XXXXX.XX MINUTES” (space padded) Range:0.00 to 60.00 Defaults:54.00 | 1.00 | Correction to original description (no format change |
| 1300 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1301-1340 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | THRESHOLD FOR HOURLY OUTLIER ACCUMULATION…”XXXXX.XX MM” (space padded) Range:50.00 to 800.00 Default:400.00 | 1.00 | Correction to original description (no format change |
| 1341 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1342-1381 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | HOURLY GAGE ACCUMULATION SCAN ENDING TIME…”XXXXX.XX MINUTES” (space padded) Range:0.00 to 59.00 Default:0.00 | 1.00 | Correction to original description (no format change |
| 1382 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1383-1422 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAX ACCUMULATION PER SCAN-TO-SCAN PERIOD…”XXXXX.XX MM” (space padded) Range:50.00 to 400.00 Default:400.00 | 1.00 | Correction to original description (no format change |
| 1423 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1424-1463 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MAX ACCUMULATION PER HOURLY PERIOD…”XXXXX.XX MM” (space padded) Range:50.00 to 1600.00 Default:800.00 | 1.00 | Correction to original description (no format change |
| 1464 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |
| 1465 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1466-1505 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | MINUTES AFTER CLOCK HOUR WHEN BIAS IS UPDATED…”XXXXX.XX MM” (space padded) Range:50.00 to 59.00 Default:50.00 | 1.00 | Correction to original description (no format change |
| 1506 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1507-1546 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | THRESHOLD # OF GAGE/RADAR PAIRS NEEDED TO SELECT BIAS…”XXXXX.XX” (space padded) Range:6.00 to 30.00 Default:10.00 | 1.00 | Correction to original description (no format change |
| 1547 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1548-1587 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | RESET VALUE OF GAGE/RADAR BIAS ESTIMATE…”XXXXX.XX” (space padded) Range:0.5 to 2.0 Default:1.0 | 0.10 | Correction to original description (no format change |
| 1588 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1589-1628 | Site Adaptable Parameters | CHAR*80 | 8 Bit ASCII | LONGEST ALLOWABLE LAG FOR USE OF BIAS FROM BIAS TABLE…”XXXXX.XX HOURS” (space padded) Range:100.00 to 1000.00 Default:168.00 | 1.00 | Correction to original description (no format change |
| 1629 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |

The following is the tabular alphanumeric block coming from one of the products by the use

of the CVG

CVT using variable CV_ORPG_BUILD to set ORPG Build to 6

*** ORPG DATABASE PRODUCT LOAD UTILITY ***

-> Number of Products Available=5417

-> Message ID=5395

-> Product Info: LBuffer# 105 MSGLEN 010680 VOLNUM 158 ELEV 07

Tabular Alphanumeric Block

Block Divider =   -1

Block ID =        3

Length of Block = 3258 bytes

Block Divider =   -1

Number of Pages:  5

Tabular Alphanumeric Block - Page: 1

1-HOUR PRECIPITATION ACCUMULATION                  10/09/03 13:25

GAGE/RADAR BIAS ESTIMATE .........................       1.255

SAMPLE SIZE (EFFECTIVE NO. GAGE/RADAR PAIRS) .....      13.494

MEMORY SPAN (HOURS) OVER WHICH BIAS DETERMINED ...     168.006

PRODUCT ADJUSTED BY BIAS ESTIMATE? ...............     YES

Tabular Alphanumeric Block - Page: 2

RADAR HALF POWER BEAM WIDTH.................................      0.90 DEG

MAXIMUM ALLOWABLE PERCENT OF BEAM  BLOCKAGE.................     50.00  %

MAXIMUM ALLOWABLE PERCENT LIKELIHOOD OF CLUTTER.............     50.00

PERCENT OF BEAM REQUIRED TO COMPUTE AVERAGE POWER...........     50.00

PERCENT OF HYBRID SCAN NEEDED TO BE CONSIDERED FULL.........     99.70

LOW REFLECTIVITY THRESHOLD (dBZ) FOR BASE DATA..............    -32.00 DBZ

REFLECTIVITY (dBZ) REPRESENTING SIGNIFICANT RAIN............     20.00

AREA WITH REFLECTIVITY EXCEEDING SIGNIFICANT RAIN THRESHOLD.     80.00 KM**2

THRESHOLD TIME WITHOUT RAIN FOR RESETTING STP ..............     60.00 MINUTES

REFLECT-TO-PRECIP RATE CONVERSION MULTIPLICATIVE COEFFICIENT    300.00 DBZ

REFLECT-TO-PRECIP RATE CONVERSION POWER COEFFICIENT.........      1.40 DBZ

MIN DBZ FOR CONVERTING TO PRECIP RATE (VIA TABLE LOOKUP)....      0.00

MAX DBZ FOR CONVERTING TO PRECIP RATE (VIA TABLE LOOKUP)....     70.00

NUMBER OF EXCLUSION ZONES...................................      3.00

Tabular Alphanumeric Block - Page: 3

RANGE BEYOND WHICH TO APPLY RANGE-EFFECT CORRECTION.........    230.00 KM

1ST COEFFICIENT OF RANGE-EFFECT FUNCTION....................      0.00 DBR

2ND COEFFICIENT OF RANGE-EFFECT FUNCTION....................      1.00

3RD COEFFICIENT OF RANGE-EFFECT FUNCTION....................      0.00

MIN RATE SIGNIFYING PRECIPITATION...........................      0.00 MM/Hr

MAX PRECIPITATION RATE......................................    103.80 MM/Hr

Tabular Alphanumeric Block - Page: 4

REINITIALIZATION TIME LAPSE THRESHOLD (FOR ACCUM PROCESS)...     60.00 MINUTES

MAX TIME DIFFERENCE BETWEEN SCANS FOR INTERPOLATION.........     30.00 MINUTES

MIN TIME NEEDED TO ACCUMULATE HOURLY TOTALS.................     54.00 MINUTES

THRESHOLD FOR HOURLY OUTLIER ACCUMULATION...................    400.00 MM

HOURLY GAGE ACCUMULATION SCAN ENDING TIME...................      0.00 MINUTES

MAX ACCUMULATION PER SCAN-TO-SCAN PERIOD....................    400.00 MM

MAX ACCUMULATION PER HOURLY PERIOD..........................    800.00 MM

Tabular Alphanumeric Block - Page: 5

MINUTES AFTER CLOCK HOUR WHEN BIAS IS UPDATED...............     50.00 MINUTES

THRESHOLD # OF GAGE/RADAR PAIRS NEEDED TO SELECT BIAS.......     10.00

RESET VALUE OF GAGE/RADAR BIAS ESTIMATE.....................      1.00

LONGEST ALLOWABLE LAG FOR USE OF BIAS FROM BIAS TABLE.......    168.00 HOURS

TAB Message Complete

program complete