Precipitation Processing System (PPS) Product Format Description

Supplemental Precipitation Data (SPD)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | Correction |
| Daniel Stein & Jihong Liu | 17 Apr 06 | Updated description, and fields range; current for Build 9 |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | Alphanumeric |  |
| Time generated | Once every volume scan |  |
| Product size | 2834 |  |
| Compression | None |  |
| Resolution | N/A |  |
| AWIPS ID | WSRSPDxxx | xxx is site ID for originating WFO |
| Description/Purpose Supplemental Precipitation Data (SPD) produces 2 pages of information. (See the end of this document for sample SPD output.) On the first page, there are two sections of data generated by two separate algorithms. The “Gage Bias” portion is computed by the prcpadju (precip adjustment) algorithm; the remainder of the first page is generated by the EPRE algorithm. The first section is a listing of local bias data, including the “best” bias estimate, its corresponding memory span, sample size, and the date/time of the last bias update. The parameter “EFFECTIVE # G/R PAIRS” is selectable by the user via adaptation data. This parameter determines which row from page 2 is used to compute the bias multiplicative factor. On the first row where “EFFECTIVE NO. G-R PAIRS” is greater than the input parameter, the value in the “MEAN FIELD BIAS” field is multiplied against the radar-measured precip values to derive a biased precip value. The second section of page 1 lists some of the supplemental data generated by the EPRE algorithm including the total number of blockage bins rejected, clutter bins rejected, final bins smoothed, percent hybrid scan bins filled, highest elevation used, and total rain area in square kilometers. Note that the field “FINAL BINS SMOOTHED” is always 0. This is because the smoothing algorithm has not been implemented yet, but this field is in the SPD product as a placeholder. On page 2, the SPD displays a copy of the bias table ingested from AWIPS Bias table. Currently, the PPS uses a “canned” (i.e. non real-time) bias table. Each bias value is computed by dividing the average gage value by the average radar value. SPD is a stand-alone tabular alphanumeric product, so the symbology block and graphic alphanumeric block do not apply to SPD. |  |  |

The following table provides a detailed specification of the SPD product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a Halfword is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 82 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 2834 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 82 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 |  | 0 |  |  |
| 28 | Not used | INT*2 |  | 0 |  |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Not used | INT*2 | N/A | 0 | N/A |  |
| 32 | Not used | INT*2 | N/A | 0 | N/A |  |
| 33 | Not used | INT*2 | N/A | 0 | N/A |  |
| 34 | Not used | INT*2 | N/A | 0 | N/A |  |
| 35 | Not used | INT*2 | N/A | 0 | N/A |  |
| 36 | Not used | INT*2 | N/A | 0 | N/A |  |
| 37 | Not used | INT*2 | N/A | 0 | N/A |  |
| 38 | Not used | INT*2 | N/A | 0 | N/A |  |
| 39 | Not used | INT*2 | N/A | 0 | N/A |  |
| 40 | Not used | INT*2 | N/A | 0 | N/A |  |
| 41 | Not used | INT*2 | N/A | 0 | N/A |  |
| 42 | Not used | INT*2 | N/A | 0 | N/A |  |
| 43 | Not used | INT*2 | N/A | 0 | N/A |  |
| 44 | Not used | INT*2 | N/A | 0 | N/A |  |
| 45 | Not used | INT*2 | N/A | 0 | N/A |  |
| 46 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewData Level 16 Threshold | INT*2 | N/A | 0 | N/A |  |
| 47 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewMax rainfall | INT*2 | N/A | 0 | N/A |  |
| 48 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewMean-field Bias Three Hour Average | INT*2 | N/A | 0 | N/A |  |
| 49 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewffective No. G-R Pairs (Sample Size) Three Hour Average | INT*2 | N/A | 0 | N/A |  |
| 50 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewRainfall End Date | INT*2 | N/A | 0 | N/A |  |
| 51 | Not used INT*2 0 From cvt view Not used INT*2 0 From cvt viewRainfall End Time | INT*2 | N/A | 0 | N/A |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 1 to 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Symbology | INT*4 | Halfwords | 0 | N/A | NOTE: SPD does not have a Symbology Block so the value in this field is not used |
| 57-58 | Offset to Graphic | INT*4 | Halfwords | 0 | 1 |  |
| 59-60 | Offset to Tabular | INT*4 | Halfwords | 60 | 1 |  |

[PRODUCT SYMBOLOGY BLOCK IS NOT APPLICABLE FOR SPD]

[GRAPHIC ALPHANUMERIC BLOCK IS NOT APPLICABLE FOR SPD]

STAND-ALONE TABULAR ALPHANUMERIC BLOCK (TAB) IS COMPLETELY ALPHANUMERIC PRODUCT

References

2620001F (Class I Users ICD):

Section 3.2.1.4 “Tabular Alphanumeric Block”

Fig 3-3 “Message Header”

Fig 3-6 “Graphic Product Message” Sheet 7 and Sheet 10

Fig 3-16 Stand-Alone Tabular Alphanumeric Product Message

Fig 3-28 Sheet 7

26200003F (Product Spec ICD):

Appendix C, Format IX “Precipitation Adaptation Data”, Sheets 1-4

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Number of Pages | INT*2 | N/A | 2 | 1 |  |
| 63 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 64-103 | Character Data | CHAR*80 | 8 Bit ASCII | SUPPLEMENTAL PRECIPITATION DATA - RDA ID XXX MM/DD/YY HH:MM (space padded) | N/A |  |
| 104 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 105-144 | Character Data | CHAR*80 | 8 Bit ASCII | ‘ ‘ (Blank Line) | N/A |  |
| 145 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 146-185 | Character Data | CHAR*80 | 8 Bit ASCII | VOLUME COVERAGE PATTERN = XXX MODE = X TIME CONT: XXXX (space padded) | N/A | CCR# NA04-28102 CCR# NA04-32201 |
| 186 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 187-226 | Character Data | CHAR | 8 Bit ASCII | ‘ ‘ (Blank Line) | N/A |  |
| 227 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 228-267 | Character Data | CHAR*80 | 8 Bit ASCII | GAGE BIAS APPLIED - XXX (space padded) (range from Y ES to NO) | N/A |  |
| 268 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 269-308 | Character Data | CHAR*80 | 8 Bit ASCII | BIAS ESTIMATE - XXXXX.XX (space padded) (range from 0.01 to 100.00, default 1.00) | 0.01 | Correction to original description (no format change) |
| 309 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 310-349 | Character Data | CHAR*80 | 8 Bit ASCII | EFFECTIVE # G/R PAIRS - XXXXX.XX (space padded) (range from 0.00 to 999.99, default 0.00 | 0.01 | Correction to original description (no format change) |
| 350 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 351-390 | Character Data | CHAR*80 | 8 Bit ASCII | MEMORY SPAN (HOURS) - XXXXX.XX (space padded) (range from 0.001 to 10.**7, default 1.00) | 0.01 | Correction to original description (no format change) |
| 391 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 392-431 | Character Data | CHAR*80 | 8 Bit ASCII | DATE/TIME LAST BIAS UPDATE – MM/DD/YY HH:MM (space padded) | N/A |  |
| 432 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 433-472 | Character Data | CHAR*80 | 8 Bit ASCII | TOTAL NO. OF BLOCKAGE BINS REJECTED - XXXXXXXX (space padded) (range from 0 to 82800, default 0) | 1 |  |
| 473 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 474-513 | Character Data | CHAR*80 | 8 Bit ASCII | CLUTTER BINS REJECTED - XXXXXXXX (space padded) (range from 0 to 82800) | 1 |  |
| 514 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 515-554 | Character Data | CHAR*80 | 8 Bit ASCII | FINAL BINS SMOOTHED - XXXXXXXX (space padded) (range from 0 to 82800) | 1 |  |
| 555 | Number of Characters | INT*2 | N/A | 80 | 1 | From Fig 3-6 Sheet 7 |
| 556-595 | Character Data | CHAR*80 | 8 Bit ASCII | HYBRID SCAN PERCENT BINS FILLED - XXXXX.XX (space padded) (range from 90.00 to 100.00) | 0.10 |  |
| 596 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 597-636 | Character Data | CHAR*80 | 8 Bit ASCII | HIGHEST ELEV. USED (DEG) - XXXXX.XX (space padded) (range from 0.00 to 19.50) | 0.01 |  |
| 637 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 638-677 | Character Data | CHAR*80 | 8 Bit ASCII | TOTAL RAIN AREA (KM**2) - XXXXXX.X (space padded) (range from 0.0 to 166190.3) | 0.1 | CCR #NA04-27811 |
| 678 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 679-718 | Character Data | CHAR*80 | 8 Bit ASCII | ‘ ‘ (Blank Line) | N/A |  |
| 719 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 720-759 | Character Data | CHAR*80 | 8 Bit ASCII | MISSING PERIOD: MM/DD/YY HH:MM MM/DD/YY HH:MM (space padded) (range from NONE to [Missing beginning MM/DD/YY HH:MM to ending time MM/DD/YY HH:MM]) | N/A |  |
| 760 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |
| 761 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 762-801 | Character Data | CHAR*80 | 8 Bit ASCII | GAGE-RADAR MEAN FIELD BIAS TABLE | N/A |  |
| 802 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 803-842 | Character Data | CHAR*80 | 8 Bit ASCII | ‘ ‘ (Blank Line) | N/A |  |
| 843 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 844-883 | Character Data | CHAR*80 | 8 Bit ASCII | LAST BIAS UPDATE TIME: MM/DD/YY HH:MM BIAS APPLIED ? YES/NO | N/A |  |
| 884 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 885-924 | Character Data | CHAR*80 | 8 Bit ASCII | ‘ ‘ (Blank Line) | N/A |  |
| 925 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 926-965 | Character Data | CHAR*80 | 8 Bit ASCII | MEMORY SPAN ¦ EFFECTIVE NO. ¦ AVG. GAGE ¦ AVG. RADAR ¦ MEAN FIELD ¦ | N/A |  |
| 966 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 967-1006 | Character Data | CHAR*80 | 8 Bit ASCII | (HOURS) ¦ G-R PAIRS ¦ VALUE (MM) ¦ VALUE (MM) ¦ BIAS ¦ | N/A |  |
| 1007 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1008-1047 | Row 1: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 1: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 1: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 1: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 1: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1048 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1049-1088 | Row 2: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 2: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 2: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 2: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 2: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1089 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1090-1129 | Row 3: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 3: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 3: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 3: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 3: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1130 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1131-1170 | Row 4: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 4: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 4: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 4: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 4: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1171 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1172-1211 | Row 5: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 5: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 5: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 5: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 5: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1212 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1213-1252 | Row 6: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 6: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 6: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 6: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 6: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1253 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1254-1293 | Row 7: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 7: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 7: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 7: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 7: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1294 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1295-1334 | Row 8: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 8: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 8: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 8: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 8: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1335 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1336-1375 | Row 9: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 9: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 9: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 9: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 9: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1376 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 1377-1416 | Row 10: Memory Span (Hours) | CHAR*12 | HRS | “XXXXXXXX.XXX” Range: 0.001 to 10.**7 | 0.001 |  |
|  | Row 10: EFFECTIVE NO. G-R Pairs | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” Range: 0.000 to 999.999 | 0.001 |  |
|  | Row 10: AVG. GAGE VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 10: AVG. Row1: RADAR VALUE (MM) | CHAR*16 | MM | “XXXXXXXXXXXX.XXX” Range: 0.000 to 400.000 | 0.001 |  |
|  | Row 10: MEAN FIELD BIAS | CHAR*16 | N/A | “XXXXXXXXXXXX.XXX” (4 space padded) Range: 0.001 to 100.0 | 0.001 |  |
| 1417 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |

The following is abstracted from the generated product to demonstrate the content of the Stand-Alone TAB.

Note: Some field values in the bias portion are derived from the AWIPS bias table on the second page.

CVT using variable CV_ORPG_BUILD to set ORPG Build to 6

*** ORPG DATABASE PRODUCT LOAD UTILITY ***

-> Number of Products Available=5417

-> Message ID=5264

-> Product Info: LBuffer# 109 MSGLEN 002930 VOLNUM 154 ELEV 06

Stand Alone Tabular Alphanumeric Product

Block Divider =   -1

Number of Pages:  2

Stand Alone Tabular Alphanumeric Product - Page: 1

SUPPLEMENTAL PRECIPITATION DATA - RDA ID   999  10/09/03 13:03

VOLUME COVERAGE PATTERN = 121   MODE = A

GAGE BIAS APPLIED               -      YES

BIAS ESTIMATE              -     1.25

EFFECTIVE # G/R PAIRS      -    13.49

MEMORY SPAN (HOURS)        -   168.01

DATE/TIME LAST BIAS UPDATE - 08/22/98 16:12

TOTAL NO. OF BLOCKAGE BINS REJECTED     -        0

CLUTTER BINS REJECTED      -    12035

FINAL BINS SMOOTHED        -        0

HYBRID SCAN PERCENT BINS FILLED        -    99.71

HIGHEST ELEV. USED (DEG)   -     6.00

TOTAL RAIN AREA (KM**2)    -  16870.0

MISSING PERIOD: 10/09/03 02:03 10/09/03 00:03

Stand Alone Tabular Alphanumeric Product - Page: 2

GAGE-RADAR MEAN FIELD BIAS TABLE

LAST BIAS UPDATE TIME:  08/22/98 16:12                      BIAS APPLIED ?  YES

MEMORY SPAN  | EFFECTIVE NO. |   AVG. GAGE   |   AVG. RADAR  |   MEAN FIELD  |

(HOURS)    |   G-R PAIRS   |   VALUE (MM)  |   VALUE (MM)  |      BIAS     |

0.001           0.000           1.016           1.090           0.932

1.000           0.000           1.863           1.538           1.211

2.000           0.000           2.648           1.933           1.370

3.001           0.000           2.825           2.032           1.391

4.998           0.000           2.908           2.089           1.392

10.004           0.000           2.935           2.118           1.386

168.006          13.494           2.627           2.094           1.255

719.819         126.212           2.417           2.104           1.149

2160.295         212.154           2.381           2.109           1.129

9999044.000         277.982           2.365           2.112           1.120

SATAP Message Complete

program complete