Precipitation Processing System (PPS) Product Format Description

Three Hour Surface Rainfall Accumulation (THP)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 16-level |  |
| Time generated | Once every volume scan | It generates the blank images before it reaches the first three hours. The image of Rainfall accumulation changes with every three hours |
| Product size | N/A |  |
| Compression | None |  |
| Resolution | 1.1-Nmi x 1-deg |  |
| Range | 124 |  |
| AWIPS ID | WSRTHPxxx | xxx is site ID for originating WFO |
| Description/Purpose THP provides three-hour rainfall accumulation maps displayed as an image. |  |  |

The following table provides a detailed specification of the THP product.

Highlighted areas in the description below indicate changes since the previous Build

[Note: a Halfword is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 79 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 4 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 79 | N/A |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | Not used | INT*2 |  | 0 | N/A |  |
| 28 | Not used | INT*2 |  | 0 | N/A |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Not used | INT*2 | N/A | 0 | N/A |  |
| 31 | Data Level 1 Threshold | INT*2 | Inches | Code for ND (i.e. 0) | 1 |  |
| 32 | Data Level 2 Threshold | INT*2 | Inches | Code for > 0.00 | 1 |  |
| 33 | Data Level 3 Threshold | INT*2 | Inches | Code for > 0.10 | 1 |  |
| 34 | Data Level 4 Threshold | INT*2 | Inches | Code for > 0.25 | 1 |  |
| 35 | Data Level 5 Threshold | INT*2 | Inches | Code for > 0.50 | 1 |  |
| 36 | Data Level 6 Threshold | INT*2 | Inches | Code for > 0.75 | 1 |  |
| 37 | Data Level 7 Threshold | INT*2 | Inches | Code for > 1.00 | 1 |  |
| 38 | Data Level 8 Threshold | INT*2 | Inches | Code for > 1.25 | 1 |  |
| 39 | Data Level 9 Threshold | INT*2 | Inches | Code for > 1.50 | 1 |  |
| 40 | Data Level 10 Threshold | INT*2 | Inches | Code for > 1.75 | 1 |  |
| 41 | Data Level 11 Threshold | INT*2 | Inches | Code for > 2.00 | 1 |  |
| 42 | Data Level 12 Threshold | INT*2 | Inches | Code for > 2.50 | 1 |  |
| 43 | Data Level 13 Threshold | INT*2 | Inches | Code for > 3.00 | 1 |  |
| 44 | Data Level 14 Threshold | INT*2 | Inches | Code for > 4.00 | 1 |  |
| 45 | Data Level 15 Threshold | INT*2 | Inches | Code for > 6.00 | 1 |  |
| 46 | Data Level 16 Threshold | INT*2 | Inches | Code for > 8.00 | 1 |  |
| 47 | Max rainfall | INT*2 | Inches | 0.0 to 189.0 | 0.1 |  |
| 48 | Mean-field Bias Three Hour Average | INT*2 | N/A | 0.01 to 99.99 | 0.01 |  |
| 49 | Effective No. G-R Pairs (Sample Size) Three Hour Average | INT*2 | N/A | 0.00 to 9999.99 | 0.01 |  |
| 50 | Rainfall End Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 51 | Rainfall End Time | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 52 | Not used | INT*2 | N/A | 0 | N/A |  |
| 53 | Not used | INT*2 | N/A | 0 | N/A |  |
| 54 | Version | INT*1 | N/A | 1 to 2 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Symbology | INT*4 | Halfwords | 60 | 1 |  |
| 57-58 | Offset to Graphic | INT*4 | Halfwords | 0 | 1 |  |
| 59-60 | Offset to Tabular | INT*4 | Halfwords | 0 to 400000 | 1 | Varies, depending on size of Symbology Block |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” Sheet 3 and Sheet 8

Fig 3-10 “Radial Data Packet”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Block ID | INT*2 | N/A | 1 | N/A |  |
| 63-64 | Length of Block | INT*4 | Bytes | 1 to 400000 | 1 |  |
| 65 | Number of Layers | INT*2 | N/A | 1 | 1 |  |
| 66 | Layer Divider | INT*2 | N/A | -1 | N/A |  |
| 67-68 | Length of Data Layer | INT*4 | N/A | 1 to 400000 | 1 |  |
| 69 | Packet Code | INT*2 | N/A | AF1F (Hex) | N/A |  |
| 70 | Index of First Range Bin | INT*2 | N/A | 0 | 1 |  |
| 71 | Number of Range Bins | INT*2 | N/A | 115 | 1 |  |
| 72 | I Center of Sweep | INT*2 | Km/4 | 256 | 1 |  |
| 73 | J Center of Sweep | INT*2 | Km/4 | 280 | 1 |  |
| 74 | Scale Factor | Scaled Integer | Pixels | 2.0 | 0.001 |  |
| 75 | Number of Radials | INT*2 | N/A | 360 | 1 |  |
| 76 | Number of RLE Halfwords in Radial | INT*2 | Halfword | 1 to 116 | 1 | }Repeat |
| 77 | Radial Start Angle | Scaled Integer (INT*2) | Degrees | 0.0 to 359.0 | 0.1 | } Each } of } |
| 78 | Radial Angle Delta | Scaled Integer (INT*2) | Degrees | 1.0 to 2.0 | 0.1 | } Radial } } |
| 79 | Run (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Run (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| … | … | … | … | … | … | } |
| … | … | … | … | … | … | } } |
| … | … | … | … | … | … | } |
|  | Run (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
|  | Color Code (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |

[GRAPHIC ALPHANUMERIC BLOCK IS NOT APPLICABLE FOR THP]

TABULAR ALPHANUMERIC BLOCK – CONTAINS THP PAIRED ALPHANUMERIC PRODUCT

References

2620001F (Class 1 Users ICD):

Section 3.2.1.4 “Tabular Alphanumeric Block”

Fig 3-3 “Message Header”

Fig 3-6 “Graphic Product Message” Sheet 7 and Sheet 10

Fig 3-10 “Radial Data Packet”

Table V “Product Dependent Halfword Definition for Product Description Block”

Fig 3-28 Sheet 7

26200003F (Product Spec ICD):

Appendix C, Format IX “Precipitation Adaptation Data”, Sheets 1-4

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Offset+1 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 2 | Block ID | INT*2 | N/A | 3 | N/A |  |
| 3-4 | Length of Block | INT*4 | Bytes | 1 to 65535 | 1 |  |
| 5 | Message Code | INT*2 | N/A | 108 | N/A |  |
| 6 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 7-8 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 9-10 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 11 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 12 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 13 | Number of Blocks | INT*2 | N/A | 4 | 1 |  |
| 14 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 15-16 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 17-18 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 19 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 20 | Product Code | INT*2 | N/A | 79 | N/A |  |
| 21 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 22 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 23 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 24 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 25 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 26-27 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 28 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 29-30 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 31 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 32 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 33 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 34 | Not Used | INT*2 |  | 0 | N/A |  |
| 35 | Data Level 1 Threshold | INT*2 | Inches | Code for ND (i.e. 0) | 1 |  |
| 36 | Data Level 2 Threshold | INT*2 | Inches | Code for >0.00 | 1 |  |
| 37 | Data Level 3 Threshold | INT*2 | Inches | Code for >0.10 | 1 |  |
| 38 | Data Level 4 Threshold | INT*2 | Inches | Code for >0.25 | 1 |  |
| 39 | Data Level 5 Threshold | INT*2 | Inches | Code for >0.50 | 1 |  |
| 40 | Data Level 6 Threshold | INT*2 | Inches | Code for >0.75 | 1 |  |
| 41 | Data Level 7 Threshold | INT*2 | Inches | Code for >1.00 | 1 |  |
| 42 | Data Level 8 Threshold | INT*2 | Inches | Code for >1.25 | 1 |  |
| 43 | Data Level 9 Threshold | INT*2 | Inches | Code for >1.50 | 1 |  |
| 44 | Data Level 10 Threshold | INT*2 | Inches | Code for >1.75 | 1 |  |
| 45 | Data Level 11 Threshold | INT*2 | Inches | Code for >2.00 | 1 |  |
| 46 | Data Level 12 Threshold | INT*2 | Inches | Code for >2.50 | 1 |  |
| 47 | Data Level 13 Threshold | INT*2 | Inches | Code for >3.00 | 1 |  |
| 48 | Data Level 14 Threshold | INT*2 | Inches | Code for > 4.00 | 1 |  |
| 49 | Data Level 15 Threshold | INT*2 | Inches | Code for >6.00 | 1 |  |
| 50 | Data Level 16 Threshold | INT*2 | Inches | Code for >8.00 | 1 |  |
| 51 | Max rainfall | INT*2 | Inches | 0.0 to 189.0 | 0.1 |  |
| 52 | Mean-field Bias | INT*2 | N/A | 0.01 to 99.99 | 0.01 |  |
| 53 | Effective No. G-R Pairs (Sample Size) | INT*2 | N/A | 0.00 to 9999.99 | 0.01 |  |
| 54 | Rainfall End Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 55 | Rainfall End Time | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 56 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 57 | Not Used | INT*2 | N/A | 0 | N/A |  |
| 58 | Version | INT*1 | N/A | 1 to 2 | 1 |  |
| 58 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 59-60 | Offset to Symbology | INT*4 | Halfwords | 60 | 1 |  |
| 61-62 | Offset to Graphic | INT*4 | Halfwords | 0 | 1 |  |
| 63-64 | Offset to Tabular | INT*4 | Halfwords | 0 to 400000 | 1 |  |
| 65 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 66 | Number of Pages | INT*2 | N/A | 5 | 1 |  |
| 67 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 68-107 | Title | CHAR*80 | 8 Bit ASCII | 3-HOUR PRECIPITATION ACCUMULATION MM/DD/YY HH:MM (space padded) | N/A |  |
| 108 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 109-148 | Blank Line | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A |  |
| 149 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 150-189 | Blank Line | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A |  |
| 190 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 191-230 | Line 4 | CHAR*80 | 8 Bit ASCII | NUMBER OF CONTRIBUTING HOURS : X (value range from 1 to 3) (space padded) | 1 | Correction to original description (no format change) |
| 231 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 232-271 | Line 5 | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A |  |
| 272 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 273-312 | Line 6 | CHAR*80 | 8 Bit ASCII | ‘ ‘ | N/A |  |
| 313 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 314-353 | Title1 | CHAR*80 | 8 Bit ASCII | DATE ENDING ADJUSTED BIAS SAMPLE SIZE MEM SPAN (space padded) | N/A |  |
| 354 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 355-394 | Title2 | CHAR*80 | 8 Bit ASCII | ...... HOUR (Y/N) .... (# G-R PAIRS) (HOURS) (space padded) | N/A |  |
| 395 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 396---- | Row 1: DATE_END_HOUR | CHAR*14 | 8 Bit ASCII | MM/DD/YY HH:MM (7 space padded) | N/A |  |
|  | Row 1: ADJUSTED | CHAR*1 | 8 Bit ASCII | X (range from Y/N) (7 space padded) | N/A |  |
|  | Row 1: BIAS | CHAR*5 | 8 Bit ASCII | “XX.XX” Range:0.01 to 100.00 Default:1.25 (4 space padded) | 0.01 | Correction to original description (no format change) |
|  | Row 1: SAMPLE SIZE (# G-R PAIRS) | CHAR*8 | 8 Bit ASCII | “XXXXX.XX” Range:0.00 to 9999.99 Default:13.49 (5 space padded) | 0.01 | Corrections to original description (no format change) |
| 435 | Row1: MEMORY SPAN (HOURS) | CHAR*8 | 8 Bit ASCII | ”XXXXX.XX” Range:0.01 to 10.**3 Default:168.01 (space padded) | 0.01 | Corrections to original description (no format change) |
| 436 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 437---- | Row 2: DATE_END_HOUR | CHAR*14 | 8 Bit ASCII | MM/DD/YY HH:MM (7 space padded) | N/A |  |
|  | Row 2: ADJUSTED | CHAR*1 | 8 Bit ASCII | X (range from Y/N) (7 space padded) | N/A |  |
|  | Row 2: BIAS | CHAR*5 | 8 Bit ASCII | “XX.XX” Range:0.01 to 100.00 Default:1.25 (4 space padded) | 0.01 | Correction to original description (no format change) |
|  | Row 2: SAMPLE SIZE (# G-R PAIRS) | CHAR*8 | 8 Bit ASCII | “XXXXX.XX” Range:0.00 to 9999.99 Default:13.49 (5 space padded) | 0.01 | Corrections to original description (no format change) |
| 476 | Row2: MEMORY SPAN (HOURS) | CHAR*8 | 8 Bit ASCII | ”XXXXX.XX” Range:0.01 to 10.**3 Default:168.01 (space padded) | 0.01 | Corrections to original description (no format change) |
| 477 | Number of Characters | INT*2 | N/A | 80 | 1 |  |
| 478---- | Row 3: DATE_END_HOUR | CHAR*14 | 8 Bit ASCII | MM/DD/YY HH:MM (7 space padded) | N/A |  |
|  | Row 3: ADJUSTED | CHAR*1 | 8 Bit ASCII | X (range from Y/N) (7 space padded) | N/A | Correction to original description (no format change) |
|  | Row 3: BIAS | CHAR*5 | 8 Bit ASCII | “XX.XX” Range:0.01 to 100.00 Default:1.25 (4 space padded) | 0.01 | Corrections to original description (no format change) |
|  | Row 3: SAMPLE SIZE (# G-R PAIRS) | CHAR*8 | 8 Bit ASCII | “XXXXX.XX” Range:0.00 to 9999.99 Default:13.49 (5 space padded) | 0.01 | Corrections to original description (no format change) |
| 517 | Row3: MEMORY SPAN (HOURS) | CHAR*8 | 8 Bit ASCII | ”XXXXX.XX” Range:0.01 to 10.**3 Default:168.01 (space padded) | 0.01 | Corrections to original description (no format change) |
| 518 | End of Page Flag | INT*2 | N/A | -1 | N/A |  |

The following is the tabular alphanumeric block coming from one of the products by the use of

the CVG

Tabular Alphanumeric Block

Block Divider =   -1

Block ID =        3

Length of Block = 1036 bytes

new offset = 8314

Block Divider =   -1

Number of Pages:  1  Current Page:  1

New page = 1

Tabular Alphanumeric Block - Page: 1

buf len=892, buf =

3-HOUR PRECIPITATION ACCUMULATION                08/21/98 16:17

NUMBER OF CONTRIBUTING HOURS :  3

DATE     ENDING   ADJUSTED    BIAS   SAMPLE SIZE    MEM SPAN

......   HOUR      (Y/N)         ....  (# G-R PAIRS)                 (HOURS)

08/21/98 16:00           Y                     1.25       13.49                  168.01

08/21/98 15:00           Y                     1.25       13.49                  168.01

08/21/98 14:00           Y                     1.25       13.49                  168.01