Precipitation Processing System (PPS) Product Format Description

User Selectable Storm Total Precipitation (USP)

| Change History Log |  |  |
| --- | --- | --- |
| Author | Date | Build |
| Kelley Miles | 3/8/2005 | 8 |
| Jihong Liu | 3/17/2005 | correction |

| Setting | Value | Comment |
| --- | --- | --- |
| Product type | 16-level | Two possible scales based on the maximum data value detected; one for OHP & THP, one for STP |
| Time generated | Once every volume scan | It generates the blank images before it reaches the user selected accumulation period |
| Product size | N/A |  |
| Compression | none |  |
| Resolution | 1.1-Nmi x 1-deg |  |
| Range | 124 |  |
| AWIPS ID | WSRUSPxxx | xxx is site ID for originating WFO |
| Description/Purpose This product shall provide a rainfall accumulation map displayed as an image, for user selected accumulation period. The product format and content shall be the same as the Surface Rainfall Accumulation and Storm Total Rainfall Accumulation products, except the accumulation period shall be of variable duration (in whole clock hours), ranging from a beginning to an ending time specified by the user. The product will usually be generated by request, but may also be generated routinely for limited, designated periods. One of these periods shall be the Default Period, which will span a 24 hour period from 1200z of the previous day to 1200z of the present day. The default version of the product may be generated any time following the completion of the first volume scan to start after 1200z each day, up to 6 hours later. The period of accumulation shall be defined by the operator via two parameters: the Ending Hour (ranging from 0 to 23z), and the Time Span (ranging form 1 to 24 whole clock hours prior to the Ending Hour.) No more than 30 hours of rainfall data prior to the most recent clock-hour will be required to generate this product. If a requested product cannot be generated due to an error condition, a message will be displayed explaining why, and the available hours of precipitation accumulation in the precipitation data base will be listed. "This product shall include annotations for product name, radar ID, maximum data value detected, radar position, times and dates of the beginning and end (clock hour) of the rainfall rate integration, radar position, radar elevation above MSL, radar operational mode, the bias in the radar estimate of the precipitation rate, and the error variance of the bias estimate. |  |  |

Data Levels

The User Selectable Rainfall Accumulation is displayed in 16 data levels on one of two possible scales, based on the maximum data value detected: one scale corresponding to that presently in use for the Surface Rainfall Accumulation products (OHP & THP); the other scale corresponding to that presently in use for the Storm Total Rainfall Accumulation product (STP).  The scale used for the User Selectable product will switch automatically between the two, with the OHP & THP scale in effect unless the product maximum data value exceeds the lower limit of the maximum data level (e.g. 8 inches), in which case the STP scale will be invoked.  Each data level code may vary with operational mode and with NEXRAD (or agency) system adaptation data.

Color Level Code Tables

See sections 28.2.2 and 29.2.2.

Range/Data Resolution

The resolution of this graphic product is 1.1 nmi (range) by 1 deg (azimuth) out to a range of 124 nmi.

Alphanumeric Display Graphic Screen

A tabular format (Appendix B, Format III, Configuration 6) will be displayable on the graphic display screens.  The tabular format will include:

(a)	Gage Bias Flag

(b)	Number of Hours in product

(c)	End Times

(d)	Bias

(e)	Hours Included Flag

Standard Annotations

Beginning/ending Date and Time of Rainfall Rate Integration

Maximum Data Value

Radar Rate Bias Estimate

Error Variance of Bias Estimate

Gage Adjustment Bias Flag

Product Interaction

The following overlay products are displayable on this product:

 Hail Index

 Mesocyclone

 Severe Weather Probability

 Storm Tracking Information

 Tornado Vortex Signatures.

The following table provides a detailed specification of the DHR product.

Highlighted areas in the description below indicate changes since the previous Build

Note: a Halfword is 16 bits]

MESSAGE HEADER

References

2620001F (Class I User ICD):

Fig 3-3 “Message Header”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Message Code | INT*2 | N/A | 31 | N/A |  |
| 02 | Date of Message | INT*2 | Julian Date | 1 to 32,767 | 1 |  |
| 03-04 | Time of Message | INT*4 | Seconds | 0 to 86,399 | 1 |  |
| 05-06 | Length of Message | INT*4 | N/A | 18 to 409856 | 1 |  |
| 07 | Source ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 08 | Destination ID | INT*2 | N/A | 0 to 999 | 1 |  |
| 09 | Number of Blocks | INT*2 | N/A | 3 | 1 |  |

PRODUCT DESCRIPTION BLOCK

References

2620001F (Class I Users ICD):

Fig 3-6 “Graphic Product Message” Sheet 2, Sheet 6, Sheet 7

Table III “Message Codes for Products”

Table V “Product Dependent Halfword Definition for Product Description Block”

2620003F (Product Spec ICD):

Section 28.2.2 “Color Level Code Table”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Block Divider | INT*2 | NA | -1 | N/A |  |
| 11-12 | Latitude of Radar | INT*4 | Degrees | -90 to +90 | 0.001 |  |
| 13-14 | Longitude of Radar | INT*4 | Degrees | -180 to +180 | 0.001 |  |
| 15 | Height of Radar | INT*2 | Feet | -100 to +11000 | 1 |  |
| 16 | Product Code | INT*2 | N/A | 31 |  |  |
| 17 | Operational Mode | INT*2 | N/A | 0 to 2 | N/A |  |
| 18 | Volume Coverage Pattern | INT*2 | N/A | 1 to 767 | N/A |  |
| 19 | Sequence Number | INT*2 | N/A | -13, 0 to 32767 | 1 |  |
| 20 | Volume Scan Number | INT*2 | N/A | 1 to 80 | 1 |  |
| 21 | Volume Scan Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 22-23 | Volume Scan Start Time | INT*4 | Seconds GMT | 0 to 86399 | 1 |  |
| 24 | Product Generation Date | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 25-26 | Product Generation Time | INT*4 | Seconds | 0 to 86399 | 1 |  |
| 27 | End Hour | INT*2 | Hours | 0 to 23 | 1 |  |
| 28 | Time Span | INT*2 | Hours | 1 to 24 | 1 |  |
| 29 | Elevation Number | INT*2 | N/A | 0 to 20 | 1 |  |
| 30 | Null Product Flag | INT*2 | N/A | 0 to 1 | N/A |  |
| 31 | Data Level 1 Threshold | INT*2 | Inches | OHP/THP Code for ND (i.e. 0); STP Code for ND (i.e. 0) | 1 |  |
| 32 | Data Level 2 Threshold | INT*2 | Inches | OHP/THP Code for > 0.00; STP Code for > 0.0 | 1 |  |
| 33 | Data Level 3 Threshold | INT*2 | Inches | OHP/THP Code for > 0.10; STP Code for > 0.3 | 1 |  |
| 34 | Data Level 4 Threshold | INT*2 | Inches | OHP/THP Code for > 0.25; STP Code for > 0.6 | 1 |  |
| 35 | Data Level 5 Threshold | INT*2 | Inches | OHP/THP Code for > 0.50; STP Code for 1.0 | 1 |  |
| 36 | Data Level 6 Threshold | INT*2 | Inches | OHP/THP Code for > 0.75; STP Code for 1.5 | 1 |  |
| 37 | Data Level 7 Threshold | INT*2 | Inches | OHP/THP Code for > 1.00; STP Code for 2.0 | 1 |  |
| 38 | Data Level 8 Threshold | INT*2 | Inches | OHP/THP Code for > 1.25; STP Code for 2.5 | 1 |  |
| 39 | Data Level 9 Threshold | INT*2 | Inches | OHP/THP Code for > 1.50; STP Code for 3.0 | 1 |  |
| 40 | Data Level 10 Threshold | INT*2 | Inches | OHP/THP Code for > 1.75; STP Code for 4.0 | 1 |  |
| 41 | Data Level 11 Threshold | INT*2 | Inches | OHP/THP Code for > 2.00; STP Code for 5.0 | 1 |  |
| 42 | Data Level 12 Threshold | INT*2 | Inches | OHP/THP Code for > 2.50; STP Code for 6.0 | 1 |  |
| 43 | Data Level 13 Threshold | INT*2 | Inches | OHP/THP Code for > 3.00; STP Code for 8.0 | 1 |  |
| 44 | Data Level 14 Threshold | INT*2 | Inches | OHP/THP Code for > 4.00; STP Code for 10.0 | 1 |  |
| 45 | Data Level 15 Threshold | INT*2 | Inches | OHP/THP Code for > 6.00; STP Code for 12.0 | 1 |  |
| 46 | Data Level 16 Threshold | INT*2 | Inches | OHP/THP Code for > 8.00; STP Code for 15.0 | 1 |  |
| 47 | Max rainfall | INT*2 | Inches | 0.0 to 327.6 | 0.1 |  |
| 48 | Begin Date Rainfall | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 49 | Begin Time Rainfall | INT*2 | Minutes | 1 to 1439 | 1 |  |
| 50 | End Date Rainfall | INT*2 | Julian Date | 1 to 32767 | 1 |  |
| 51 | End Time Rainfall | INT*2 | Minutes | 0 to 1439 | 1 |  |
| 52 | Average Mean-field Bias | INT*2 | N/A | 0.01 to 99.99 | 0.01 |  |
| 53 | Average Effective No. G-R Pairs (Sample Size) | INT*2 | N/A | 0.00 to 9999.99 | 0.01 |  |
| 54 | Version | INT*1 | N/A | 0 | 1 |  |
| 54 | Spot Blank | INT*1 | N/A | 0 to 1 | 1 |  |
| 55-56 | Offset to Symbology | INT*4 | Halfwords | 60 | 1 |  |
| 57-58 | Offset to Graphic | INT*4 | Halfwords | 0 to 400000 | 1 |  |
| 59-60 | Offset to Tabular | INT*4 | Halfwords | 0 | 1 | Varies, depending on size of Symbology Block |

PRODUCT SYMBOLOGY BLOCK

References

2620001F (Class I User ICD):

Section 3.2.1.2 “Product Symbology Block”

Fig 3-6 “Graphic Product Message” Sheet 3 and Sheet 8

Fig 3-10 “Radial Data Packet”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 62 | Block ID | INT*2 | N/A | 1 | N/A |  |
| 63-64 | Length of Block | INT*4 | Bytes | 1 to 400000 | 1 |  |
| 65 | Number of Layers | INT*2 | N/A | 1 | 1 |  |
| 66 | Layer Divider | INT*2 | N/A | -1 | N/A |  |
| 67-68 | Length of Data Layer | INT*4 | N/A | 1 to 400000 | 1 |  |
| 69 | Packet Code | INT*2 | N/A | AF1F (Hex) | N/A |  |
| 70 | Index of First Range Bin | INT*2 | N/A | 0 | 1 |  |
| 71 | Number of Range Bins | INT*2 | N/A | 115 | 1 |  |
| 72 | I Center of Sweep | INT*2 | Km/4 | 256 | 1 |  |
| 73 | J Center of Sweep | INT*2 | Km/4 | 280 | 1 |  |
| 74 | Scale Factor | Scaled Integer | Pixels | 2.0 | 0.001 |  |
| 75 | Number of Radials | INT*2 | N/A | 360 | 1 |  |
| 76 | Number of RLE Halfwords in Radial | INT*2 | Halfword | 1 to 116 | 1 | }Repeat |
| 77 | Radial Start Angle | Scaled Integer (INT*2) | Degrees | 0.0 to 359.9 | 0.1 | } Each } of } |
| 78 | Radial Angle Delta | Scaled Integer (INT*2) | Degrees | 1.0 to 2.0 | 0.1 | } Radial } } |
| 79 | Run (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (0) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Run (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| 79 | Color Code (1) | 4 Bit INT | N/A | 0 to 15 | 1 | } } |
| … | … | … | … | … | … | } |
| … | … | … | … | … | … | } } |
| … | … | … | … | … | … | } |
|  | Run (N) | 4 Bit INT | N/A | 0 to 15 | 1 | } |
|  | Color Code (N) | 4 Bit INT | N/A | 0 to 15 | 1 |  |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |
|  | End of Block | 4 Bit INT | N/A | 0000 | N/A | End of } block } marker } |

GRAPHIC ALPHANUMERIC BLOCK

References:

2620001F (Class I User ICD):

Figure 3-6 “Graphic Product Message” Sheet 4, Sheet 9 and Sheet 11

Section 3.2.1.3 “Graphic Alphanumeric Block”

Packet Code 8 (Text and Special Symbol Packets), shown in Figure 3-8b

Packet Code 10 (Unlinked Vector Packet), shown in Figure 3-8

Table VII. Product Dependent Definition for Graphic Alphanumeric Block: Fig 3-62

2620003F (Product Spec ICD):

Appendix B “Graphic Display Formats” Format III “Attribute Area”

| HALFWORD | FIELDNAME | TYPE | UNITS | VALUE | PRECISION/ ACCURACY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| Offset+1 | Block Divider | INT*2 | N/A | -1 | N/A |  |
| 2 | Block ID | INT*2 | N/A | 2 | N/A |  |
| 3-4 | Length of Block | INT*4 | Bytes | 604 | 1 |  |
| 5 | Number of Pages | INT*2 | N/A | 2 | 1 | Correction to original description (no format change) |
| 6 | Page Number | INT*2 | N/A | 1 | 1 |  |
| 7 | Length of Page | INT*2 | Bytes | 590 | 1 |  |
| 8 | Text Packet | INT*2 | N/A | 8 | N/A |  |
| 9 | Length of Data Block (Bytes) | INT*2 | Bytes | 86 | 1 |  |
| 10 | Value of Text String | INT*2 | N/A | 0 | 1 | Color level of text |
| 11 | I Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 12 | J Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 52 | Message to follow | Char*80 | 8 bit ASCII | “GAGE BIAS – XXXX” Range: APPLIED, NOT APPLIED (space padded). | N/A |  |
| 53 | Text Packet | INT*2 | N/A | 8 | N/A |  |
| 54 | Length of Data Block (Bytes) | INT*2 | Bytes | 86 | 1 |  |
| 55 | Value of Text String | INT*2 | N/A | 0 | 1 | Color level of text |
| 56 | I Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 57 | J Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 97 | Message to follow | Char*80 | 8 bit ASCII | “XX OF XX HOURS IN PRODUCT” Range from 1 to 24 (space padded) | 1 | Correction to original description (no format change) |
| 98 | Text Packet | INT*2 | N/A | 8 | N/A |  |
| 99 | Length of Data Block (Bytes) | INT*2 | Bytes | 86 | 1 |  |
| 100 | Value of Text String | INT*2 | N/A | 0 | 1 | Color level of text |
| 101 | I Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 102 | J Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 142 | Message to Follow | Char*80 | 8 bit ASCII | “END TIMES xxZ xxZ xxZ ….” (end times range from 00 to 23) | 1 | The number of END TIMES depends on how many ending times the user selects. Correction to original description (no format change) |
| 143 | Text Packet | INT*2 | N/A | 8 | N/A |  |
| 144 | Length of Data Block (Bytes) | INT*2 | Bytes | 86 | 1 |  |
| 145 | Value of Text String | INT*2 | N/A | 0 | 1 | Color level of text |
| 146 | I Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 147 | J Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 187 | Message to Follow | Char*80 | 8 bit ASCII | “BIAS X.XX X.XX X.XX…” (each bias value ranges from 0.00 to 99.9 9) | 0.01 | The number of BIAS depends on the how many ending times the user selects |
| 188 | Text Packet | INT*2 | N/A | 8 | N/A |  |
| 189 | Length of Data Block (Bytes) | INT*2 | Bytes | 86 | 1 |  |
| 190 | Value of Text String | INT*2 | N/A | 0 | 1 | Color level of text |
| 191 | I Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 192 | J Start | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 232 | Message to Follow | Char*80 | 8 bit ASCII | “HOURS INCLUDED? XXX XXX XXX…” Values: “YES”, “ NO”) | N/A |  |
| 233 | Text Packet | INT*2 | N/A | 10 | N/A |  |
| 234 | Length of Data Block (Bytes) | INT*2 | Bytes | 50 | 1 |  |
| 235 | Value (level) of vector | INT*2 | N/A | 5 | 1 |  |
| 236 | Begin I Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 237 | Begin J Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 238 | End I Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 239 | End J Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 240 | Begin I Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 241 | Begin J Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 242 | End I Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 243 | End J Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 244 | Begin I Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 245 | Begin J Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 246 | End I Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 247 | End J Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 248 | Begin I Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 249 | Begin J Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 250 | End I Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 251 | End J Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 252 | Begin I Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 253 | Begin J Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 254 | End I Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 255 | End J Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 256 | Begin I Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 257 | Begin J Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 258 | End I Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 259 | End J Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 300 | Text Packet | INT*2 | N/A | 10 | N/A |  |
| 301 | Length of Data Block (Bytes) | INT*2 | Bytes | 82 | 1 |  |
| 302 | Value (level) of vector | INT*2 | N/A | 5 | 1 |  |
| 303 | Begin I Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 304 | Begin J Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 305 | End I Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 306 | End J Vector 1 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 307 | Begin I Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 308 | Begin J Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 309 | End I Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 310 | End J Vector 2 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 311 | Begin I Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 312 | Begin J Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 313 | End I Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 314 | End J Vector 3 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 315 | Begin I Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 316 | Begin J Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 317 | End I Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 318 | End J Vector 4 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 319 | Begin I Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 320 | Begin J Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 321 | End I Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 322 | End J Vector 5 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 323 | Begin I Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 324 | Begin J Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 325 | End I Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 326 | End J Vector 6 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 327 | Begin I Vector 7 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 328 | Begin J Vector 7 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 329 | End I Vector 7 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 330 | End J Vector 7 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 331 | Begin I Vector 8 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 332 | Begin J Vector 8 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 333 | End I Vector 8 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 334 | End J Vector 8 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 335 | Begin I Vector 9 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 336 | Begin J Vector 9 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 337 | End I Vector 9 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 338 | End J Vector 9 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 339 | Begin I Vector 10 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 340 | Begin J Vector 10 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 341 | End I Vector 10 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |
| 342 | End J Vector 10 | INT*2 | Km/4 or Pixels | -2048 to +2047 | 1 |  |

[TABULAR ALPHANUMERIC BLOCK NOT APPLICABLE FOR USP]

The following is USP Graphical Alphanumerical Block (GAB) abstracted from CVG

Page 1:

Page2: