Description of Cross Section Information for the Illinois River

These cross sections were prepared in 1994 by the consulting engineering firm Freeze and Nichols for a dam failure study.  The cross sections were derived from USGS 1” = 2000’ topographic maps and not from field surveys. Figure 1 shows the location and numbering system of the cross sections at major points of interest to help the user navigate through the data. There are 131 cross sections in this study.  Manning’s roughness coefficients are defined for the 130 reaches between the cross sections.

Figure 1. Schematic of Cross Section Numbers

Figure 2 describes the format of the cross section information.  The cross sections are described using the input notation and form used by the National Weather Service hydraulic routing models.  Note:  we do not describe the cross section as a series of explicit x,y coordinate pairs starting from one side of the channel and progressing to the other. Rather, each cross section is described as a set of 8 pairs of top width of flow versus elevation above mean sea level as shown in Table 1.

Figure 2.  Explanation of cross section data

Table 1.  Example Cross Section Data.

| Number of Cross Section | River Mile above Talequah USGS Gage | Comments/ Figure No. | Elevations in feet above Mean Sea Level; Top width active flow, feet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 34.96 | Most upstream cross section on Illinois R. Main channel | Elevation in feet above mean sea level. | 867 |  | 880 | 885 | 900 | 920 | 940 | 960 | 980 |
|  |  | Figure 6 | top width active flow, feet | 100 | 300 |  | 1600 | 3750 | 4100 | 4130 | 4160 | 4163 |
|  |  |  | Composite Manning's N value | 0.045 | 0.045 |  | 0.045 | 0.0714 | 0.0815 | 0.0855 | 0.088 | 0.0881 |